| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF2024-v2 |
| --- | --- |

| Title: | Exploration Experiment on Enhanced Compression beyond VVC capability (EE2) |  |  |
| --- | --- | --- | --- |
| Status: | Output document to JVET |  |  |
| Purpose: | EE description |  |  |
| Author(s) or Contact(s): | Vadim Seregin Jie Chen Roman Chernyak Karam Naser Jacob Ström Fan Wang Martin Winken Xiaoyu Xiu Kai Zhang | Tel: Email: | vseregin@qti.qualcomm.com jiechen.cj@alibaba-inc.com chernyak@tencent.com Karam.Naser@InterDigital.com jacob.strom@ericsson.com wangfan6@oppo.com martin.winken@hhi.fraunhofer.de xiaoyuxiu@kwai.com zhangkai.video@bytedance.com |
| Source: | EE coordinators |  |  |

_____________________________

# Abstract

This document describes Exploration Experiments (EEs) planned to be performed between 32nd and 33rd JVET meetings to evaluate enhanced compression tools beyond VVC capability.

# Introduction

EE focus is to evaluate individual coding technologies and their combinations. Contributions improving compression efficiency further is highly encouraged.

EE related discussions shall happen on JVET reflector.

EE tests should be implemented on top the ECM software, ECM-11.0 is used as an anchor in the tests.

Tests shall be performed according to the CTC described in JVET-AF2017.

TGM class tests is required for SCC tests and is optional otherwise.

Palette mode shall be enabled for classes F and TGM in both ECM anchor and EE tests.

For RPR tests, in addition to ECM CTC the tests are performed following JVET-Q2015, where only LB configuration is mandatory, and the sequences length is reduced to 5 seconds for all classes.

AI and RA test configurations are required for intra tool testing, while RA and LB test configurations are required for inter tool testing. LP configuration is optional. In LB and LP configurations, the sequences length is reduced to 5 seconds for all classes.

If encoder modification is included in EE tests, such encoder optimization, if applicable, introduced to the anchor should be tested.

# Timeline

T1 = 3 weeks (November 10, 2023) after JVET meeting: ECM is released

T2 = T1 + 1 week (November 17, 2023): EE description is finalized

T3 = T2 + 2 weeks (December 1, 2023): Initial software release for EE tests

T4 = JVET meeting start – 3 weeks (December 27, 2023): Software in EE branches is frozen

# List of tests

|  | Tests | Tester | Cross-checker |
| --- | --- | --- | --- |
| 1 Intra prediction |  |  |  |
| 1.1a | Decoder derived CCP mode | Y.-J. Chang (Qualcomm) |  |
| 1.1b | Test 1.1a with decoder derived CCP fusion modes | Y.-J. Chang (Qualcomm) |  |
| 1.2 | IntraTMP with merge candidates | K. Naser (InterDigital) |  |
| 1.3 | SGPM with IntraTMP and IBC | K. Naser (InterDigital) |  |
| 1.4 | IntraTMP extension to DIMD | K. Naser (InterDigital) |  |
| 1.5 | IntraTMP extension to LIC | F. Le Léannec (InterDigital) |  |
| 1.6 | Test 1.4 + Test 1.5 | F. Le Léannec (InterDigital) |  |
| 1.7a | Test 1.6 + Test 1.3 | K. Naser (InterDigital) |  |
| 1.7b | Test 1.7a + Test 1.2 | F. Le Léannec (InterDigital) |  |
| 1.8a | The length of the auto-relocated BVP trace path is 1 (i.e, n=1) | N. Zhang (Bytedance) |  |
| 1.8b | The length of the auto-relocated BVP trace path is 2 (i.e, n=2) | N. Zhang (Bytedance) |  |
| 1.8c | No constraint for the length of the auto-relocated BVP trace path | N. Zhang (Bytedance) |  |
| 1.9 | Intra TMP fusion probing | J.-L. Lin (Qualcomm) |  |
| 1.10 | Bilateral filtering for intra prediction | W. Yin (Bytedance) |  |
| 1.11a | IntraTMP search area extension with sampling factor proportional to CU distance | D. Ruiz Coll (Ofinno) |  |
| 1.11b | IntraTMP search area extension with sampling factor proportional to search area dimension | K. Naser (InterDigital) |  |
| 1.11c | Test 1.11a + Test 1.11b | D. Ruiz Coll (Ofinno) K. Naser (InterDigital) |  |
| 1.12a | IBC with extended reference area up to the picture’s upper side boundary | Y. Kidani (KDDI) |  |
| 1.12b | IBC with four CTU rows instead of two CTU rows in HD resolution or less | Y. Kidani (KDDI) |  |
| 1.13a | Test 1.11c + Test 1.12a | Y. Kidani (KDDI) D. Ruiz Coll (Ofinno) K. Naser (InterDigital) |  |
| 1.13b | Test 1.11c + Test 1.12b | Y. Kidani (KDDI) D. Ruiz Coll (Ofinno) K. Naser (InterDigital) |  |
| 1.14a | Encoder run-time reduction methods for extrapolation filter-based intra prediction mode only | L. Xu (OPPO) |  |
| 1.14b | Encoder run-time reduction methods for extrapolation filter-based intra prediction and its merge mode | L. Xu (OPPO) |  |
| 1.15a | CCP merge fusion | H. Huang (OPPO) Z. Deng (Bytedance) |  |
| 1.15b | Inheriting LB-CCP flag in CCP merge mode | H. Huang (OPPO) Z. Deng (Bytedance) |  |
| 1.15c | Test1.15a + Test1.15b | H. Huang (OPPO) Z. Deng (Bytedance) |  |
| 1.16 | Slope adjustment for IBC LIC | C. Ma (Kwai) |  |
| 1.17a | IBC GPM with block vector difference | C. Ma (Kwai) |  |
| 1.17b | IBC GPM with split mode reordering | C. Ma (Kwai) |  |
| 1.17c | Test 1.17a + Test 1.17b | C. Ma (Kwai) |  |
| 1.18a | DIMD mode derivation from spatial blocks | J. Huo, J. Fan (Xidian Univ.) M. Li (OPPO) |  |
| 1.18b | DIMD mode derivation with reduced complexity | J. Huo, J. Fan (Xidian Univ.) M. Li (OPPO) |  |
| 1.19 | IBC-LIC model merge mode | L. Zhang (OPPO) |  |
| 1.20a | TIMD fusion with non-angular predictor | P. Andrivon (Ofinno) |  |
| 1.20b | Test 1.20a + TIMD sample-based fusion | P. Andrivon (Ofinno) |  |
| 1.21 | TIMD fusion reference line determination | C. Zhou (vivo) |  |
| 1.22a | Test 1.20a + Test 1.21 | P. Andrivon (Ofinno) C. Zhou (vivo) |  |
| 1.22b | Test 1.20b + Test 1.21 | P. Andrivon (Ofinno) C. Zhou (vivo) |  |
| 2 Inter prediction |  |  |  |
| 2.1 | A LIC flag for inter prediction merge modes | Y. Zhang (Qualcomm) |  |
| 2.2 | Enable PU level BDMVR and BDOF for bi-predicted LIC | Y. Zhang (Qualcomm) | A. Robert (InterDigital) |
| 2.3 | LIC with multiple templates | Y. Wang (Bytedance) X. Xiu (Kwai) |  |
| 2.4 | LIC with slope adjustment | Y. Wang (Bytedance) |  |
| 2.5 | Non-local illumination compensation | X. Xiu (Kwai) |  |
| 2.6a | Test 2.1 + Test 2.2 | Y. Zhang (Qualcomm) |  |
| 2.6b | Test 2.3 + Test 2.4 | Y. Wang (Bytedance) X. Xiu (Kwai) |  |
| 2.6c | Test 2.3 + Test2.5 | X. Xiu (Kwai) Y. Wang (Bytedance) |  |
| 2.6d | Test 2.1 + Test2.2 + Test 2.3 + Test 2.4 + Test 2.5 | Y. Zhang (Qualcomm) |  |
| 2.7 | AMVP with SbTMVP mode | R.-L. Liao (Alibaba) |  |
| 2.8a | DMVR for non-equal POC distance cases | M. Salehifar (Bytedance) | P. Le Guyadec (InterDigital) |
| 2.8b | DMVR for non-equal POC distance cases with constraint | M. Salehifar (Bytedance) |  |
| 2.8c | BDOF layer with new subblock sizes including 16×16 | M. Salehifar (Bytedance) |  |
| 2.8d | Mean removed formula for BDOF | M. Salehifar (Bytedance) |  |
| 2.8e | Test 2.8b + Test 2.8c + Test 2.8d | M. Salehifar (Bytedance) |  |
| 2.9 | CIIP with subblock-based motion compensation | L. Zhao (Bytedance) |  |
| 2.10 | GPM with affine prediction | K. Zhang (Bytedance) |  |
| 2.11a | Regression-based GPM blending | P. Bordes (InterDigital) |  |
| 2.11b | Test 2.11a without modified index coding | P.Bordes (InterDigital) |  |
| 2.11c | Test-2.11a without modified blending | P.Bordes (InterDigital) |  |
| 2.12 | Adjusting out-of-boundary prediction samples | P. Astola (Nokia) |  |
| 3 Transform and coefficients coding |  |  |  |
| 3.1a | Intra contexts for intra CUs in inter slices | F. Lo Bianco (InterDigital) |  |
| 3.1b | Test 3.1a + inter contexts for IntraTMP/IBC CUs | F. Lo Bianco (InterDigital) |  |
| 3.1c | Test 3.1b without context switch for CBF | F. Lo Bianco (InterDigital) |  |
| 3.1d | Test 3.1 with retrained context initialization of the affected by the test contexts | F. Lo Bianco (InterDigital) |  |
| 3.2a | Transform coefficient coding | P. Nikitin (Qualcomm) |  |
| 3.2b | Test 3.2a with retrained context initialization | P. Nikitin (Qualcomm) |  |
| 3.3 | Utilizing LFNST/NSPT for inter coding | F. Wang (OPPO) | F. Le Léannec (InterDigital) |
| 4 In-loop filtering |  |  |  |
| 4.1a | Adaptive clipping with signalled min/max values from MCTF prefiltered picture | K. Cui (Qualcomm) |  |
| 4.1b | Adaptive clipping with signalled min/max values from original picture | K. Cui (Qualcomm) |  |
| 4.2 | Fixed filter for chroma ALF | N. Hu (Qualcomm) |  |
| 4.3 | Coefficient precision adjustment for ALF | W. Yin (Bytedance) N. Hu (Qualcomm) |  |
| 5 Entropy coding |  |  |  |
| 5.1 | Retrain CABAC initialization parameters for all slices | F. Galpin (InterDigital) V. Seregin (Qualcomm) |  |
| 5.2 | Window slice type-dependent offsets initialization | V. Seregin (Qualcomm) F. Galpin (InterDigital) |  |
| 5.3a | Spatial CABAC tuning | J. Lainema (Nokia) | P. Nikitin (Qualcomm) |
| 5.3b | Spatial CABAC tuning in combination with retrained context initialization for inter slices | J. Lainema (Nokia) | K. Andersson (Ericsson) |

# Tools description

## 1. Intra prediction

### Test 1.1: Decoder derived cross-component prediction (JVET-AF0176)

In the first test, a decoder derived cross-component prediction (DDCCP) mode with a mode flag signalling is applied to derive the best CCP mode by comparing the template costs between the evaluated CCPs. On top of the first test, the second test further introduces a DDCCP fusion mode to derive CCP fusion modes selected based on the template costs. The CCP modes for fusions are based on the same derived CCP mode lists existed in the first test. Same block size constraints are applied to both tests.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 1.1a | Decoder derived CCP mode | Y.-J. Chang (Qualcomm) |
| 1.1b | Test 1.1a with decoder derived CCP fusion mode | Y.-J. Chang (Qualcomm) |

### Test 1.2: IntraTMP with merge candidates (JVET-AF0137)

In this test, the block vectors obtained from the merge candidates issued from previous blocks coded in IntraTMP or IBC mode are added to the IntraTMP block vectors candidate list. The new added block vectors are compared to the existing ones and selected according to the IntraTMP template matching cost.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 1.2 | IntraTMP with merge candidates | K. Naser (InterDigital) |

### Test 1.3: SGPM with IntraTMP and IBC (JVET-AF0226)

SGPM is an intra mode with prediction signal generated by blending 2 intra prediction modes with specific split line. In this test, SGPM is extended by including block-vector based prediction obtained from IntraTMP or IBC modes of the neighboring blocks. The new prediction is examined, along with regular intra prediction to generate the SGPM candidate list.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 1.3 | SGPM with IntraTMP and IBC | K. Naser (InterDigital) |

### Tests 1.4-1.7: IntraTMP extension to DIMD and LIC (JVET-AF0275)

This test is about two extensions to the IntraTMP prediction mode in ECM. First extension concerns DIMD. It is proposed to adaptively select between planar or block-vector based prediction obtained from IntraTMP or IBC mode of neighbouring blocks for the blending with an angular intra prediction. Second extension consists in allowing Local Illumination Compensation (LIC) in blocks coded in IntraTMP mode.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 1.4 | IntraTMP extension to DIMD | K. Naser (InterDigital) |
| 1.5 | IntraTMP extension to LIC | F. Le Léannec (InterDigital) |
| 1.6 | Test 1.4 + Test 1.5 | F. Le Léannec (InterDigital) |
| 1.7a | Test 1.6 + Test 1.3 | K. Naser (InterDigital) |
| 1.7b | Test 1.7a + Test 1.2 | F. Le Léannec (InterDigital) |

### Test 1.8: Auto relocated block vector prediction (JVET-AF0129)

Auto relocated block vector prediction (AR-BVP) is introduced into IBC merge/AMVP candidate list construction. AR-BVP can be derived as a guiding BV plus a BV of a reference block, located by the guiding BV. The AR-BVP candidates are inserted after the HBVP candidates.

As shown in Figure 1, a guiding block vector BV0,1 associated with the current block B0 points to a reference block B1. If B1 has a BV denoted as BV1,2 pointing to a reference block B2, then BV0,2,given by BV0,2 = BV0,1 +BV1,2, is defined as the AR-BVP, guided by BV0,1. Similarly, BV0,n+1 can be derived by

BV0,n+1=BV0,n+BVn,n+1 = BV0,1+BV1,2 +…+BVn-1,n+BVn,n+1.

The initial guiding vector BV0,1is set to be an existing BVP already in the IBC merge/AMVP candidate list.

When deriving BVn,n+1guided by BV0,n, all five positions including top-left (e.g., LT in Figure 2), top-right (e.g., RT in Figure 2), center (e.g., Ctr in Figure 2), bottom-left (e.g., LB in Figure 2), and bottom-right (e.g., RB in Figure 2) positions of Bn are checked to find BVn,n+1.

Figure 1. An example of how to derive AR-BVP.

Figure 2. The five positions in Bn.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 1.8a | The length of the auto-relocated BVP trace path is 1 (i.e, n=1) | N. Zhang (Bytedance) |
| 1.8b | The length of the auto-relocated BVP trace path is 2 (i.e, n=2) | N. Zhang (Bytedance) |
| 1.8c | No constraint for the length of the auto-relocated BVP trace path | N. Zhang (Bytedance) |

### Test 1.9: Intra TMP fusion probing (JVET-AF0166)

In this test, an Intra TMP fusion probing method selects a fusion candidate with minimum probing cost from a fusion candidate list. The probing cost is derived as the SAD between the reference and current block’s probing lines. A flag is signalled to indicate whether this proposed fusion probing mode is enabled.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 1.9 | IntraTMP fusion probing | J.-L. Lin (Qualcomm) |

### Test 1.10: Bilateral filtering for intra prediction (JVET-AF0199)

The Bilateral Filtering (BF) can be used to improve the quality of intra prediction samples. In this test, the generated intra prediction samples are further filtered by bilateral filtering to reduce the noise level inside the prediction samples.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 1.10 | Bilateral filtering for intra prediction | W. Yin (Bytedance) |

### Test 1.11: IntraTMP search range extension with adaptive sampling (JVET-AF0161)

In this test, the IntraTMP search areas is extended to 2 CTU lines, similar to IBC. Specifically, two methods are proposed to determine an adaptive search range sampling factor.: In the first one, the sampling factor is proportional to the distance from the current CU. In the second one, the sampling rate is proportional to search area dimension.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 1.11a | IntraTMP search area extension with sampling factor proportional to CU distance | D. Ruiz Coll (Ofinno) |
| 1.11b | IntraTMP search area extension with sampling factor proportional to search area dimension | K. Naser (InterDigital) |
| 1.11c | Test 1.11a + Test 1.11b | D. Ruiz Coll (Ofinno) K. Naser (InterDigital) |

### Test 1.12: IBC with upward-extended reference area (JVET-AF0115)

In this test, the reference area of IBC is further extended in an upward direction. The following two extension methods are tested for camera-captured content and screen contents: Test 1.12a is IBC with the extended reference area to the picture's upper side boundary; Test 1.12b with four CTU rows instead of the existing two CTU rows above the current CTU in HD resolution or less.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 1.12a | IBC with extended reference area up to the picture’s upper side boundary | Y. Kidani (KDDI) |
| 1.12b | IBC with four CTU rows instead of two CTU rows in HD resolution or less | Y. Kidani (KDDI) |

### Test 1.13: Combination tests of IntraTMP search range extension with adaptive sampling and IBC with upward-extended area (JVET-AF0161, JVET-AF0115)

Combination of Test 1.11 and Test 1.12.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 1.13a | Test 1.11c + Test 1.12a | Y. Kidani (KDDI) |
| 1.13b | Test 1.11c + Test 1.12b | Y. Kidani (KDDI) |

### Test 1.14: Extrapolation filter-based intra prediction mode (JVET-AF0080)

In this test, the encoder run-time reduction methods are investigated for the extrapolation filter-based intra prediction mode (EIP) and the EIP merge mode.

The mode is restricted to blocks with sizes not greater than 32x32 and luma component only.

The extrapolation filter-based intra prediction mode consists of three steps:

- The extrapolation filter coefficients are derived from a neighboring reconstructed area of the current block or filter shape and coefficients are inherited from a previous block coded with this mode. For the latter, a mode flag is signalled, and candidate list is constructed using spatial, temporal, non-adjacent and history blocks coded with this mode. The candidate list is reordered based on SAD cost measured on the L-shape template of size 1. An index is further signalled to identify a candidate from the list.

- The extrapolation process generates predicted signals from the top-left to bottom-right corner within the current block.

- An intra prediction angle is derived by analyzing the gradient of the predicted block, and then the corresponding intra mode is used to select MTS, NSPT, and LFNST kernels for transform.

Three filer shapes are used in this method, a choice of reconstructed area and filter shape shown in the next figure is signalled.

The selected filter shape moves in the selected reconstructed area with a one-sample step either horizontally or vertically to collect input samples and output samples, then the filter coefficients are derived using CCCM solver.

In the current block, the recursive predictor is derived from the top-left to the bottom-right position by a diagonal prediction order as shown in the next figure, where the predicted results of the previous diagonal are used.

With the EIP merge mode, the filter shape and the filter coefficients can be inherited from the previous decoded blocks with EIP or EIP merge mode. The decoder decodes an EIP merge flag to decide whether the proposed merge mode is used when the current block uses the EIP mode. A merge index is further decoded when the EIP merge flag is true. The EIP merge list includes spatial adjacent and non-adjacent candidates, temporal candidates, and history candidates. The constructed EIP merge list will be reordered based on the SAD cost measured on an L-shape template with column width and row height equal to 1. In the SAD calculation, predictions of the template area by EIP filters are generated only from reconstructed (neighbouring and template) samples, allowing the EIP filters to be applied in parallel rather than sequentially.

The positions and inclusion order of the spatial adjacent, temporal, non-adjacent, shifted temporal and history candidates are the same as those defined in ECM-10.0 for the CCP merge prediction candidates.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 1.14a | Encoder run-time reduction methods for extrapolation filter-based intra prediction mode only | L. Xu (OPPO) |
| 1.14b | Encoder run-time reduction methods for extrapolation filter-based intra prediction and its merge mode | L. Xu (OPPO) |

### Test 1.15: Enhancements on CCP merge for chroma intra coding

In test a, CCP merge fusion with a mode flag signaling is applied on CCP merge mode, where the final chroma prediction combines the CCP merge prediction with the MM-CCCM prediction or the DIMD prediction. A fusion type flag is signalled to indicate whether the MM-CCCM or DIMD is selected.

In test b, the local-boosting cross-component prediction (LBCCP) flag is inherited in the CCP merge mode.

In test c, a combination of test a and test b is tested.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 1.15a | CCP merge fusion | H. Huang (OPPO) Z. Deng (Bytedance) |
| 1.15b | Inheriting LB-CCP flag in CCP merge mode | H. Huang (OPPO) Z. Deng (Bytedance) |
| 1.15c | Test1.15a + Test1.15b | H. Huang (OPPO) Z. Deng (Bytedance) |

### Test 1.16: Slope adjustment for IBC LIC (JVET-AF0116)

In the test, slope adjustment is applied to IBC LIC, where an offset parameter is signaled to update the slope parameter of IBC LIC. The proposed method is only applied in IBC AMVP mode.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 1.16 | Slope adjustment for IBC LIC | C. Ma (Kwai) |

### Test 1.17: Extensions of IBC GPM (JVET-AF0117)

In the test, IBC GPM is further extended from two aspects:

First, IBC-GPM with block vector difference is introduced. Specifically, by the method, one additional BV difference (BVD) is added on top of the BV of one IBC partition. The method is only applied to IBC GPM partitions which are generated based on IBC merge.

Second, the split modes of IBC GPM is reordered based on template matching costs. One index is signaled to indicate the selected GPM split mode in the reordered list.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 1.17a | IBC GPM with block vector difference | C. Ma (Kwai) |
| 1.17b | IBC GPM with split mode reordering | C. Ma (Kwai) |
| 1.17c | Test 1.17a + Test 1.17b | C. Ma (Kwai) |

### Test 1.18: DIMD mode derivation from spatial candidates (JVET-AF0106)

In this test, DIMD mode is derived by the spatial blocks, including with non-adjacent spatial candidates. The Histograms of Gradients (HoGs) of neighbouring blocks are used to derive the intra prediction mode to predict the current block. This method adds non-adjacent spatial candidates. Furthermore, optimized architecture to reduce the complexity will be investigated.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 1.18a | DIMD mode derivation from spatial blocks | J. Huo, J. Fan (Xidian Univ.) M. Li (OPPO) |
| 1.18b | DIMD mode derivation with reduced complexity | J. Huo, J. Fan (Xidian Univ.) M. Li (OPPO) |

### Test 1.19: IBC-LIC Model Merge mode (JVET-AF0084)

In this test, an IBC-LIC model merge mode is tested. It inherits an IBC-LIC model from a candidate list which includes spatial adjacent and non-adjacent neighbours, history candidates, and default models. A flag is signalled to indicate whether the IBC-LIC model merge mode is applied or not. If this flag is true, an index is further signalled to indicate which candidate model is used by the current block.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 1.19 | IBC-LIC model merge mode | L. Zhang (OPPO) |

### Test 1.20: TIMD fusion with non-angular predictor (JVET-AF0053)

In test a, a non-angular predictor is introduced in the TIMD fusion process additionally to the other selected TIMD modes.

In test b, additionally to test a, TIMD predictors are blended using a location-dependent sample-based fusion process similarly to DIMD.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 1.20a | TIMD fusion with non-angular predictor | P. Andrivon (Ofinno) |
| 1.20b | Test 1.20a + TIMD sample-based fusion | P. Andrivon (Ofinno) |

### Test 1.21: TIMD fusion reference line determination (JVET-AF0132)

This test proposes an optimization method for reference lines selection in TIMD blending mode. For the second TIMD mode, the determination of which reference line to be used is based on selected TIMD modes.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 1.21 | TIMD fusion reference line determination | C. Zhou (vivo) |

### Test 1.22: Combination of TIMD fusion related tests (JVET-AF0053, JVET-AF0132)

Combinations of Test 1.20 and Test 1.21 are investigated.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 1.22a | Test 1.20a + Test 1.21 | P.Andrivon (Ofinno) C. Zhou (vivo) |
| 1.22b | Test 1.20b + Test 1.21 | P.Andrivon (Ofinno) C. Zhou (vivo) |

## 2. Inter prediction

### Tests 2.1-2.2: LIC flag in merge mode (JVET-AF0194)

In this test, a LIC flag for inter-prediction merge modes is signalled instead of inheriting the flag value from a merge candidate.

When a bi-predicted block applies LIC and meets BDMVR and/or BDOF conditions, the PU MVs are refined by minimizing bilateral matching cost and BDOF luma adjustment is applied to the block.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 2.1 | LIC flag for inter prediction merge modes | Y. Zhang (Qualcomm) |
| 2.2 | Enable PU level BDMVR and BDOF for bi-predicted LIC | Y. Zhang (Qualcomm) |

### Test 2.3: LIC with multiple templates (JVET-AF0191, JVET-AF0200)

In the test, LIC with multiple templates is proposed. Besides using a template comprising neighbouring samples both top to and left to the current block, two new LIC modes using top-only or left-only template are proposed to derive the parameters of LIC. LIC with multiple templates is only applied to AMVP uni-predicted blocks.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 2.3 | LIC with multiple templates | Y. Wang (Bytedance) X. Xiu (Kwai) |

### Test 2.4: LIC with slope adjustment (JVET-AF0200)

In the test, LIC with slope adjustment is proposed, in which an adjustment parameter is used to modify parameters of LIC. The adjustment parameter is signalled for AMVP mode. LIC with slope adjustment is not applied to bi-prediction. Three contexts are used for the LIC flag.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 2.4 | LIC with slope adjustment | Y. Wang (Bytedance) |

### Test 2.5: Non-local illumination compensation (JVET-AF0191)

In the test, non-local illumination compensation is studied, where extra linear models are derived based on the minimization of the reconstruction and prediction samples of inter blocks. Then, for one current CU, a number of the non-local candidates obtained from spatial adjacent and non-adjacent locations are included into the regular and sub-block merge lists.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 2.5 | Non-local illumination compensation | X. Xiu (Kwai) |

### Test 2.6 Combination of LIC related tests (JVET-AF0194, JVET-AF0191, JVET-AF0200)

Combinations of Tests 2.1 - 2.5.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 2.6a | Test 2.1 + Test 2.2 | Y. Zhang (Qualcomm) |
| 2.6b | Test 2.3 + Test 2.4 | Y. Wang (Bytedance) X. Xiu (Kwai) |
| 2.6c | Test 2.3 + Test2.5 | X. Xiu (Kwai) Y. Wang (Bytedance) |
| 2.6d | Test 2.1 + Test2.2 + Test 2.3 + Test 2.4 + Test 2.5 | Y. Zhang (Qualcomm) |

### Test 2.7: AMVP with SbTMVP mode (JVET-AF0134)

In this test, SbTMVP mode is applied to regular AMVP mode. Given a CU coded in AMVP with SbTMVP mode, the CU is predicted in a similar way as that of SbTMVP in merge mode except that the motion shift is signaled in the bitstream instead of being derived from neighboring blocks. A flag is signaled to indicate whether the CU is predicted with SbTMVP. If the SbTMVP is used, another flag is signaled to indicate whether MVD exists, and an MVD index is signaled to indicate the motion direction and motion magnitude if MVD exists.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 2.7 | AMVP with SbTMVP mode | R.-L. Liao (Alibaba) |

### Test 2.8: On DMVR Extensions (JVET-AF0160)

This proposal introduces improvements for DMVR. Specifically, DMVR is extended to non-equal POC distance cases and 16×16 BDOF DMVR is added. Also, a mean removed formula is introduced for BDOF DMVR.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 2.8a | DMVR for non-equal POC distance cases | M.Salehifar (Bytedance) |
| 2.8b | DMVR for non-equal POC distance cases with constraint | M.Salehifar (Bytedance) |
| 2.8c | BDOF layer with new subblock sizes including 16×16 | M.Salehifar (Bytedance) |
| 2.8d | Mean removed formula for BDOF | M.Salehifar (Bytedance) |
| 2.8e | Test 2.8b + Test 2.8c + Test 2.8d | M.Salehifar (Bytedance) |

### Test 2.9: CIIP with subblock-based motion compensation (JVET-AF0170)

In this test, the inter prediction signal of CIIP may be generated based on affine or SbTMVP motion compensation, where a subblock-based merge candidate list is constructed. A subbock-based CIIP flag is signalled when CIIP flag is true. If subblock-based CIIP flag is true, an index indicating specific candidate in the subblock-based merge list may be signalled.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 2.9 | CIIP with subblock-based motion compensation | L. Zhao (Bytedance) |

### Test 2.10: GPM with affine prediction (JVET-AF0168)

This proposal presents a method of geometric partitioning mode (GPM) with affine prediction. A GPM partition can be predicted by affine motion compensation (AMC) or non-affine motion compensation (non-AMC), indicated by a flag. A GPM partition predicted by AMC can be blended with the other GPM partition predicted by AMC, non-AMC, or intra-prediction.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 2.10 | GPM with affine prediction | K. Zhang (Bytedance) |

### Test 2.11: Regression-based GPM blending (JVET-AF0118)

This proposal presents an enhanced GPM mode where the splitting shape and the blending function are derived from the template. Also the GPM index coding is modified with template matching based reordering.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 2.11a | Regression-based GPM blending | P.Bordes (InterDigital) |
| 2.11b | Test 2.11a without modified index coding | P.Bordes (InterDigital) |
| 2.11c | Test-2.11a without modified blending | P.Bordes (InterDigital) |

### Test 2.12: Adjusting out-of-boundary prediction samples (JVET-AF0121)

In this test improved out-of-boundary (OOB) prediction is tested. The improved OOB prediction is obtained by adjusting the uni-prediction samples using a scaling parameter and an offset to better match with the bi-prediction samples. The scale and offset are derived using the uni-prediction samples and the averaged bi-prediction samples.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 2.12 | Adjusting out-of-boundary prediction samples | P. Astola (Nokia) |

## 3. Transform and coefficients coding

### Test 3.1: CABAC inter/intra model switch for residual coding (JVET-AF0164)

In this test, intra contexts models are used for the entropy coding of residual information of some CUs in inter slices; conversely, inter context models are used for the entropy coding of residual information of some CUs in intra slices.

In Test 3.1a, intra context models are used for the entropy coding of residual information of intra coded CUs in inter slices. In Test 3.1b, on top of Test 3.1a, inter context models are used for the entropy coding of residual information of IntraTMP and IBC CUs in intra slices. In Test 3.1c, model switch is performed under the same conditions as Test 3.1b, but no switch is performed for contexts of CBF. In test 3.1d, context initialization of the affected contexts by the tests is retrained using the method in test 5.1.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 3.1a | Intra contexts for intra CUs in inter slices | F. Lo Bianco (InterDigital) |
| 3.1b | Test 3.1a + inter contexts for IntraTMP/IBC CUs | F. Lo Bianco (InterDigital) |
| 3.1c | Test 3.1b without context switch for CBF | F. Lo Bianco (InterDigital) |
| 3.1d | Test 3.1 with retrained context initialization of the affected by the test contexts | F. Lo Bianco (InterDigital) |

### Test 3.2: Transform coefficient coding (JVET-AF0185)

In this test, the number of gtX flags coded in the regular mode of the arithmetic coding using adaptive context models before remainder coding is increased. There are N+1 flags coded in the regular mode: sig_coeff_flag, gt1_flag, gt2_flag, gt3_flag, …, gtN_flag. Parity flag is bypass coded after gtN_flag. In the experiments N was set to 8. Absolute coefficient level is reconstructed as follows:

abs_coeff_level = sig +gt1+ gt2 +gt3 +… + gtN + par + 2*remLevel,

Test introduces new contexts for gt2_flag and gt4_flag, all flags larger than gt3, {gt4, gt5, gt6, gt7, gt8}, share the same context modelling indexes table.

The parity flag location can be kept as in ECM with the extended gtX flags:

abs_coeff_level = sig + gt1 + par + 2*(gt3 + gt5 + gt7 + … + gtN + remLevel).

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 3.2a | Transform coefficient coding | P. Nikitin (Qualcomm) |
| 3.2b | Test 3.2a with retrained context initialization | P. Nikitin (Qualcomm) |

### Test 3.3: Utilizing LFNST/NSPT for inter coding (JVET-AF0082)

In this test, the utilization of LFNST/NSPT for inter coding is tested. For inter coded block, an intra prediction mode is first derived according to the inter prediction block with a DIMD-like process. Then the derived intra prediction mode is used to select an LFNST/NSPT transform set and the transform can be processed with the selected LFNST/NSPT kernel, like in the intra coding process. The signaling of inter LFNST/NSPT index is different from that of intra LFNST/NSPT index.

The focus of this test is to reduce the encoding time while maintaining the coding gain.

The results for LDB configuration will be provided with intra LFNST/NSPT enabled as anchor.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 3.3 | Utilizing LFNST/NSPT for inter coding | F. Wang (OPPO) |

## 4. In-loop filtering

### Test 4.1: Adaptive clipping with signalled lower and upper bounds (JVET-AF0169)

This test presents a method of applying adaptive clipping with signalled lower and upper bounds. For each picture, the minimum and maximum values of the luma channel, which are derived from the original picture or MCTF prefiltered picture, are signalled and used to perform clipping at the reconstruction stage. When clipping is performed in LMCS mapped domain, the min/max values are further derived by applying forward LMCS look-up table to the signalled min/max values.

In Test 4.1a, the min/max values are derived from the MCTF prefiltered picture if MCTF is applied to that picture. Otherwise, they are derived from the original picture.

In Test 4.1b, the min/max values are always derived from the original picture.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 4.1a | Adaptive clipping with signalled min/max values from MCTF prefiltered picture | K. Cui (Qualcomm) |
| 4.1b | Adaptive clipping with signalled min/max values from original picture | K. Cui (Qualcomm) |

### Test 4.2: Fixed filter for chroma ALF (JVET-AF0179)

In this test, fixed filter in ALF is extended to chroma components. Similar to luma, Laplacian and variance-based classifiers are applied to chroma samples to determine which fixed filter is used. The fixed filter results are then used as additional input to the signalled chroma filters. The fixed filters are trained with non-CTC sequences.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 4.2 | Fixed filter for chroma ALF | N. Hu (Qualcomm) |

### Test 4.3: Coefficient precision adjustment for ALF (JVET-AF0198)

In general, higher coefficient precision for ALF leads to higher reconstruction quality and higher signalling cost. In this test, the coefficient precision for online-trained filters of ALF is adjusted to achieve a better balance between restoration quality and rate cost.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 4.3 | Coefficient precision adjustment for ALF | W. Yin (Bytedance) N. Hu (Qualcomm) |

## 5. Entropy coding

### Test 5.1: CABAC parameters retraining (JVET-AF0151, JVET-AF0202)

The retrained context initialization parameters are implemented on top of ECM-11.0 and are tested under CTC with all intra, random access and low delay B configurations. The training dataset is composed of all CTC (mandatory and optional TGM) sequences, encoded in all configurations (AI, RA, LDB, LDP). The training script should be provided.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 5.1 | Retrain CABAC initialization parameters for all slices | F. Galpin (InterDigital) V. Seregin (Qualcomm) |

### Test 5.2: CABAC context initialization retraining and slice type based window offsets (JVET-AF0202)

CABAC window offsets are split per slice type and initialization parameters are retrained, additional memory cost should be reported. The training script should be provided.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 5.2 | Window slice type-dependent offsets initialization | V. Seregin (Qualcomm) F. Galpin (InterDigital) |

### Test 5.3: Spatial CABAC tuning (JVET-AF0109)

This test studies performance of the spatial CABAC tuning technique proposed in JVET-AF0109. The approach uses bins related to the bottom CUs of the above CTU to adapt the context probability states when starting to process a new CTU.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 5.3a | Spatial CABAC tuning | J. Lainema (Nokia) |
| 5.3b | Spatial CABAC tuning in combination with retrained context initialization for inter slices | J. Lainema (Nokia) |