| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0012-v1 |
| --- | --- |

| Title: | JVET AHG report: Enhanced compression beyond VVC capability (AHG12) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | AHG Report |  |  |
| Author(s) or Contact(s): | M. Karczewicz Y. Ye L. Zhang B. Bross X. Li R. Chernyak K. Naser Y. Yu | Tel: Email: | martak@qti.qualcomm.com yan.ye@alibaba-inc.com lizhang.idm@bytedance.com benjamin.bross@hhi.fraunhofer.de xlxiangli@google.com chernyak@global.tencent.com karam.naser@interdigital.com yue.yu@oppo.com |
| Source: | AHG 12 |  |  |

_____________________________

# Abstract

This document reports the work of the JVET ad hoc group on enhanced compression beyond VVC capability (AHG12) between the 32nd meeting, Hannover, DE, 13–20 October 2023, and the 33rd meeting, by teleconference, 17–26 January 2024.

# Introduction

The mandates given to the AHG are:

- Solicit and study non-neural-network video coding tools with enhanced compression capabilities beyond VVC.

- Discuss and propose refinements to the ECM11 algorithm description JVET-AF2025.

- Coordinate with AHG7 to study the performance and complexity tradeoff of these video coding tools.

- Coordinate with AHG6 on ECM software development.

- Support AHG6 in generating anchors according to the test conditions in JVET-AF2017.

- Analyse the results of exploration experiments described in JVET-AF2024 in coordination with the EE coordinators.

- Coordinate with AHG11 to study the interaction with neural network-based coding tools.

The regular JVET e-mail reflector was used for discussions (jvet@lists.rwth-aachen.de). A few emails were exchanged regarding EE2.

# Activities

The primary activity of the AHG was the “Exploration experiment on enhanced compression beyond VVC capability” (JVET-AF2024). The combined improvements of the ECM-11.0 over VTM-11.0ecm11.0 anchor for AI, RA and LB configurations are:

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -10.81% | -21.80% | -29.33% | 859.1% | 400.6% |
| Class A2 | -16.95% | -28.77% | -31.78% | 860.2% | 405.4% |
| Class B | -11.31% | -26.84% | -25.03% | 784.4% | 411.0% |
| Class C | -11.41% | -16.72% | -17.24% | 810.7% | 381.9% |
| Class E | -15.01% | -24.77% | -23.07% | 750.1% | 431.9% |
| Overall | -12.81% | -23.73% | -24.81% | 808.6% | 405.0% |
| Class D | -9.48% | -14.56% | -14.65% | 795.1% | 383.1% |
| Class F | -26.60% | -35.72% | -35.58% | 513.6% | 392.7% |
| Class TGM | -39.53% | -47.43% | -46.69% | 457.3% | 461.2% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -22.47% | -27.84% | -36.88% | 875.4% | 769.1% |
| Class A2 | -26.01% | -36.30% | -40.11% | 824.7% | 958.3% |
| Class B | -20.80% | -35.53% | -33.16% | 714.1% | 774.7% |
| Class C | -22.22% | -27.13% | -27.05% | 748.2% | 829.1% |
| Class E |  |  |  |  |  |
| Overall | -22.56% | -31.91% | -33.67% | 775.1% | 821.9% |
| Class D | -23.10% | -28.71% | -28.75% | 768.1% | 898.2% |
| Class F | -28.70% | -37.47% | -37.77% | 614.7% | 505.2% |
| Class TGM | -36.82% | -43.33% | -43.43% | 566.6% | 413.2% |

|  | Low Delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -17.76% | -41.31% | -38.53% | 661.6% | 649.7% |
| Class C | -19.49% | -31.72% | -31.66% | 618.2% | 678.1% |
| Class E | -16.28% | -30.12% | -30.39% | 587.1% | 406.5% |
| Overall | -17.97% | -35.32% | -34.21% | 627.8% | 586.1% |
| Class D | -21.22% | -34.09% | -33.02% | 659.2% | 772.5% |
| Class F | -24.87% | -39.63% | -38.99% | 575.6% | 491.8% |
| Class TGM | -35.47% | -44.37% | -44.64% | 538.8% | 424.4% |

The rate reduction for natural sequences over VTM in RA configuration for {Y, U, V} increased from ECM-10.0’s {-22.23%, -30.91%, -32.63%} to ECM-11.0’s {-22.56%, -31.91%, -33.67%}. For SCC sequences (class TGM) the rate reduction for RA configuration increased from ECM-10.0’s {-36.63%, -43.02%, -43.08%} to ECM-11.0’s { -36.82%, -43.33%, -43.43%}.

# Contributions

In addition to 40 EE2 contributions, 43 (comparing to 48 last meeting) EE2-related and AHG12-related contributions were received. The EE2-related and AHG12-related contributions can be subdivided as follows:

## Intra (9)

JVET-AG0075, "AHG12: Adaptive MRL Fusion", S. Blasi, J. Lainema (Nokia)

JVET-AG0078, "AHG12: Intra-prediction using Merged Histogram of Gradients", S. Blasi, I. Zupancic, P. Astola, J. Lainema (Nokia)

JVET-AG0084, "AHG12: DIMD Merge List", M. Blestel, P. Andrivon (Ofinno)

JVET-AG0106, "AHG 12: TIMD merge mode", R. G. Youvalari, M. Abdoli (Xiaomi)

JVET-AG0120, "Non-EE2: On line buffer restriction", Z. Deng, K. Zhang, L. Zhang (Bytedance)

JVET-AG0121, "Non-EE2: Block vector guided LUT for chroma prediction", J. Huo, X. Hao, M. Chen, N. Qiu, Z. Zhang, Y. Ma, F. Yang (Xidian Univ.), M. Li, F. Wang, J. Ren (OPPO)

JVET-AG0138, "Non-EE2: Chroma intra prediction mode reordering", X. Li, R.-L. Liao, J. Chen, Y. Ye (Alibaba)

JVET-AG0141, "AHG 12: Occurrence-Based Intra Coding (OBIC)", R. G. Youvalari, M. Abdoli (Xiaomi)

JVET-AG0197, "Non-EE2: Matrix based intra prediction replacing conventional intra modes", B. Ray, H. Wang, V. Seregin, M. Karczewicz, P. Garus (Qualcomm)

## IntraTMP and IntraBC (8)

JVET-AG0063, "EE2-1.2 related: AR-BVP for IntraTMP merge candidates", L. Zhang, Y. Yu, F. Wang, H. Yu, D. Wang (OPPO)

JVET-AG0074, "Non-EE2: IntraTMP with HMVP Candidates", C. Zhu, G. Li, T. Tang, L. Luo, H. Guo (UESTC), Y. Huo, Y. Liu (Transsion)

JVET-AG0080, "EE2-related: Extend block vector prediction for IntraTMP merge candidates", N. Qiu, J. Huo, Y. Ma, F. Yang (Xidian Univ.)

JVET-AG0113, "Non-EE2: FIBC Extension", J. Kim, J. Kang, H. Han, H. Choi (HNU), W. Lim, S.-C. Lim (ETRI)

JVET-AG0186, "Non-EE2: FIBC Extension", H.-J. Jhu, X. Xiu, W. Chen, C.-W. Kuo, N. Yan, C. Ma, X. Wang (Kwai)

JVET-AG0193, "Non-EE2: Enhancements on IntraTMP", W. Chen, X. Xiu, C. Ma, H.-J. Jhu, C.-W. Kuo, N. Yan, X. Wang (Kwai)

JVET-AG0231, "EE2-1.7b related: unrestricted 1.7b performances" F. Le Léannec, K. Naser, T. Dumas, Y. Chen, M. Radosavljević, T. Poirier, (InterDigital)

JVET-AG0243, " EE2-1.4: IntraTMP extension to DIMD", K. Naser, F. Le Léannec, T. Poirier, H. Guermoud, T. Dumas, (InterDigital)

## Inter (10)

JVET-AG0073, "Non-EE2: Chained motion vector prediction", Y. Kidani, H. Kato, K. Kawamura (KDDI)

JVET-AG0096, "Non-EE2: On temporal buffer handling", Z. Deng, K. Zhang, L. Zhao, L. Zhang (Bytedance)

JVET-AG0125, "AHG12: Parallel friendly use of boundary distortion for DMVR", K. Andersson, R. Yu (Ericsson)

JVET-AG0149, "Non-EE2: Improvements to subblock merge mode", J. Chen, R.-L. Liao, Y. Zheng, X. Li, Y. Ye (Alibaba)

JVET-AG0150, "EE2-related: Adaptive GPM blending", L. Zhao, K. Zhang, L. Zhang (Bytedance)

JVET-AG0172, "EE2-related: Chroma LIC derivation with template costs", T. Bae, S. Deshpande (Sharp)

JVET-AG0194, "Non-EE2: Reference filtering for inter-prediction", A. Filippov, V. Rufitskiy, K. Suverov (Ofinno)

JVET-AG0195, "Non-EE2: LIC model parameter inheritance for merge modes", C.-C. Chen, H. Huang, V. Seregin, M. Karczewicz

JVET-AG0200, "Non-EE2: Inter CCP merge mode with zero luma CBF", Z. Deng, K. Zhang, L. Zhang (Bytedance)

JVET-AG0202, "Non-EE2: Geometry partitioning mode with inter prediction and intra block copy", Y. Wang, K. Zhang, L. Zhang (Bytedance)

## In Loop Filters (3)

JVET-AG0065, "Non-EE2: Adaptive coefficient precision for CCALF", N. Song, Y. Yu, H. Yu, D. Wang (OPPO)

JVET-AG0198, "Non-EE2: Coding Information based ALF Classification", W. Yin, K. Zhang, Y. Wang, Z. Deng, L. Zhao, N. Zhang, M. Salehifar, L. Zhang (Bytedance)

JVET-AG0233, " EE2-related: Adaptive precision for CCALF coefficients", N. Hu, M. Karczewicz, V. Seregin, H. Wang (Qualcomm)

## Transform (4)

JVET-AG0062, "Non-EE2: Multiple Transform Sets Selection for LFNST/NSPT", F. Wang, Y. Yu, H. Yu, D. Wang (OPPO)

JVET-AG0208, "EE2-related: On LFNST/NSPT index signalling", M. Koo, J. Zhao, J. Lim, S. Kim (LGE)

JVET-AG0230, "EE2-3.3 related: On Inter-LFNST”, S. Puri, K. Naser, C. Bonnineau, F. Le Léannec", (InterDigital)

JVET-AG0237, "EE2-3.3 related: Fix on LFNST/NSPT index signalling",	M. Koo, J. Zhao, J. Lim, S. Kim (LGE)

## Coefficients and Entropy Coding (7)

JVET-AG0064, "AHG12: On Context modeling in Chroma Coefficient Coding", L. Xu, H. Yu, Y. Yu, D. Wang (OPPO)

JVET-AG0066, "EE2-3.2 related: On Regular Residual Coding", Y. Yu, L. Xu, J. Gan, H. Yu, L. Zhang, H. Huang, F. Wang, Z. Xie, N. Song, D. Wang (OPPO)

JVET-AG0068, "EE2-3.2 related: On CABAC bin budget", J. Gan, Y. Yu, H. Yu (OPPO)

JVET-AG0108, "AhG12 Entropy coding extension", F. Galpin, F. Lo Bianco, C. Salmon-Legagneur, M. Balcilar (InterDigital)

JVET-AG0110, "Ahg12: 16 States TCQ with State Exchange", M. Balcilar, K. Naser, Y. Chen, Franck Galpin, Fabrice Léannec (InterDigital)

JVET-AG0102, "AHG12: New context model parameters for low delay B condition", R.-L. Liao, Y. Ye, J. Chen, X. Li (Alibaba)

JVET-AG0185, "Non-EE2: Slice based Rice parameter selection for transform skip residual coding", H.-J. Jhu, X. Xiu, W. Chen, C.-W. Kuo, N. Yan, C. Ma, X. Wang (Kwai)

## Partitioning (1)

JVET-AG0126, "AhG12: On ECM temporal partitioning prediction", G. Laroche, P. Onno (Canon)

## Other (1)

JVET-AG0116, "AHG12: GOP-based RPR encoder control for ECM", K. Andersson, J. Ström, P. Wennersten, R. Yu, W. Ahmad (Ericsson)

# Recommendations

The AHG recommends to:

- To review all the related contributions.