| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0052-v1 |
| --- | --- |

| Title: | AHG9: On using SEI processing order for NNPF grouping |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry Junghak Nam Seung hwan Kim Jaehyun Lim | Email: | dr.hendry@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

This contribution proposed some changes to sei processing order SEI message as the result of studying the SEI message for enabling it to also support grouping of NNPF SEI messages. The changes are asserted to enable the sei message to also support parallel grouping and to handle the case where SEI messages with prefix information and no prefix information present are included in sei processing order SEI message.

# Introduction

During the 32nd JVET meeting, mechanism for grouping of neural-network post-filter (NNPF) SEI messages was discussed. There were two schools of thought regarding this, that is, the first one is to enable sei processing order SEI message to also be able to support grouping of NNPF SEI messages and the second one is to use neural-network post-filter group characteristic (NNPFGC) SEI message and neural-network post-filter group activation (NNPFGA) SEI message. No consensus was reached for the solution for grouping of NNPF SEI messages but a new syntax element to specify Id (i.e., po_id) was agreed to be added to sei processing order SEI message and experts were further encouraged to study ways to use the SEI message to support grouping of NNPF SEI messages.

This contribution contains issues we found from our study to enable sei processing order SEI message to support grouping of NNPF SEI messages and possible solutions to those asserted issues.

# Asserted problems and proposed solutions

## Way to describe parallel grouping

It is asserted that the grouping mechanism should at least support the following grouping types: cascading grouping, alternative grouping, and parallel grouping. For the first two grouping types, in our opinion, the mechanism described in JVET-AF0061 works but it lacks the support for enabling sei processing order SEI message to describe parallel grouping.

To provide the support for parallel grouping in conjunction with mechanism described in JVET-AF0061, we propose to add a flag in sei processing order SEI message to specify whether SEI messages with the same processing order value are preferred to be invoked in parallel or there is no preference for them. Note that this is proposed to be used not only for parallel grouping of NNPF SEI messages but can also be applicable for any SEI message.

The text implementation for this change is shown in snippets below.

Syntax:

| sei_processing_order( payloadSize ) { | Descriptor |
| --- | --- |
| po_id | u(8) |
| po_num_sei_messages_minus2 | u(87) |
| po_parallel_processing_enabled_flag | u(1) |
| for( i = 0, i < po_num_sei_messages_minus2 + 2; i++ ) { |  |
| po_sei_wrapping_flag[ i ] | u(1) |
| po_sei_importance_flag[ i ] | u(1) |
| po_sei_payload_type[ i ] | u(13) |
| po_sei_prefix_flag[ i ] | u(1) |
| po_sei_processing_order[ i ] | u(8) |
| } |  |
| for( i = 0; i < po_num_sei_messages_minus2 + 2; i++ ) |  |
| if( po_sei_prefix_flag[ i ] ) { |  |
| po_num_bits_in_prefix_indication_minus1[ i ] | u(8) |
| for( j = 0; j <= po_num_bits_in_prefix_indication_minus1[ i ]; j++ ) |  |
| po_sei_prefix_data_bit[ i ][ j ] | u(1) |
| while( !byte_aligned( ) ) |  |
| po_byte_alignment_bit_equal_to_one /* equal to 1 */ | f(1) |
| } |  |
| } |  |

Semantic:

…

po_num_sei_messages_minus2 plus 2 indicates the number of types of SEI messages for which the preferred order of processing is indicated in the SEI processing order SEI message.

po_parallel_processing_enabled_flag equal 1 specifies that SEI messages included in this sei processing order SEI message that have the same processing order are preferred to be invoked in parallel. po_parallel_processing_enabled_flag equal 0 specifies that there is no preference for processing order for SEI messages included in this sei processing order SEI message that have the same processing order.

….

## NNPF in sei processing order SEI message without prefix information

Although it is envisioned that when NNPF SEI messages are included in an sei processing order SEI message, they will be included with some prefix information (i.e., to include nnpfc_id to differentiate different NNFPC SEI messages), we should also anticipate the case when an NNPFC SEI message is included in sei processing order SEI message without prefix information and two more NNPFC SEI messages (e.g., nnpfc_A and nnpfc_B_ are also included with prefix information. In such case, it is not clear how to interpret such situation as whether nnpfc_A and nnpfc_B should be considered included in the NNPFC in the sei processing order SEI message without prefix information, or they should be considered excluded.

The situation described above may also be applicable to SEI messages with other payload types as well. Therefore, our recommendation is that the solution to clarify / handle the situation above should apply to not only NNPF SEI messages but also any other SEI messages.

The following options may be considered to handle the situation described above:

Option 1: Define a constraint to disallow situation of mix of SEI messages with the same payload type to be included in sei processing order SEI message in which one of the SEI message has no prefix information while the rest have prefix information.

The text implementation for this option is as follows:

…

The semantics of the SEI processing order SEI message uses the concept of types of SEI messages. SEI messages that have different payloadType values are considered different types of SEI messages. Additionally, different SEI messages that have the same payloadType value but are differentiated by values of syntax elements in the SEI payload are considered different types of SEI messages. Such differentiation by values of syntax elements in the SEI payload is to be performed by comparing values sent using po_sei_prefix_data_bit[ i ][ j ] syntax elements (when present) or values sent as SEI messages within a processing order nesting SEI message (when present). For example, neural-network post-filter characteristics (NNPFC) SEI messages can be differentiated by having different nnpfc_id values.

When an SEI message of a particular payloadType value is included in an sei processing order SEI message without prefix information, there shall be no SEI message with the same payloadType value included in an sei processing order SEI message, i.e., in the same or different sei processing order SEI message, with prefix information present.

…

Option 2: Specify that when two or more SEI messages with the same payload types are included in one or more sei processing order SEI messages and one of the SEI messages has no prefix information while the rest have, processing order of those SEI messages are treated independently (i.e., the ones with no prefix information represent all the SEI messages of that type but excluding those included in the sei processing order SEI message with prefix information).

The text implementation for this option is as follows:

…

The semantics of the SEI processing order SEI message uses the concept of types of SEI messages. SEI messages that have different payloadType values are considered different types of SEI messages. Additionally, different SEI messages that have the same payloadType value but are differentiated by values of syntax elements in the SEI payload are considered different types of SEI messages. Such differentiation by values of syntax elements in the SEI payload is to be performed by comparing values sent using po_sei_prefix_data_bit[ i ][ j ] syntax elements (when present) or values sent as SEI messages within a processing order nesting SEI message (when present). For example, neural-network post-filter characteristics (NNPFC) SEI messages can be differentiated by having different nnpfc_id values.

When an SEI message of a particular payloadType value is included in an sei processing order SEI message without prefix information and there is another SEI message with the same payloadType value and has prefix information included in an sei processing order SEI message, i.e., in the same or different sei processing order SEI message, the two SEI messages are considered different SEI messages.

…

Option 3: Specify that when two or more SEI messages with the same payload types are included in one or more sei processing order SEI messages and one of the SEI messages has no prefix information while the rest have, those SEI messages with prefix information are considered part of the one without prefix information and both of the following apply:

- The sei processing order SEI message that contains the SEI messages with prefix information shall not contain SEI message with other payload type.

- The SEI message with no prefix information and the SEI messages with prefix information shall not be included in the same sei processing order SEI message.

The text implementation for this option is as follows:

…

The semantics of the SEI processing order SEI message uses the concept of types of SEI messages. SEI messages that have different payloadType values are considered different types of SEI messages. Additionally, different SEI messages that have the same payloadType value but are differentiated by values of syntax elements in the SEI payload are considered different types of SEI messages. Such differentiation by values of syntax elements in the SEI payload is to be performed by comparing values sent using po_sei_prefix_data_bit[ i ][ j ] syntax elements (when present) or values sent as SEI messages within a processing order nesting SEI message (when present). For example, neural-network post-filter characteristics (NNPFC) SEI messages can be differentiated by having different nnpfc_id values.

When an SEI message of a particular payloadType value is included in an sei processing order SEI message without prefix information and there are two or more SEI messages with the same payloadType value and have prefix information included in an sei processing order SEI message, i.e., in the same or different sei processing order SEI message, the SEI messages with prefix information present are considered part of the SEI message without prefix information and both of the following apply:

- The sei processing order SEI message that contains the SEI messages with prefix information shall not contain SEI message with other payload type.

- The SEI message with no prefix information and the SEI messages with prefix information shall not be included in the same sei processing order SEI message.

…

# Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).