| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0066-v1 |
| --- | --- |

| Title: | EE2-3.2 related: On Regular Residual Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yue Yu, Luhang Xu, Jonathan Gan, Haoping Yu, Lai Zhang, Hang Huang, Fan Wang, Zhihuang Xie, Nan Song, Dong Wang | Email: | yue.yu@oppo.com |
| Source: | OPPO |  |  |

# Abstract

EE2-3.2 extended the number of the greater than X flags and moved the parity flag after the greater than X flags in the coding order. The greater than X flags and the parity flag are coded in the first coding pass in the regular residual coding process. However, the parity flag is coded as a bypass bin in the EE2-3.2. As a result, the context coded bins and a bypass-coded bin are interleaved in the first coding pass. This contribution proposes that the parity flag is coded as a context bin so that all bins in the first coding pass are coded as context coded bins. On top of EE2-3.2, the simulation results are reported as follows. There is no coding performance change compared to EE2-3.2 while the proposed method is friendly for hardware implementation.

Over ECM-11.0

AI:   	-0.20% (Y), -0.17% (U), -0.13% (V) for natural content

-0.07% (Y), -0.05% (U), 0.26% (V) for class F

-0.05% (Y), -0.03% (U), -0.05% (V) for class TGM

Over EE2-3.2

AI:   	0.00% (Y), 0.02% (U), 0.08% (V) for natural content

0.02% (Y), 0.09% (U), 0.18% (V) for class F

-0.03% (Y), -0.02% (U), 0.01% (V) for class TGM

# Introduction

EE2-3.2 proposed to extend the number of the greater than X flags and move the parity flag after the greater than X flags in the coding order. More specifically, there are N+1 flags coded in the regular residual coding: sig_flag, gt1_flag, gt2_flag, …, gtN_flag where N is set as 7. The parity flag is coded after gt7_flag and as a bypass bin. Absolute coefficient level, abs_coeff_level, is reconstructed as follows:

abs_coeff_level = sig_flag +gt1_flag + gt2_flag +… + gt7_flag + parity_flag + 2*remLevel

where remLevel is the remaining part after coding the greater than X flags and parity flag for the current coefficient.

# Proposed method

The parity flag is coded in the first coding pass after coding sig_flag and the greater than X flags. If the context coded and bypass coded bins are denoted as “c” and “b”, respectively. The bins coded in the first coding pass will be “c”, “c” ,“c”, “c”, “c”, “c” ,“c”, “c”, “b”, “c” ,“c”, “c”…..in EE2-3.2. The context coded bin and a bypass bin will be interleaved in the first coding pass. This contribution proposes that the parity flag is coded as a context bin instead of a bypass bin. As a result, all bins coded in the first coding pass are context coded bins.

# Simulation results

The proposed method is implemented on top of the EE2-3.2 and tested based on the common test condition [2]. The test results are summarized in the following Tables.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.21% | -0.20% | -0.17% | 96.1% | 102.0% | 101.3% | 99.9% |
| Class A2 | -0.22% | -0.14% | -0.09% | 96.6% | 101.6% | 101.3% | 99.9% |
| Class B | -0.19% | -0.13% | -0.16% | 96.0% | 101.3% | 100.0% | 100.0% |
| Class C | -0.22% | -0.28% | -0.18% | 95.5% | 101.0% | 100.4% | 100.0% |
| Class E | -0.18% | -0.07% | 0.00% | 101.4% | 101.1% | 100.0% | 100.0% |
| Overall | -0.20% | -0.17% | -0.13% | 96.9% | 101.4% | 100.5% | 100.0% |
| Class D | -0.26% | -0.51% | -0.28% | 96.0% | 100.8% | 100.0% | 100.0% |
| Class F | -0.07% | -0.05% | 0.26% | 100.4% | 101.1% | 101.0% | 100.0% |
| Class TGM | -0.05% | -0.03% | -0.05% | 100.6% | 99.0% | 100.0% | 99.9% |

Table 1. The proposed method over ECM-11.0

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over EE2-3.2 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | 0.02% | 0.06% | 0.05% | 93.7% | 98.6% | 100.1% | 99.9% |
| Class A2 | -0.01% | 0.00% | 0.03% | 94.4% | 98.6% | 101.3% | 100.0% |
| Class B | 0.00% | -0.03% | 0.06% | 93.5% | 97.9% | 100.0% | 100.0% |
| Class C | 0.00% | 0.09% | 0.08% | 92.9% | 100.7% | 100.0% | 100.0% |
| Class E | -0.01% | -0.04% | 0.21% | 98.9% | 99.2% | 100.0% | 100.0% |
| Overall | 0.00% | 0.02% | 0.08% | 94.4% | 99.0% | 100.2% | 100.0% |
| Class D | -0.01% | -0.21% | -0.02% | 94.4% | 101.7% | 100.0% | 100.0% |
| Class F | 0.02% | 0.09% | 0.18% | 99.2% | 101.4% | 100.9% | 100.0% |
| Class TGM | -0.03% | -0.02% | 0.01% | 98.1% | 99.1% | 100.0% | 100.0% |

Table 2. The proposed method over EE2-3.2

Note: the running time in the tables are not accurate.

# Conclusions

This contribution proposes that the parity flag is coded as a context coded bin instead of a bypass-coded bin. Compared with EE2-3.2, there is no coding performance change. It is suggested to adopt the proposed method along with EE2-3.2 to ECM.

# References

- ECM-11.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-11.0

- V. Seregin, J. Chen, R. Chernyak, K. Naser, J. Ström, F. Wang, M. Winken, X. Xiu, K. Zhang, “Exploration Experiment on Enhanced Compression beyond VVC capability (EE2)”, JVET-AF2024, Oct. 2023.

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-AF2017, April, 2023.

# Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd. (OPPO) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

_______________________________