| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, Online, 17-26 January2024 | Document: JVET-AG0080-v2 |
| --- | --- |

| Title: | EE2-related: Extend block vector prediction for IntraTMP merge candidates |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Nengfu Qiu, Junyan Huo, Yanzhuo Ma, Fuzheng Yang, | Email: | nfqiu@stu.xidian.edu.cn jyhuo@mail.xidian.edu.cn yzma@mail.xidian.edu.cn fzhyang@mail.xidian.edu.cn |
| Source: | Xidian University |  |  |

_____________________________

- Abstract

EE2-1.7b, including EE2-1.2,EE2-1.3,EE2-1.4 and EE2-1.5.

Contribution JVET-AF0137(EE2-1.2) proposed extending the IntraTMP candidate list with all the merge candidates block vectors.

Contribution JVET-AF0226(EE2-1.3) proposed further including block-vector based prediction obtained from IntraTMP or IBC modes of the neighboring blocks in the candidate list of SGPM.

Contribution JVET-AF0275(EE2-1.4 and EE2-1.5) proposed IntraTMP extension to DIMD and LIC.

Contribution JVET-AF0129(EE2-1.8) proposed an auto relocated block vector prediction (AR-BVP) is introduced into IBC merge/AMVP candidate list construction.

This contribution merges both EE2-1.7b and EE2-1.8. In this contribution, an extension block vector prediction (EBVP) list is introduced into IntraTMP merge candidate list construction. EBVP can be derived as a guiding BV plus a BV of a reference block, located by the guiding BV.

On top of the ECM-11.0, simulation results of EE2-1.7b+EE2-1.8 are reported as below:

AI:

Class F: {-0.88%, -1.07%, -0.65% 105.4%, 106.6%, xx%};

Class TGM: {-1.63%, -1.51%, -1.54%, 103.2%, 106.7%, xx%};

On top of the ECM-11.0, simulation results of the proposed method are reported as below:

AI:

Class F: {-1.13%, -1.11%, -0.83% 106.7%, 108.3%, xx%};

Class TGM: {-2.22%, -2.08%, -2.08%, 104.4%, 107.9%, xx%};

Introduction

In EE2-1.2, the block vectors obtained from the merge candidates issued from previous blocks coded in IntraTMP or IBC mode are added to the IntraTMP block vectors candidate list. The new added block vectors are compared to the existing ones and selected according to the IntraTMP template matching cost.

In EE2-1.3, SGPM is extended by including block-vector based prediction obtained from IntraTMP or IBC modes of the neighboring blocks. The new prediction is examined, along with regular intra prediction to generate the SGPM candidate list.

In EE2-1.4, DIMD is adaptively select between planar or block-vector based prediction obtained from IntraTMP or IBC mode of neighbouring blocks for the blending with an angular intra prediction.

In EE2-1.5, it’s proposed to allow Local Illumination Compensation (LIC) in blocks coded in IntraTMP mode.

In EE2-1.8, Auto relocated block vector prediction (AR-BVP) is introduced into IBC merge/AMVP candidate list construction. AR-BVP can be derived as a guiding BV plus a BV of a reference block, located by the guiding BV. The AR-BVP candidates are inserted after the HBVP candidates.

Proposed methods

It is proposed to extend the IntraTMP Merge list with BV from the initial IntraTMP Merge list that has been built. The new IntraTMP Merge list’ length is set to N.

Specifically, the steps are as follows:

Firstly, based on each BV in the initial IntraTMP Merge list, check in turn whether the five positions of the PU corresponding to this BV0 are encoded with IntraTMP/IBC mode, as shown in Figure 1.

Then, if the BV1 is available, the BV1 + BV0 is added to the IntraTMP Merge list as BV0’, as shown in Figure 2. If it is not available, check the next location until all five locations are checked and the IntraTMP Merge list is updated.

At last, repeat with the updated list as the initial list until the IntraTMP Merge list is filled.

Figure 1. The five positions in reference PU

Figure 2. A method of EBVP’s construction.

Experimental Results

The proposed method has been implemented on top of the ECM-11.0 [1]. Simulations are performed following the common test conditions of EE2 [5]. The simulation results are shown below.

Table 1: Coding performance of EE2-1.7b+EE2-1.8.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over anchor |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.08% | -0.01% | 0.05% | 105.0% | 102.0% | 100.1% | 100.1% |
| Class A2 | -0.38% | -0.33% | -0.32% | 104.6% | 104.6% | 100.1% | 100.2% |
| Class B | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |
| Class C | -0.19% | -0.18% | -0.13% | 104.8% | 104.7% | 100.1% | 100.1% |
| Class E | -0.37% | -0.14% | -0.26% | 105.2% | 105.1% | 100.1% | 100.1% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |
| Class D | -0.12% | -0.19% | 0.08% | 102.4% | 102.9% | 100.1% | 100.2% |
| Class F | -0.88% | -1.07% | -0.65% | 105.4% | 106.6% | 100.1% | 100.2% |
| Class TGM | -1.63% | -1.51% | -1.54% | 103.2% | 106.7% | 100.0% | 100.1% |

Table 2: Coding performance of proposed method

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over anchor |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |
| Class C | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |
| Class E | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |
| Class D | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |
| Class F | -1.13% | -1.11% | -0.83% | 106.7% | 108.3% | 99.5% | 100.1% |
| Class TGM | -2.22% | -2.08% | -2.08% | 104.4% | 107.9% | 100.0% | 100.0% |

- Conclusions

In this contribution, EBVP is introduced into IntraTMP merge candidate list construction. Results from this contribution demonstrate that the proposed method brings BD-Rate coding gains compared to ECM-11.0 and EE2-1.7b+EE2-1.8. It is recommended to study this method in next round of ECM EE2.

References

[1] https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tags/ECM-11.0

[2] Karam Naser, Fabrice Le Léannec, “IntraTMP with Merge Candidates” JVET-AF0137, Oct. 2023.

[3] F. Le Léannec, K. Naser, “Intra TMP extension to DIMD, LIC and SGPM” JVET-AF0275, Oct. 2023.

[4] Na Zhang, Kai Zhang, Li Zhang, “Auto relocated block vector prediction” JVET-AF0129, Oct. 2023.

- Patent rights declaration(s)

Xidian University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for the implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).