| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0088-v1 |
| --- | --- |

| Title: | AHG9: Usage of the neural-network post-filter characteristics SEI message to define the generator NN of the generative face video SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela, Francesco Cricri, Honglei Zhang Nokia Technologies, Tampere, Finland | Email: | {miska.hannuksela, francesco.cricri, honglei.1.zhang}@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution proposes to enable the use of a neural-network post-filter (NNPF) defined in the NNPF characteristics (NNPFC) SEI message as the generator neural network (GenerativeNN) of the generative face video (GFV) SEI message design proposed in JVET-AF0234. Ingredients of this proposal include the following:

- The use of the NNPF auxiliary input mechanism to input face parameters and matrices to the NNPF inference, including the following aspects:

  - nnpfc_auxiliary_inp_idc & 2 greater than 1 specifies that the auxiliary input is derived from SEI message(s) of a given payload type value (nnpfc_sei_aux_payload_type).

  - When nnpfc_sei_aux_payload_type is equal to the payload type value of the GFV SEI message, nnpfc_sei_aux_id is present and indicates the gfv_id value of the GFV SEI message for which this NNPF serves as the GenerativeNN.

  - When nnpfc_sei_aux_payload_type is equal to the payload type value of the GFV SEI message, the auxiliary inputs are inputDriveKeyPoint, inputDriveMatrix, inputBaseKeyPoint, and inputBaseMarix as derived in the semantics of the GFV SEI message.

- Addition of generative modelling among the purposes indicated by nnpfc_purpose.

  - Generative modelling is proposed to be added as the next available bit in nnpfc_purpose.

  - It is proposed to constrain generative modelling and general visual quality enhancement to be mutually exclusive.

# Introduction

## Motivation

The generative face video (GFV) SEI message proposed in JVET-AF0234 [1] utilizes a generator neural network (GenerativeNN) to create a picture derived from a base picture, a drive picture (optional), and the face key points and matrices derived from the GFV SEI messages of the base picture and the current picture. The specific GenerativeNN is not identified in the GFV SEI message.

It is asserted that better interoperability can be achieved by letting the encoder to specify which specific GenerativeNN the decoder is intended to use. This contribution proposes a mechanism to provide means for the encoder to identify GenerativeNN in the bitstream so that the decoder is able to use the intended GenerativeNN.

## Use of the neural-network post-filter characteristics (NNPFC) SEI message to define the GenerativeNN

The neural-network post-filter characteristics (NNPFC) SEI message provides the formatting of the input tensors to the neural network inference, the formatting of the output tensors resulting from the neural network inference, and characterization of the complexity of the neural network. Since these features are asserted to be useful for the GenerativeNN, this contribution proposes to use the NNPFC SEI message to define the GenerativeNN. The GenerativeNN is hence regarded as a neural-network post-filter (NNPF).

## Use of the NNPF auxiliary input mechanism

The NNPFC SEI message provides the extensible auxiliary input mechanism, which enables to input data other than the input pictures to the neural network inference. This contribution proposes to use the auxiliary input mechanism to pass the face key points and matrices to the inference of GenerativeNN defined as an NNPF.

The value range of nnpfc_auxiliary_inp_idc is specified to be up to 255, inclusive. In VSEI version 3, only one auxiliary input type, namely the QP-based StrengthControlVal[ i ], was defined, which is indicated by nnpfc_auxiliary_inp_idc equal to 1. It is envisioned that it can be useful to specify nnpfc_auxiliary_inp_idc as a bit field to allow mixed use of auxiliary input types. The value range for specifying auxiliary input types is relatively limited. Consequently, rather than specifying an auxiliary input type specifically for the GFV SEI message only, this contribution proposes a generic auxiliary input type for deriving auxiliary input from another SEI message that is identified by its payload type value (nnpfc_sei_aux_payload_type). Thus, if any other future SEI message is used to specify auxiliary input for an NNPF defined in a NNPFC SEI message, the same auxiliary input type can be used. It is proposed to use bit position 2 for the proposed auxiliary input type, i.e., when nnpfc_auxiliary_inp_idc & 2 is greater than 1, the auxiliary input derived from SEI messages of payload type value nnpfc_sei_aux_payload_type is in use for the NNPF inference.

Additional syntax elements that are gated by the payload type value nnpfc_sei_aux_payload_type can be present in the NNPFC SEI message. For nnpfc_sei_aux_payload_type equal to the payload type value of the GFV SEI message (denoted as constant SEI_GFV_PAYLOAD_TYPE), it is proposed to include nnpfc_sei_aux_id, which indicates the gfv_id value of the GFV SEI message for which this NNPF serves as the GenerativeNN.

## Generative modelling as NNPF purpose

Since the GenerativeNN generates a picture based on the input picture(s) and the auxiliary input, the output of the GenerativeNN cannot be considered as visual quality enhancement and none of the existing nnpfc_purpose definitions is suitable. It is therefore proposed to define a new purpose for generative modelling, which is proposed to be mutually exclusive to "general visual quality enhancement" (nnpfc_purpose & 0x01 greater than 0).

# Proposed specification text

## Use of the NNPF auxiliary input mechanism

| … |  |
| --- | --- |
| nnpfc_auxiliary_inp_idc | ue(v) |
| if( nnpfc_auxiliary_inp_idc & 2 ) { |  |
| nnpfc_sei_aux_payload_type | u(16) |
| if( nnpfc_sei_aux_payload_type = = SEI_GFV_PAYLOAD_TYPE ) |  |
| nnpfc_sei_aux_id | ue(v) |
| } |  |
| nnpfc_inp_order_idc | ue(v) |
| … |  |

When ( nnpfc_auxiliary_inp_idc & 2 ) is equal to 2 and nnpfc_sei_aux_payload_type is equal to SEI_GFV_PAYLOAD_TYPE, nnpfc_num_input_pics_minus1 shall be equal to 1.

…

nnpfc_auxiliary_inp_idc greater than 0 indicates that auxiliary input data is present in the input tensor of the NNPF or auxiliary input tensor(s) are provided to the NNPF. nnpfc_auxiliary_inp_idc equal to 0 indicates that auxiliary input data is not present in the input tensor and no auxiliary input tensors are provided to the NNPF.

( nnpfc_auxiliary_inp_idc & 1 ) equal to 1 specifies that auxiliary input data is derived as specified in Equation 95.

( nnpfc_auxiliary_inp_idc & 2 ) equal to 2 specifies that auxiliary input data or auxiliary input tensor(s) are derived based on a given payload type value of an SEI message and a given identifier value.

The value of nnpfc_auxiliary_inp_idc shall be in the range of 0 to 255, inclusive. Values of 24 to 255, inclusive, for nnpfc_auxiliary_inp_idc are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders conforming to this version of this Specification shall ignore NNPFC SEI messages with nnpfc_auxiliary_inp_idc in the range of 24 to 255, inclusive.

nnpfc_sei_aux_payload_type specifies the SEI message payload type value that is used to derive auxiliary input data or auxiliary input tensor(s) to the NNPF. nnpfc_sei_aux_payload_type values not equal to SEI_GFV_PAYLOAD_TYPE are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders conforming to this version of this Specification shall ignore NNPFC SEI messages with nnpfc_sei_aux_payload_type not equal to SEI_GFV_PAYLOAD_TYPE.

nnpfc_sei_aux_id specifies an identifying number that may be used to identify which one(s) of the SEI messages of payload type equal to nnpfc_sei_aux_payload_type are used to derive auxiliary input data or auxiliary input tensor(s) to the NNPF. nnpfc_sei_aux_id shall be in the range of 0 to 232 − 2, inclusive. When nnpfc_sei_aux_payload_type is equal to SEI_GFV_PAYLOAD_TYPE, it is a requirement of bitstream conformance that when a generative face video SEI message with gfv_id equal to nnpfc_sei_aux_id is present in a picture unit, the picture unit shall include an NNPFA SEI message with nnpfa_target_id equal to nnpfc_id.

…

When ( nnpfc_auxiliary_inp_idc & 2 ) is equal to 2 and nnpfc_sei_aux_payload_type is equal to SEI_GFV_PAYLOAD_TYPE, the following applies:

–	inputBaseY, inputBaseCb (when applicable), inputBaseCr (when applicable), inputBaseKeyPoint, and inputBaseMatrix are derived as specified for the semantics of the generative face video SEI message that has gfv_id equal to nnpfc_sei_aux_id and gfv_base_pic_flag equal to 1. If there are multiple such generative face video SEI messages present, the derivation is based on the last one preceding, in decoding order, the picture unit containing an NNPFA SEI message with nnpfa_target_id equal to nnpfc_id used for activating the NNPF.

–	The associated generative face video SEI message is the generative face video SEI message that has gfv_id equal to nnpfc_sei_aux_id and is present in the picture unit that contains an NNPFA SEI message with nnpfa_target_id equal to nnpfc_id. Subsequently, the syntax elements of the generative face video SEI message refer to the associated generative face video SEI message.

–	inputDriveKeyPoint and inputDriveMatrix are derived as specified for the semantics of the associated generative face video SEI message.

–	When gfv_drive_pic_fusion_flag is equal to 0, sample values of CroppedYPic[ 0 ], CroppedCbPic[ 0 ] (when applicable), and CroppedCrPic[ 0 ] (when applicable) are all set equal to 0.

NOTE – An input picture to the NNPF that has sample arrays with sample value 0 is a dummy picture as defined in the generative face video SEI message.

–	The input picture with index 1 is overwritten with the base picture as defined in the generative face video SEI message as follows: CroppedYPic[ 1 ] is set equal to inputBaseY, CroppedCbPic[ 1 ] is set equal to inputBaseCb (when applicable), and CroppedCrPic[ 1 ] is set equal to inputBaseCr.

The process DeriveInputTensors( ), for deriving the input tensor inputTensor for a given vertical sample coordinate cTop and a horizontal sample coordinate cLeft specifying the top-left sample location for the patch of samples included in the input tensor, the number of auxiliary input tensors numAuxInputTensors, and the auxiliary input tensors auxInputTensor[ j ], if any, for each value of j in the range of 0 to numAuxInputTensors − 1, inclusive, is specified as follows:

if( ( nnpfc_auxiliary_inp_idc & 2 ) &&
		( nnpfc_sei_aux_payload_type  = =  SEI_GFV_PAYLOAD_TYPE ) )
		numAuxInputTensors = 2
	else
		numAuxInputTensors = 0
for( i = 0; i < numInputPics; i++ ) {
	if( nnpfc_inp_order_idc  = =  0 )
		for( yP = −nnpfc_overlap; yP < inpPatchHeight + nnpfc_overlap; yP++)
			for( xP = −nnpfc_overlap; xP < inpPatchWidth + nnpfc_overlap; xP++ ) {
				inpVal = InpY( InpSampleVal( cTop + yP, cLeft + xP, CroppedHeight,
						CroppedWidth, CroppedYPic[ i ], 0 ) )
				yPovlp = yP + nnpfc_overlap
				xPovlp = xP + nnpfc_overlap
				if( !nnpfc_component_last_flag )
					inputTensor[ 0 ][ i ][ 0 ][ yPovlp ][ xPovlp ] = inpVal
				else
					inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 0 ] = inpVal
				if( nnpfc_auxiliary_inp_idc  = =  1 )
					if( !nnpfc_component_last_flag )
						inputTensor[ 0 ][ i ][ 1 ][ yPovlp ][ xPovlp ] = strengthControlScaledVal[ i ]
					else
						inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 1 ] = strengthControlScaledVal[ i ]
			}
	else if( nnpfc_inp_order_idc  = =  1 )		(96)
		for( yP = −nnpfc_overlap; yP < inpPatchHeight + nnpfc_overlap; yP++)
			for( xP = −nnpfc_overlap; xP < inpPatchWidth + nnpfc_overlap; xP++ ) {
				inpCbVal = InpC( InpSampleVal( cTop + yP, cLeft + xP, CroppedHeight / SubHeightC,
						CroppedWidth / SubWidthC, CroppedCbPic[ i ], 1 ) )
				inpCrVal = InpC( InpSampleVal( cTop + yP, cLeft + xP, CroppedHeight / SubHeightC,
						CroppedWidth / SubWidthC, CroppedCrPic[ i ], 2 ) )
				yPovlp = yP + nnpfc_overlap
				xPovlp = xP + nnpfc_overlap
				if( !nnpfc_component_last_flag ) {
					inputTensor[ 0 ][ i ][ 0 ][ yPovlp ][ xPovlp ] = inpCbVal
					inputTensor[ 0 ][ i ][ 1 ][ yPovlp ][ xPovlp ] = inpCrVal
				} else {
					inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 0 ] = inpCbVal
					inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 1 ] = inpCrVal
				}
				if( nnpfc_auxiliary_inp_idc  = =  1 )
					if( !nnpfc_component_last_flag )
						inputTensor[ 0 ][ i ][ 2 ][ yPovlp ][ xPovlp ] = strengthControlScaledVal[ i ]
					else
						inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 2 ] = strengthControlScaledVal[ i ]
			}
	else if( nnpfc_inp_order_idc  = =  2 )
		for( yP = −nnpfc_overlap; yP < inpPatchHeight + nnpfc_overlap; yP++)
			for( xP = −nnpfc_overlap; xP < inpPatchWidth + nnpfc_overlap; xP++ ) {
				yY = cTop + yP
				xY = cLeft + xP
				yC = yY / SubHeightC
				xC = xY / SubWidthC
				inpYVal = InpY( InpSampleVal( yY, xY, CroppedHeight,
						CroppedWidth, CroppedYPic[ i ], 0 ) )
				inpCbVal = InpC( InpSampleVal( yC, xC, CroppedHeight / SubHeightC,
						CroppedWidth / SubWidthC, CroppedCbPic[ i ], 1 ) )
				inpCrVal = InpC( InpSampleVal( yC, xC, CroppedHeight / SubHeightC,
						CroppedWidth / SubWidthC, CroppedCrPic[ i ], 2 ) )
				yPovlp = yP + nnpfc_overlap
				xPovlp = xP + nnpfc_overlap
				if( !nnpfc_component_last_flag ) {
					inputTensor[ 0 ][ i ][ 0 ][ yPovlp ][ xPovlp ] = inpYVal
					inputTensor[ 0 ][ i ][ 1 ][ yPovlp ][ xPovlp ] = inpCbVal
					inputTensor[ 0 ][ i ][ 2 ][ yPovlp ][ xPovlp ] = inpCrVal
				} else {
					inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 0 ] = inpYVal
					inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 1 ] = inpCbVal
					inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 2 ] = inpCrVal
				}
				if( nnpfc_auxiliary_inp_idc  = =  1 )
					if( !nnpfc_component_last_flag )
						inputTensor[ 0 ][ i ][ 3 ][ yPovlp ][ xPovlp ] = strengthControlScaledVal[ i ]
					else
						inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 3 ] = strengthControlScaledVal[ i ]
			}
	else if( nnpfc_inp_order_idc  = =  3 )
		for( yP = −nnpfc_overlap; yP < inpPatchHeight + nnpfc_overlap; yP++)
			for( xP = −nnpfc_overlap; xP < inpPatchWidth + nnpfc_overlap; xP++ ) {
				yTL = cTop + yP * 2
				xTL = cLeft + xP * 2
				yBR = yTL + 1
				xBR = xTL + 1
				yC = cTop / 2 + yP
				xC = cLeft / 2 + xP
				inpTLVal = InpY( InpSampleVal( yTL, xTL, CroppedHeight,
						CroppedWidth, CroppedYPic[ i ], 0 ) )
				inpTRVal = InpY( InpSampleVal( yTL, xBR, CroppedHeight,
						CroppedWidth, CroppedYPic[ i ], 0 ) )
				inpBLVal = InpY( InpSampleVal( yBR, xTL, CroppedHeight,
						CroppedWidth, CroppedYPic[ i ], 0 ) )
				inpBRVal = InpY( InpSampleVal( yBR, xBR, CroppedHeight,
						CroppedWidth, CroppedYPic[ i ], 0 ) )
				inpCbVal = InpC( InpSampleVal( yC, xC, CroppedHeight / 2,
						CroppedWidth / 2, CroppedCbPic[ i ], 1 ) )
				inpCrVal = InpC( InpSampleVal( yC, xC, CroppedHeight / 2,
						CroppedWidth / 2, CroppedCrPic[ i ], 2 ) )
				yPovlp = yP + nnpfc_overlap
				xPovlp = xP + nnpfc_overlap
				if( !nnpfc_component_last_flag ) {
					inputTensor[ 0 ][ i ][ 0 ][ yPovlp ][ xPovlp ] = inpTLVal
					inputTensor[ 0 ][ i ][ 1 ][ yPovlp ][ xPovlp ] = inpTRVal
					inputTensor[ 0 ][ i ][ 2 ][ yPovlp ][ xPovlp ] = inpBLVal
					inputTensor[ 0 ][ i ][ 3 ][ yPovlp ][ xPovlp ] = inpBRVal
					inputTensor[ 0 ][ i ][ 4 ][ yPovlp ][ xPovlp ] = inpCbVal
					inputTensor[ 0 ][ i ][ 5 ][ yPovlp ][ xPovlp ] = inpCrVal
				} else {
					inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 0 ] = inpTLVal
					inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 1 ] = inpTRVal
					inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 2 ] = inpBLVal
					inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 3 ] = inpBRVal
					inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 4 ] = inpCbVal
					inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 5 ] = inpCrVal
				}
				if( nnpfc_auxiliary_inp_idc  = =  1 )
					if( !nnpfc_component_last_flag )
						inputTensor[ 0 ][ i ][ 6 ][ yPovlp ][ xPovlp ] = strengthControlScaledVal[ i ]
					else
						inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 6 ] = strengthControlScaledVal[ i ]
			}
	if( ( nnpfc_auxiliary_inp_idc & 2 ) &&
		( nnpfc_sei_aux_payload_type  = =  SEI_GFV_PAYLOAD_TYPE ) ) {		if( i  = =  0 ) {
			auxInputTensor0[ 0 ][ i ] = inputDriveKeyPoint
			auxInputTensor1[ 0 ][ i ] = inputDriveMatrix
		} else { // i  = =  1, base picture
			auxInputTensor0[ 0 ][ i ] = inputBaseKeyPoint
			auxInputTensor1[ 0 ][ i ] = inputBaseMatrix
		}
	}
}

…

An NNPF PostProcessingFilter( ) is the target NNPF as derived in the semantics of the NNPFA SEI message. The NNPF PostProcessingFilter( ) inputs inputTensor, followed by numAuxInputTensors tensors auxInputTensorj where j is in the range of 0 to numAuxInputTensors − 1, inclusive. The following example process may be used, with the NNPF PostProcessingFilter( ), to generate, in a patch-wise manner, the filtered and/or interpolated picture(s), which contain Y, Cb, and Cr sample arrays FilteredYPic, FilteredCbPic, and FilteredCrPic, respectively, as indicated by nnpfc_out_order_idc:

if( nnpfc_inp_order_idc  = =  0  | |  nnpfc_inp_order_idc  = =  2 )
	for( cTop = 0; cTop < CroppedHeight; cTop  +=  inpPatchHeight )
		for( cLeft = 0; cLeft < CroppedWidth; cLeft  +=  inpPatchWidth ) {
			inputTensor = DeriveInputTensors( )
			outputTensor = PostProcessingFilter( inputTensor )
			StoreOutputTensors( outputTensor )
		}
else if( nnpfc_inp_order_idc  = =  1 )
	for( cTop = 0; cTop < CroppedHeight / SubHeightC; cTop  +=  inpPatchHeight )
		for( cLeft = 0; cLeft < CroppedWidth / SubWidthC; cLeft  +=  inpPatchWidth ) {	(98)
			inputTensor = DeriveInputTensors( )
			outputTensor = PostProcessingFilter( inputTensor )
			StoreOutputTensors( outputTensor )
		}
else if( nnpfc_inp_order_idc  = =  3 )
	for( cTop = 0; cTop < CroppedHeight; cTop  +=  inpPatchHeight * 2 )
		for( cLeft = 0; cLeft < CroppedWidth; cLeft  +=  inpPatchWidth * 2 ) {
			inputTensor = DeriveInputTensors( )
			outputTensor = PostProcessingFilter( inputTensor )
			StoreOutputTensors( outputTensor )
		}

## Generative modelling as NNPF purpose

nnpfc_purpose indicates the purpose of the NNPF as specified in Table 20, where ( nnpfc_purpose & bitMask ) not equal to 0 indicates that the NNPF has the purpose associated with the bitMask value in Table 20. When nnpfc_purpose is greater than 0 and ( nnpfc_purpose & bitMask ) is equal to 0, the purpose associated with the bitMask value is not applicable to the NNPF. When nnpfc_pupose is equal to 0, the NNPF may be used as determined by the application.

All NNPFC SEI messages with a particular value of nnpfc_id within a CLVS shall have the same value of nnpfc_purpose.

The value of nnpfc_purpose shall be in the range of 0 to 127 63, inclusive, in bitstreams conforming to this version of this Specification. Values of 128 64 to 65 535, inclusive, for nnpfc_purpose are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders conforming to this version of this Specification shall ignore NNPFC SEI messages with nnpfc_purpose in the range of 128 64 to 65 535, inclusive.

Table 20 – Definition of nnpfc_purpose

| bitMask | Interpretation |
| --- | --- |
| 0x01 | General visual quality improvement |
| 0x02 | Chroma upsampling (from the 4:2:0 chroma format to the 4:2:2 or 4:4:4 chroma format, or from the 4:2:2 chroma format to the 4:4:4 chroma format) |
| 0x04 | Resolution resampling (increasing or decreasing the width or height) |
| 0x08 | Picture rate upsampling |
| 0x10 | Bit depth upsampling (increasing the luma bit depth or the chroma bit depth) |
| 0x20 | Colourization |
| 0x40 | Generative modelling of NNPF output picture(s) |

The variables GeneralVisualQualityImprovementFlag, ChromaUpsamplingFlag, ResolutionResamplingFlag, PictureRateUpsamplingFlag, BitDepthUpsamplingFlag, and ColourizationFlag, and GenerativeModellingFlag, specifying whether nnpfc_purpose indicates the purpose of the NNPF to include general visual quality improvement, chroma upsampling, resolution resampling, picture rate upsampling, bit depth upsampling, and colourization, and generative modelling, respectively, are derived as follows:

GeneralVisualQualityImprovementFlag = ( nnpfc_purpose & 0x01 )
ChromaUpsamplingFlag = ( ( nnpfc_purpose & 0x02 ) > 0 ) ? 1 : 0
ResolutionResamplingFlag = ( ( nnpfc_purpose & 0x04 ) > 0 ) ? 1 : 0
PictureRateUpsamplingFlag = ( ( nnpfc_purpose & 0x08 ) > 0 ) ? 1 : 0	(75)
BitDepthUpsamplingFlag = ( ( nnpfc_purpose & 0x10 ) > 0 ) ? 1 : 0
ColourizationFlag = ( ( nnpfc_purpose & 0x20 ) > 0 ) ? 1 : 0
GenerativeModellingFlag = ( ( nnpfc_purpose & 0x40 ) > 0 ) ? 1 : 0

NOTE 2– When a reserved value of nnpfc_purpose is taken into use in the future by ITU-T | ISO/IEC, the syntax of this SEI message could be extended with syntax elements whose presence is conditioned by nnpfc_purpose being equal to that value or any one of a set of values including that value.

NOTE 3– GenerativeModellingFlag equal to 1 indicates that the NNPF output picture(s) do not represent the same visual content as the input picture(s) but are generated by the NNPF based on the input pictures and auxiliary inputs, if any.

When ChromaFormatIdc is equal to 3, ChromaUpsamplingFlag shall be equal to 0.

When ChromaUpsamplingFlag is equal to 1, ColourizationFlag shall be equal to 0.

When GeneralVisualQualityImprovementFlag is equal to 1, GenerativeModellingFlag shall be equal to 0.

# References

- B. Chen, J. Chen, Y. Ye (Alibaba), S. Wang (CityU), S. McCarthy, P. Yin, G.-M. Su, A. K. Choudhury, W. Husak, G. J. Sullivan (Dolby), "AHG9: Common text for proposed generative face video SEI message", JVET-AF0234.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).