| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0096-v1 |
| --- | --- |

| Title: | Non-EE2: On temporal buffer handling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhipin Deng1, Kai Zhang2, Lei Zhao1, Li Zhang2 1Zijinshumayuan 4th building F9 18 Zhongguancun Nansijie Haidian District, 100190, Beijing, China 28910 University Center Lane San Diego, CA 92122, USA | Tel: Email: | {zhipin.deng, zhangkai.video, zhaolei.0324, lizhang.idm} @bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

In ECM-11.0, some coding tools can access larger than one CTU of temporal buffer, but some coding tools follow the one CTU temporal buffer restriction. This contribution proposes to remove the one CTU temporal buffer restriction for all coding tools in ECM. The impact on coding efficiency and runtimes of the proposed test over ECM-11.0 are reportedly for {Y, U, V, EncT, DecT}:

RA { -0.02%, -0.04%, -0.01%, 100.3%, 100.4% }; LDB { -0.21%, 0.10%, 0.10%, 100.3%, 100.2% }

# Introduction

In VVC, the temporal motion vector candidate is restricted not to exceed the collocated CTU plus four sample columns right to the collocated CTU. In ECM, such restriction is firstly relaxed to the CTU row along with the adoption of TMVP/sbTMVP extension, and then further relaxed to the full reference picture range along with the adoption of temporal CCP candidate of CCP merge mode. However, in ECM-11.0, different temporal buffer restrictions are used by different coding tools, as listed in Table 1.

Table 1. The temporal buffer requirement of coding tools in ECM-11.0

| Coding methods in ECM-11.0 | Require more than one CTU row buffer? |
| --- | --- |
| intraCCP merge: temporal CCP candidate derivation | Yes |
| interCCP merge: temporal CCP candidate derivation | Yes |
| Inter merge/AMVP: TMVP candidate derivation | No |
| BM merge: TMVP candidate derivation | No |
| affine merge/AMVP: TMVP candidate derivation | No |
| affine BM merge/AMVP: TMVP candidate derivation | No |
| sbTMVP: motion shift derivation | No |
| Inter propagated IPM/CCP-model derivation | No |

# Proposed method

In this contribution, it is proposed to remove the temporal buffer restriction for all coding tools in ECM in a consistent way.

# Experimental results

The proposed method is implemented on top of ECM-11.0 [1] and tested following the common test conditions [2]. The simulation results of the test compared to ECM-11.0 are summarized in Table 2.

Table 2: Coding performance of the proposed test

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.01% | -0.15% | -0.11% | 100.4% | 100.1% | 100.1% | 100.5% |
| Class A2 | -0.02% | 0.03% | -0.07% | 100.5% | 100.7% | 100.5% | 99.9% |
| Class B | -0.01% | -0.06% | 0.07% | 100.1% | 100.4% | 99.4% | 100.2% |
| Class C | -0.04% | 0.00% | 0.00% | 100.5% | 100.3% | 99.6% | 100.1% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.02% | -0.04% | -0.01% | 100.3% | 100.4% | 99.8% | 100.2% |
| Class D | 0.00% | 0.12% | 0.10% | 99.9% | 101.1% | 100.0% | 100.0% |
| Class F | -0.04% | 0.12% | -0.04% | 100.5% | 100.3% | 99.6% | 100.0% |
| Class TGM | -0.08% | -0.13% | -0.10% | 99.9% | 100.3% | 100.6% | 100.1% |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.16% | 0.19% | 0.46% | 100.1% | 100.5% | 99.8% | 100.1% |
| Class C | -0.07% | 0.09% | -0.46% | 100.6% | 100.1% | 100.4% | 100.0% |
| Class E | -0.47% | -0.03% | 0.26% | 100.5% | 99.9% | 101.2% | 99.7% |
| Overall | -0.21% | 0.10% | 0.10% | 100.3% | 100.2% | 100.4% | 100.0% |
| Class D | -0.08% | -0.15% | -0.41% | 99.6% | 100.7% | 100.0% | 100.0% |
| Class F | -0.09% | -0.13% | -0.15% | 100.7% | 101.3% | 99.2% | 100.0% |
| Class TGM | -0.16% | -0.18% | -0.20% | 99.9% | 100.5% | 99.6% | 99.0% |

# Conclusions

The proposed method can achieve decent BD-rate gains with negligible runtime change. It is recommended to include the proposed methods in the next EE2 for a further investigation.

# References

- ECM-11.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-11.0.

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-AF2017, Oct. 2023.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).