| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0097-v2 |
| --- | --- |

| Title: | EE2-2.12: Adjusting out-of-boundary prediction samples |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Pekka Astola Jani Lainema | Tel: Email: | - first.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution reports the results on the EE2-2.12, where it is proposed to improve the out-of-boundary (OOB) prediction by adjusting the uni-prediction samples using a scaling parameter and an offset to better match with the bi-prediction samples. The scale and offset are derived using the uni-prediction samples and the averaged bi-prediction samples.

The impact on coding efficiency and runtimes over ECM-10.0 is reportedly RA {-0.03, -0.01%, -0.01%, 100.1% 99.3%, 100.0%, 99.8%}, LB {-0.04%, 0.20%, 0.22%, 99.7%, 99.3%, 99.6%, 99.6%} {for Y, U, V, EncT, DecT, EncVmPeak, DecVmPeak}.

# Proposed method

## Introduction – OOB prediction in ECM-11.0

The following summarizes the current OOB prediction in ECM-11.0. In bi-directional motion compensation the out of boundary (OOB) prediction samples are discarded and only the non-OOB predictors, when available, are used to generate the final predictor. Specifically, let Pos_xi,j and Pos_yi,j denote the position of one prediction sample in one current block, Mv_xi,jLx and Mv_yi,jLx (x = 0,1) denote the MV of the current block; PosLeftBdry, PosRightBdry, PosTopBdry and PosBottomBdry are the positions of four boundaries of the picture. One prediction sample is regarded as OOB when at least one of the following conditions is satisfied:

(Pos_xi,j+Mv_xi,jLx)>(PosRightBdry+half_pixel),

(Pos_xi,j+Mv_xi,jLx)<(PosLeftBdry- half_pixel),

(Pos_yi,j+Mv_yi,jLx)>(PosBottomBdry+ half_pixel),

(Pos_yi,j+Mv_yi,jLx)<(PosTopBdry- half_pixel)

where half_pixel is equal to 8 that represents the half-pel sample distance in the 1/16-pel sample precision.

After examining the OOB condition for each sample, the final prediction samples of one bi-directional block are generated as follows:

If Pi,jL0 is OOB and Pi,jL1 is non-OOB

Pi,jfinal=Pi,jL1

else if Pi,jL0 is non-OOB and Pi,jL1 is OOB

Pi,jfinal=Pi,jL0

else

Pi,jfinal=(Pi,jL0+Pi,jL1+1)>>1.

OOB checking process is also applicable when BCW is enabled.

## The proposed method (EE2-2.12)

In the proposed method the final prediction samples for luma are generated as follows,

if Pi,jL0 is OOB and Pi,jL1 is non-OOB

Pi,jfinal=(Pi,jL1*1+a1+b1+1)>>1

else if Pi,jL0 is non-OOB and Pi,jL1 is OOB

Pi,jfinal=Pi,jL0*1+a0+b0+1>>1

else

Pi,jfinal=(Pi,jL0+Pi,jL1+1)>>1.

The scales and offsets a0,a1,b0, b1 are derived over the non-OOB parts of PL0 and PL1 using least squares minimization,

mina0,b0 (i,j)∈Ωn(Pi,jfinal-(a0*Pi,jL0+b0))2

and

mina1,b1 (i,j)∈Ωn(Pi,jfinal-(a1*Pi,jL1+b1))2

where Ωn are the non-OOB samples of PL0 and PL1. Scales and offsets are derived using sample covariance and sample variance at integer precision. Only one set of parameters are derived, as in almost all cases only PL0 or PL1 have OOB samples – the case other case (when both predictors have OOB samples) is not considered and the default method of ECM-11.0 is applied as described in Section 1.1. The proposed method is also applicable when BCW is enabled and in such cases samples of Pfinal are derived accordingly using the BCW weights. The proposed method is applied to luma only. Chroma uses the default method described in Section 1.1.

# Coding efficiency analysis

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.05 % | -0.02 % | 0.09 % | 100.9% | 99.0% | 100.0% | 99.8% |
| Class A2 | -0.01 % | 0.02 % | -0.03 % | 100.8% | 99.0% | 100.1% | 99.6% |
| Class B | -0.04 % | 0.04 % | -0.04 % | 99.8% | 99.5% | 100.0% | 99.9% |
| Class C | -0.02 % | -0.09 % | -0.02 % | 99.5% | 99.7% | 99.9% | 100.0% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.03 % | -0.01 % | -0.01 % | 100.1% | 99.3% | 100.0% | 99.8% |
| Class D | -0.08 % | -0.02 % | -0.18 % | 99.6% | 99.8% | 100.2% | 100.0% |
| Class F | -0.02 % | -0.02 % | -0.05 % | 99.7% | 100.7% | 100.3% | 100.0% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | 0.01 % | 0.44 % | 0.29 % | 100.0% | 99.0% | 100.0% | 99.8% |
| Class C | 0.05 % | 0.43 % | -0.19 % | 99.9% | 99.6% | 99.6% | 99.8% |
| Class E | -0.23 % | -0.50 % | 0.66 % | 99.0% | 99.2% | 98.9% | 99.2% |
| Overall | -0.04 % | 0.20 % | 0.22 % | 99.7% | 99.3% | 99.6% | 99.6% |
| Class D | -0.09 % | -0.69 % | -0.62 % | 99.7% | 100.5% | 99.2% | 100.0% |
| Class F | 0.06 % | -0.11 % | -0.23 % | 99.4% | 100.3% | 99.7% | 100.3% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |

# Conclusion

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).