| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0107-v2 |
| --- | --- |

| Title: | AHG9: On phase indication SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Takeshi Chujoh Tomohiro Ikai Kei Kawamura | Email: | chujoh.takeshi@sharp.co.jp ikai.tomohiro@sharp.co.jp ki-kawamura@kddi.com |
| Source: | Sharp Corporation and KDDI Corporation |  |  |

_____________________________

- Abstract

VVC can change the picture resolution in a sequence by using RPR; however, most video applications display decoded pictures with a constant resolution, even with decoded resolution changes dynamically. When the same phase information for the resampling process of RPR is not identified between the transmitting and receiving sides, pixels of the picture may shift in time when the resolution changes dynamically in the sequence. To avoid this phenomenon, the phase indication SEI message has been adopted in VSEI to share the phase information between the transmitting and receiving sides. The remaining problem is that the bitstreams, defined as the phase information implicitly but without the SEI, are not interoperable. This contribution proposes the following:

- The phase indication SEI message is moved to the VVC text, which is made as a normative SEI message.

- Syntax elements of the phase indication SEI message have the inferred values. Their values mean the co-sited positions. Then, even if there is no phase indication SEI message in the bitstream, the phase information is defined.

These proposals do not change the conformance bitstreams and the reference software.

Problem Statement

In VVC, the picture resolution in a sequence can be changed by using RPR (Reference Picture Re-sampling). The conformance point is the resized decoded picture, and the down-sampling of the transmitter and up-sampling of the receiver is not mandatory. However, most video applications display the decoded pictures with a constant resolution, even with dynamic resolution changes. When the same phase information for the resampling process is not identified between the transmitter and receiver, the pixel will be shifted in time if the resolution changes in a sequence dynamically. Therefore, the phase indication SEI message has been adopted in VSEI to share the phase information; however, this SEI message is not mandatory information.

If the application specification defines a sample position of the transmitter, the receiver can find it implicitly. This method is not a problem in closed systems; however, the bitstream generated by this method is not interoperable. The sample position of the bitstream without the phase indication SEI message is unknown, which is a problem. The current VTM software has this problem. Because, when RPR is used, the co-sited position is implicitly assumed without the phase indication SEI message. This implementation is not defined in the VVC specification.

This contribution proposes two solutions. One solution is to move the phase indication SEI message to VVC and define a normative SEI message. The receiver should consider what resolution to display is outside the conformance point; however, the decoded pictures with dynamic resolution changes can not display correctly with the constant resolution without the phase information. Therefore, the bitstream has interoperability explicitly if the phase indication SEI message can be sent. The other solution is to specify a default sample position, and the bitstream with the default sample position without the phase indication SEI message has interoperability. The co-sited sample position should be inferred because the interpolation filters of video coding standards and down-/up-sampling in VTM software have adopted this position. Since most applications out of video codecs assume the center sample position, this specification, i.e., the co-sited position, shall be explicitly specified in the VVC text.

Proposal

This contribution proposes two solutions, which are independent and have different effects but are mutually related. Proposal 1 moves the text of the phase indication SEI to the VVC text and defines the inferred values of the syntax elements. Proposal 2 only defines the inferred values of the syntax elements. Neither proposal changes the reference software and the conformance bitstreams.

Proposal 1

The phase indication SEI message is moved to Annex D.11 of VVC, and the syntax elements of the phase indication SEI message are defined as the inferred values; the following specification is added at the end of Annex D.11.2. When there is no phase indication SEI message, the phase information shall be defined as the co-sited position.

- D.11	Phase indication SEI message

- D.11.1	Phase indication SEI message syntax

- D.11.2	Phase indication SEI message semantics

When there is no phase indication SEI message, the syntax elements pi_hor_phase_num, pi_hor_phase_den_minus1, pi_ver_phase_num, and pi_ver_phase_den_minus1 are inferred to be equal to the minimum values that satisfy the following formulas unless determined by the application by external means:

–	pi_hor_phase_num / (pi_hor_phase_den_minus1 + 1) = CroppedWidth / (2 * OrgCroppedWidth)

–	pi_ver_phase_num / (pi_ver_phase_den_minus1 + 1) = CroppedHeight / (2 * OrgCroppedHeight)

Where variables OrgCroppedWidth and OrgCroppedHeight are specified as follows.

OrgCroppedWidth = sps_pic_width_max_in_luma_samples

− SubWidthC * ( sps_conf_win_left_offset + sps_conf_win_right_offset )

OrgCroppedHeight = sps_pic_height_max_in_luma_samples

− SubHeightC * ( sps_conf_win_top_offset + sps_conf_win_bottom_offset )

Proposal 2

If the phase indication SEI message is not moved to the VVC text and the syntax elements of the phase indication SEI message are defined as the inferred values, the following specification is added in Annex D.12.12. When there is no phase indication SEI message, the phase information shall be defined as the co-sited position.

- D.12.12	Use of the phase indication SEI message

For purposes of interpretation of the phase indication SEI message, the following variables are specified:

–	CroppedWidth is set equal to pps_pic_width_in_luma_samples − SubWidthC *  ( pps_conf_win_left_offset + pps_conf_win_right_offset ).

–	CroppedHeight is set equal to pps_pic_height_in_luma_samples − SubHeightC *  ( pps_conf_win_top_offset + pps_conf_win_bottom_offset ).

When there is no phase indication SEI message, the syntax elements pi_hor_phase_num, pi_hor_phase_den_minus1, pi_ver_phase_num, and pi_ver_phase_den_minus1 are inferred to be equal to the minimum values that satisfy the following formulas unless determined by the application by external means:

–	pi_hor_phase_num / (pi_hor_phase_den_minus1 + 1) = CroppedWidth / (2 * OrgCroppedWidth)

–	pi_ver_phase_num / (pi_ver_phase_den_minus1 + 1) = CroppedHeight / (2 * OrgCroppedHeight)

Where variables OrgCroppedWidth and OrgCroppedHeight are specified as follows.

OrgCroppedWidth = sps_pic_width_max_in_luma_samples

− SubWidthC * ( sps_conf_win_left_offset + sps_conf_win_right_offset )

OrgCroppedHeight = sps_pic_height_max_in_luma_samples

− SubHeightC * ( sps_conf_win_top_offset + sps_conf_win_bottom_offset )

Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

KDDI Corporation does not have any current or pending patent rights relating to the technology described in this contribution.