| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0112-v2 |
| --- | --- |

| Title: | EE2-2.11: Regression-based GPM blending (tests a,b,c) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Philippe Bordes, Kevin Reuzé, Franck Galpin, Fabrice Urban, Karam Naser, Fabrice Le Léannec, Edouard Francois | Email: | philippe.bordes@interdigital.com, kevin.reuze@interdigital.com, franck.galpin@interdigital.com, fabrice.urban@interdigital.com, karam.naser@interdigital.com, fabrice.leleannec@interdigital.com, edouard.francois@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This contribution reports the results of EE2 Test 2.11(a,b,c) on Regression-based GPM blending. In this enhanced GPM mode, initially presented in the contribution JVET-AF0118, the splitting shape and the blending function are derived from the template.

Three EE2 tests are presented. Test 2.11a corresponds to Regression-based GPM blending as initially proposed. Test 2.11b and Test 2.11c corresponds to variants where same syntax as regular GPM is used and regular GPM blending where single merge index is coded respectively. The tests reportedly provide {Y, U, V} BD-reduction with encoder and decoder runtimes as follows:

Test-2.11a: -0.08%, -0.13%, -0.19%, 100.8% (EncT), 98.2% (DecT), LDB: -0.11%, -0.12%, -0.15%, 101.0% (EncT), 98.1% (DecT).

Test-2.11b: -0.02%, -0.03%, -0.10%, 101.0% (EncT), 98.3% (DecT), LDB: xx%, xx%, xx%, xx% (EncT), xx% (DecT).

Test-2.11c: xx%, xx%, xx%, xx% (EncT), xx% (DecT), LDB: xx%, xx%, xx%, xx% (EncT), xx% (DecT).

# Proposal

The Geometric partitioning mode (GPM) allows splitting a CU into two parts by a geometrically located straight line. Each part of a geometric partition in the CU is inter-predicted or intra predicted.

When GPM is selected, a geometric partition index (split index) indicating the partition mode of the geometric partition (angle and offset), and two merge indices (one for each partition) are further signaled.

The final prediction samples are generated by blending the two prediction signals using weighted average. Two integer blending matrices (W0 and W1) are used. The weights in the GPM blending matrices are derived from the ramp function based on the displacement from a predicted sample position to the GPM partitioning boundary. The blending area size can extend up to 8 samples on each side of the GPM partition split boundary. A CU level flag is coded to signal the selected blending area size.

Lastly, template matching based reordering for GPM split modes is applied. Given the motion information of the current GPM block, the respective TM cost values of GPM split modes are computed. Then, GPM split modes are reordered in ascending ordering of TM cost values.

The EE2 Test 2.11 proposes an additional GPM implicit mode, where the two integer blending matrices (W0 and W1) are derived from the template (1 line above, 1 column left). The blending matrices are modelled as an affine linear function of the sample positions (x,y) in the current CU:

W0(x,y) = a.x + b.y + c  		and 	W1(x,y) = 1 - W0(x,y) 		(eq.1)

The parameters (a,b,c) are derived from the reference template using the same solver (MSE minimization) as the one used for CCCM, GLM or GL-CCCM. A list of pair of candidates is built from the regular GPM candidates and re-ordered with the template cost.

The GPM implicit mode is signaled by a CU-level flag (gpm_implicit_flag). If gpm_implicit_flag is true, a merge-idx is coded to signal the pair of GPM candidates to be used. If gpm_implicit_flag is false, the regular GPM syntax elements are signaled.

# Results

Three variants have been tested:

Test 2.11a (same as described above): a list of pair of candidates is built from the regular GPM candidates and re-ordered with the template cost. A single merge-idx is coded to signal the pair of GPM candidates to be used.

Test 2.11b: proposed GPM blending but two merge-idx are coded (one for each partition) without re-ordering, same as regular GPM syntax.

Test 2.11c: the regular GPM blending is used but a single merge-idx is coded to signal the pair of GPM candidates and the split to be used. Both pairs of candidates and split are re-ordered with template.

Test 2.11a

Table 1: Performance results of proposal in RA (test 2.11a).

|  | Random Access Main10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0,09% | -0,22% | -0,17% | 100,7% | 98,2% | 101,1% | 99,9% |
| Class A2 | -0,06% | -0,12% | -0,18% | 100,4% | 98,2% | 101,7% | 99,4% |
| Class B | -0,09% | -0,06% | -0,18% | 101,3% | 98,6% | 99,9% | 99,8% |
| Class C | -0,07% | -0,14% | -0,23% | 100,4% | 97,7% | 99,9% | 100,2% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0,08% | -0,13% | -0,19% | 100,8% | 98,2% | 100,5% | 99,8% |
| Class D | -0,04% | 0,14% | -0,06% | 101,0% | 97,6% | 99,5% | 100,1% |
| Class F | -0,09% | -0,08% | -0,20% | 101,1% | 97,8% | 100,6% | 101,0% |
| Class TGM | 0,00% | 0,00% | 0,00% | 100,9% | 98,1% | 101,8% | 100,3% |

Table 2: Performance results of proposal in LDB (test 2.11a).

|  | Low Delay B Main10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0,12% | 0,19% | -0,22% | 101,0% | 98,1% | 100,2% | 101,1% |
| Class C | -0,12% | -0,24% | -0,01% | 100,8% | 98,1% | 99,9% | 102,5% |
| Class E | -0,08% | -0,50% | -0,22% | 101,2% | 98,2% | 103,6% | 103,7% |
| Overall | -0,11% | -0,12% | -0,15% | 101,0% | 98,1% | 100,9% | 102,2% |
| Class D | -0,14% | -0,24% | -1,10% | 101,1% | 98,2% | 97,5% | 100,6% |
| Class F | -0,07% | -0,07% | 0,03% | 100,7% | 97,8% | 99,9% | 102,3% |
| Class TGM | 0,00% | 0,00% | 0,00% | 99,8% | 100,4% | 103,3% | 104,3% |

Test 2.11b

Table 1: Performance results of proposal in RA (test 2.11b).

|  | Random Access Main10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0,04% | -0,12% | -0,08% | 101,1% | 98,0% | 100,9% | 99,6% |
| Class A2 | -0,03% | 0,03% | -0,08% | 101,0% | 98,1% | 101,7% | 99,5% |
| Class B | -0,03% | -0,03% | -0,17% | 101,2% | 98,4% | 100,0% | 99,8% |
| Class C | 0,00% | -0,03% | -0,04% | 100,8% | 98,6% | 100,1% | 100,2% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0,02% | -0,03% | -0,10% | 101,0% | 98,3% | 100,5% | 99,8% |
| Class D | 0,04% | 0,13% | -0,02% | 100,6% | 98,0% | 99,3% | 99,6% |
| Class F |  |  |  |  |  |  |  |
| Class TGM |  |  |  |  |  |  |  |

Table 2: Performance results of proposal in LDB (test 2.11b).

|  | Low Delay B Main10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0,05% | 0,65% | 0,48% | 101,7% | 99,8% | 100,3% | 101,0% |
| Class C | 0,11% | 0,28% | -0,04% | 100,6% | 100,2% | 99,9% | 102,1% |
| Class E | -0,03% | 0,03% | -0,09% | 100,1% | 99,1% | 104,7% | 103,0% |
| Overall | 0,00% | 0,37% | 0,16% | 100,9% | 99,7% | 101,2% | 101,9% |
| Class D | -0,06% | -0,54% | -0,38% |  |  | 97,4% |  |
| Class F |  |  |  |  |  |  |  |
| Class TGM |  |  |  |  |  |  |  |

Test 2.11c

Table 1: Performance results of proposal in RA (test 2.11c).

|  | Random Access Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-11.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | 0,02% | 0,02% | -0,01% | 100,5% | 98,1% | 100,1% |
| Class C | 0,05% | 0,08% | 0,07% | 100,9% | 98,1% | 100,0% |
| Class E |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |
| Class D | 0,06% | 0,17% | -0,08% | 100,9% | 97,9% | 99,2% |
| Class F |  |  |  |  |  |  |
| Class TGM |  |  |  |  |  |  |

Table 2: Performance results of proposal in LDB (test 2.11c).

|  | Low Delay B Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-11.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |
| Class C |  |  |  |  |  |  |
| Class E |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |
| Class D |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |
| Class TGM |  |  |  |  |  |  |

# References

- Philippe Bordes, Franck Galpin, Karam Naser, Fabrice Urban, Kevin Reuzé, Fabrice Le Léannec, Edouard Francois, “Non-EE2: Regression-based GPM blending,” document JVET-AF0118, 32nd Meeting, Hannover, DE, 13–20 October 2023.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).