| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0145-v1 |
| --- | --- |

| Title: | EE2-4.1: Adaptive clipping with signalled lower and upper bounds |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kai Cui, Zhi Zhang, Han Huang, Vadim Seregin, Marta Karczewicz (Qualcomm) | Tel: Email: | kaicui@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution reports the experimental results of EE2-4.1a and EE2-4.1b. It proposes to apply adaptive clipping with signalled lower and upper bounds. For each picture, the minimum and maximum values of the Luma channel, which are derived from the MCTF pre-filtered picture (Test-4.1a) or the original picture (Test-4.1b), are signalled and used to perform clipping at the reconstruction stage. The proposed method was implemented on top of ECM-11.0 and the test results are summarized as follows.

Test4.1a (min/max values derived from the MCTF-prefiltered picture):

AI: Y: -0.06%, U: 0.03%, V: -0.01%, EncT: 99.4%, DecT: 99.6%, EncVmPeak: 100.0%, DecVmPeak: 100.0%

RA: Y: -0.08%, U: 0.00%, V: -0.09%, EncT: 99.7%, DecT: 100.1%, EncVmPeak: 99.7%, DecVmPeak: 100.0%

LDB: Y: -0.03%, U: 0.17%, V: 0.19%, EncT: 99.4%, DecT: 99.9%, EncVmPeak: 100.4%, DecVmPeak: 99.8%

Test4.1b (min/max values derived from the original picture):

AI: identical as AI results of Test4.1a since MCTF is disabled in AI

RA: Y: -0.06%, U: -0.01%, V: -0.06%, EncT: 100.0%, DecT: 100.4%, EncVmPeak: 99.6%, DecVmPeak: 99.8%

LDB: Y: -0.01%, U: 0.31%, V: 0.31%, EncT: 99.8%, DecT: 99.4%, EncVmPeak: 99.9%, DecVmPeak: 99.6%

# Introduction

In ECM-11.0, a bit-depth based clipping is applied to multiple places to avoid data overflow. The range of the bit-depth based clipping is defined as [0, 2^bitdepth-1]. The clipping operation is applied in filtering, sampling, interpolation, weighted prediction, weighted combination, reconstruction, and other stages, to ensure that the generated prediction and reconstruction samples remain in the defined dynamic range.

# Proposal

This contribution proposes to use the signalled minimum/maximum values of the luma channel as the clipping bounds. The min/max values are obtained by scanning the sample values in the original picture. If motion compensated temporal filter (MCTF) is applied to a picture, the min/max values are obtained by scanning the sample values in the MCTF pre-filtered picture. The min/max values are used as the lower and upper bounds for clipping at the reconstruction stage (when adding the residual signal to the prediction) and before saving the reconstructed picture into the decoded picture buffer.

The delta values for min/max are derived and quantized for each picture. For I pictures, the delta values between [64, 940] and the min/max values are derived and signalled in the picture header. For other pictures, the delta values compared to the signalled min/max values of the collocated picture are signalled in the picture header.

When clipping is performed in luma mapping chroma scaling (LMCS) mapped domain, the min/max values are derived by applying forward LMCS look-up table to the signalled min/max values.

# Simulation Results

The proposed method was implemented on top of ECM-11.0. The performance was evaluated following the common test condition defined in [2]. The test results are reported in the following tables using ECM-11.0 as the anchor.

Test4.1a (min/max values are derived from the MCTF-prefiltered picture)

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.07% | -0.01% | -0.08% | 99.3% | 99.7% | 100.0% | 99.9% |
| Class A2 | -0.01% | 0.02% | 0.09% | 99.9% | 100.0% | 100.0% | 100.0% |
| Class B | -0.08% | 0.06% | -0.03% | 98.9% | 99.4% | 100.0% | 100.0% |
| Class C | -0.08% | -0.07% | 0.00% | 99.8% | 99.4% | 100.0% | 100.0% |
| Class E | -0.02% | 0.14% | 0.00% | 99.3% | 99.4% | 100.0% | 100.0% |
| Overall | -0.06% | 0.03% | -0.01% | 99.4% | 99.6% | 100.0% | 100.0% |
| Class D | -0.23% | -0.09% | 0.01% | 99.6% | 99.0% | 100.0% | 100.0% |
| Class F | -0.23% | -0.34% | 0.07% | 99.0% | 99.2% | 100.0% | 100.0% |
| Class TGM | -0.38% | -0.18% | -0.12% | 98.7% | 98.2% | 100.0% | 99.9% |

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.17% | 0.04% | -0.19% | 99.8% | 100.5% | 99.9% | 100.0% |
| Class A2 | 0.02% | 0.04% | -0.04% | 99.0% | 99.9% | 100.0% | 99.9% |
| Class B | -0.05% | -0.06% | -0.09% | 99.8% | 100.2% | 99.5% | 100.0% |
| Class C | -0.11% | 0.03% | -0.04% | 100.0% | 99.8% | 99.4% | 100.0% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.08% | 0.00% | -0.09% | 99.7% | 100.1% | 99.7% | 100.0% |
| Class D | -0.38% | -0.15% | -0.05% | 99.7% | 99.2% | 100.1% | 99.9% |
| Class F | -0.28% | -0.36% | 0.04% | 100.0% | 100.7% | 99.6% | 99.9% |
| Class TGM | -0.43% | -0.38% | -0.31% | 99.3% | 99.5% | 99.9% | 99.7% |

|  | Low delay B Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.04% | 0.48% | 0.24% | 99.2% | 99.2% | 99.8% | 99.5% |
| Class C | 0.02% | -0.14% | -0.27% | 99.8% | 101.4% | 100.6% | 100.2% |
| Class E | -0.08% | 0.04% | 0.72% | 99.2% | 99.0% | 101.2% | 99.8% |
| Overall | -0.03% | 0.17% | 0.19% | 99.4% | 99.9% | 100.4% | 99.8% |
| Class D | -0.05% | 0.52% | -0.86% | 100.7% | 98.5% | 100.2% | 100.0% |
| Class F | 0.04% | 0.32% | 0.12% | 100.9% | 102.1% | 100.5% | 100.2% |
| Class TGM | -0.21% | -0.15% | -0.25% | 98.8% | 96.4% | 99.8% | 100.0% |

Test4.1b (min/max values are derived from the original picture)

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.15% | 0.04% | -0.21% | 99.9% | 102.2% | 99.9% | 99.9% |
| Class A2 | 0.04% | 0.12% | 0.01% | 99.3% | 99.6% | 99.7% | 99.8% |
| Class B | -0.08% | -0.10% | -0.11% | 100.2% | 100.0% | 99.5% | 99.8% |
| Class C | -0.06% | -0.03% | 0.05% | 100.2% | 100.3% | 99.6% | 99.9% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.06% | -0.01% | -0.06% | 100.0% | 100.4% | 99.6% | 99.8% |
| Class D | -0.22% | 0.00% | -0.21% | 100.1% | 99.5% | 99.9% | 99.9% |
| Class F | -0.28% | -0.36% | 0.04% | 96.9% | 89.9% | 99.6% | 99.8% |
| Class TGM | -0.43% | -0.38% | -0.31% | 98.5% | 91.3% | 99.9% | 99.7% |

|  | Low delay B Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.03% | 0.75% | 0.55% | 99.1% | 98.5% | 99.8% | 99.3% |
| Class C | 0.06% | -0.02% | -0.33% | 100.2% | 100.2% | 100.0% | 100.0% |
| Class E | -0.08% | 0.02% | 0.77% | 100.4% | 100.0% | 100.0% | 99.6% |
| Overall | -0.01% | 0.31% | 0.31% | 99.8% | 99.4% | 99.9% | 99.6% |
| Class D | -0.01% | 0.12% | -1.45% | 100.3% | 97.3% | 100.2% | 99.9% |
| Class F | 0.04% | 0.32% | 0.12% | 100.4% | 101.5% | 100.5% | 100.2% |
| Class TGM | -0.21% | -0.15% | -0.25% | 99.3% | 97.4% | 99.8% | 100.0% |

# Reference

[1] ECM-11.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM.

[2] M. Karczewicz and Y. Ye, “Common test conditions and evaluation procedures for enhanced compression tool testing”, JVET-AF2017, Oct. 2023.

[3] K. Cui, Z. Zhang, H. Huang, V. Seregin and M. Karczewicz, “Non-EE2: Adaptive clipping with signalled lower and upper bounds”, JVET-AF0169, Oct. 2023.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).