| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0150-v1 |
| --- | --- |

| Title: | EE2-related: Adaptive GPM blending |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Lei Zhao1, Kai Zhang2, Li Zhang2 1Zijinshumayuan 4th building F9 18 Zhongguancun Nansijie Haidian District, 100190, Beijing, China 28910 University Center Lane San Diego, CA 92122, USA | Tel: Email: | zhaolei.0324@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

Abstract

This proposal presents the results of an enhanced GPM blending method on top of Test 2.11f [1], which combines Test 2.9, Test 2.10 and Test 2.11a. In Test 2.11f, a regression-based GPM blending method is proposed, where the splitting shape and the blending function are derived from the template. This proposal presents an adaptive blending width selection method based on the block size. It is reported that on top of ECM-11.0 [2], simulation results of the proposed method are as below:

RA: -0.31 % (Y), -0.31 % (U), -0.42% (V), 102.3% (EncT), 99.8% (DecT)

LB: -xxx % (Y), -xxx % (U), -xxx % (V), xxx% (EncT), xxx% (DecT).

On top of Test 2.11f, the results are reported as:

RA: -0.03 % (Y), -0.02 % (U), -0.14% (V), 99.9%% (EncT), 99.6% (DecT)

LB: -xxx % (Y), -xxx % (U), -xxx % (V), xxx% (EncT), xxx% (DecT).

- Introduction

In Test 2.11f, if gpm_implicit_flag is false, the prediction samples within a certain region along the GPM split boundary are generated by blending the two prediction signals. The blending width is determined from a predefined set with five fixed candidates, i.e., {τ/4, τ/2, τ, 2τ, 4τ}, and up to 8 samples on each side of the boundary are blended by a weighted sum. A CU level index is signalled to specify the blending width index.

- Proposed method

This proposal presents an adaptive GPM blending method with two alternative blending width candidate lists. In particular, list #0 ({τ/4, τ/2, τ, 2τ, 4τ}) comprises the same blending candidates as in ECM-11.0, and list #1 ({τ/2, τ, 2τ, 4τ, 8τ}) is designed for larger blocks, where at most 16 samples are blended on each side of the split boundary. The selection of the candidate list is adaptively based on the block size. If min(width, height) < 32, list #0 is used. Otherwise list #1 is used.

- Experimental results

The proposed method has been implemented on top of EE2 Test 2.11f. Results under RA/LDB configurations are presented in Table 1 and Table 2. The simulations were performed in accordance with the common test conditions [3]. More detailed results can be found in the attached spreadsheets.

Table 1: Coding performance of the proposed method on ECM-11.0

|  | RA |  |  |  |  |  | LB |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Vm Peak | Y | U | V | EncT | DecT | Vm Peak |
| Class A1 | -0.26% | -0.41% | -0.36% | 102.1% | 100.8% | 101.4% |  |  |  |  |  |  |
| Class A2 | -0.32% | -0.22% | -0.35% | 102.0% | 97.8% | 102.4% |  |  |  |  |  |  |
| Class B | -0.33% | -0.37% | -0.48% | 102.4% | 99.7% | 103.9% |  |  |  |  |  |  |
| Class C | -0.32% | -0.24% | -0.43% | 102.4% | 100.8% | 107.0% |  |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.31% | -0.31% | -0.42% | 102.3% | 99.8% | 103.9% |  |  |  |  |  |  |
| Class D | -0.20% | -0.17% | -0.48% |  |  | 102.6% |  |  |  |  |  |  |
| Class F | -0.18% | -0.07% | -0.20% |  |  | 104.1% |  |  |  |  |  |  |

Table 2: Coding performance of the proposed method on Test 2.11f

|  | RA |  |  |  |  |  | LB |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Vm Peak | Y | U | V | EncT | DecT | Vm Peak |
| Class A1 | -0.05% | -0.15% | -0.06% | 100.5% | 101.1% | 101.1% |  |  |  |  |  |  |
| Class A2 | -0.03% | 0.05% | -0.06% | 100.0% | 99.1% | 102.1% |  |  |  |  |  |  |
| Class B | -0.04% | -0.01% | -0.20% | 99.9% | 99.2% | 100.0% |  |  |  |  |  |  |
| Class C | 0.00% | 0.03% | -0.21% | 99.4% | 99.3% | 100.0% |  |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.03% | -0.02% | -0.14% | 99.9% | 99.6% | 100.6% |  |  |  |  |  |  |
| Class D | 0.03% | -0.04% | -0.35% |  |  | 100.0% |  |  |  |  |  |  |
| Class F | 0.00% | 0.05% | -0.01% |  |  | 100.1% |  |  |  |  |  |  |

- Conclusions

An adpative GPM blending method is presented in this contribution. Simulation results on top of EE2 Test 2.11f indicate xxx%/xxx% coding gain in RA/LDB configurations with negligible complexity increase. It is recommended to include the proposed methods in the next round of EE.

- References

[1] V. Seregin, J. Chen, R. Chernyak, et al.,“Exploration Experiment on Enhanced Compression beyond VVC capability (EE2)”, JVET-AF2024, Oct. 2023.

[2] ECM-11.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-11.0.

[3] M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-Y2017, Jan. 2022.

- Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).