| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0155-v2 |
| --- | --- |

| Title: | EE1-Related: On Low Complexity Operational Point for In-Loop Filter. |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Dmytro Rusanovskyy, Yun Li, Marta Karczewicz (Qualcomm) | Email: | dmytror@qti.qualcomm.com yli30@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

The LOP2 filter was adopted to NNVC in result of joint training, report provided in JVET-AF0043, with several additional contributions indicating that further complexity reduction is feasible. In this contribution, LOP2 filter design is revised toward further complexity reduction. Separable convolution was extended toward use in fusion/transition block and LOP architecture is adjusted to accommodate the released complexity reduction budget. Stage 3 training and inference results indicate that proposed design provide acceptable complexity-performance trade-off and is suitable for a very low complexity ILF configuration.

In the LOP2 complexity configuration (targeting 17kMAC/pixel), the proposed method with 9% lower complexity (15.6kMAC/pixel) indicatively provides lower validation cost, comparing to LOP2 anchor. The inference test reportedly provides {0.00%, 1.44%, 0.89%} and {0.08%, 0.98%, 0.75%} BD-rate change in RA and AI configurations, respectively.

In the very low complexity configuration (5kMAC/pixel), the BD-rate changes vs. VTM anchor are reportedly summarized as follows: RA: {-2.3%, -5.3%, -4.6%}; AI: {-2.9%, -5.1%, -5.6%}

It is proposed to investigate the method in the next round of EE1.

# Introduction

In JVET-AF meeting, LOP2 ILF filter, block diagram is shown in Figure 1, was adopted into NNVC, following joint training in JVET-AF0043[2]. In addition to this, several contributions, such as JVET-AF0071[3] and JVET-AF0206[4] indicated that additional complexity reduction to the ILF architecture are feasible targeting LOP or Very Low complexity operational point.

Analysis of the LOP2 architecture suggested that complexity (in terms of kMAC/pixel) of the Fusion-Transition module of LOP is close to 20% and may be reduced by employing separable convolutions which are already in used backbone blocks (BB).

Fig. 1. Network architecture of LOP2.

# Proposed ILF design

In this contribution, LOP2 filter design is revised toward further complexity reduction. Separable convolution with stride S=2 is employed in fusion/transition block and LOP architecture is adjusted to accommodate the released complexity reduction budget. Block diagram of resulting architecture is shown in Figure 2.

Fig. 2. Proposed network architecture.

Proposed ILF architecture was further configured to meet two operational points: Test 1 targeted LOP2 (~17kMAC/pixel) and Test 2 achieves Very Low Operational Point (~5kMAC/pixel), previously reported in JVET-AF0206, and trained over the Stage3 LOP2 dataset.

# Training and inference process

Training and inference testing for proposed method were conducted following NNVC7.1 LOP2 guideline for LOP2. Stage 3 training parameters for Test 1 and Test 2 are summarized in Table 1.

Table 1 Training information of proposed architecture.

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | Nvidia A100 |
|  | Framework: | Pytorch v3.9 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | Stage3: 60 epochs |
|  | Batch size: | Stage3: 128 |
|  | Loss function: | L1, L2 |
|  | Training time (for 1 model): | 10 days |
|  | Training data information: | DIV2K, BVI-DVC, TVD |
|  | Training configurations for generating compressed training data (if different to VTM CTC): |  |
| Optional |  |  |
|  | Number of iterations |  |
|  | Patch size | 144x144 |
|  | Learning rate: | Stage3: 4e-4 |
|  | Learning rate update strategy | Stage3: 55, 57, 58. mse-epoch:58 |
|  | Optimizer: | ADAM |
|  | Preprocessing: | flipped, rotated |
|  | Mini-batch selection process: | random cropped |
|  | Training data update strategy: |  |
|  | Other information: |  |

Table 2 Inference information of proposed architecture.

| Network Information in Inference Stage |  |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | Test 1 | Test2 |  |
| Mandatory | HW environment: |  |  |  |
|  | GPU Type | CPU only |  |  |
|  | Framework: | SADL float/ Pytorch |  |  |
|  | Number of GPUs per Task | 0 |  |  |
|  | Number of Parameters | 12945 | 48633 |  |
|  | Total Parameter Number |  |  |  |
|  | Parameter Precision (Bits) | Floating (32 bits) |  |  |
|  | Memory Parameter (MB) | 0.02 | <0.05 |  |
|  | MAC/Pixel (Kilo) | 5.0 | 15.6 |  |
| Optional |  |  |  |  |
|  | Total Conv. Layers |  |  |  |
|  | Total FC Layers | 0 |  |  |
|  | Total Memory (MB) |  |  |  |
|  | Batch size: |  |  |  |
|  | Patch size | 128x128 / 256x256 |  |  |
|  | Changes to network configuration or weights required to generate rate points |  |  |  |
|  | Peak Memory Usage (Total) |  |  |  |
|  | Peak Memory Usage (per Model) |  |  |  |
|  | Other information: |  |  |  |

# Experimental results

The proposed model is integrated into NNVC-7.1 and tested according to the common test conditions defined in [5] in floating-point against the VTM. nnIntra tool is disabled for all tests to assess performance of the tested model. Summary of the simulation results for 2 complexity configurations with floating-point model are tested configurations are shown below.

## Test 1

In Test 1, separable convolutions in fusion/transition block were tested at LOP2 architecture. Total complexity of 15.6 kMAC/pixel was achieved. Training was conducted following LOP2 Stage 3 procedure. The L1 validation costs of the tested training method, comparing to anchor LOP2 training are shown in Figure 3.

Figure 3. Validation cost of the LOP2 anchor training (blue line) and proposed method (orange line).

Inference testing of the resulting model (floating point) was tested with NNVC7.1 software. Summary of the comparative results vs. reference LOP2 ILF filter are provided in Table 3.

Table 3. Inference testing results for proposed method (float mode), comparing to LOP2 anchor.

|  | AI |  |  | RA |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | 0.13% | 1.60% | 1.41% | 0.15% | 1.23% | 0.42% |
| Class A2 | 0.06% | 0.83% | 0.48% | -0.21% | 2.09% | 0.85% |
| Class B | 0.05% | 0.74% | 0.13% | 0.00% | 1.06% | 0.81% |
| Class C | 0.13% | 1.03% | 0.93% | 0.06% | 1.58% | 1.37% |
| Class E | 0.05% | 0.88% | 1.17% |  |  |  |
| Overall | 0.08% | 0.98% | 0.75% | 0.00% | 1.44% | 0.89% |
| Class D | 0.16% | 0.86% | 0.56% | 0.05% | 1.62% | 0.64% |

## Test 2 (float)

In Test 2, separable convolutions in fusion/transition block were tested at the Very Low Operational Point (~5kMAC/pixel), configuration like that in JVET-AF0206 and trained over the Stage3 LOP2 dataset.

Compared with VTM (all NN tools are disabled), the proposed method Test 1 provides on average {-2.3%, -5.3%, -4.6%}; AI: {-2.9%, -5.1%, -5.6%} BD-rate changes for {Y, Cb, Cr} in RA and AI configurations, respectively.

|  | BD-rate: Test1 vs vtm |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | All Intra |  |  |  |  | Random Access |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -2.48% | -3.98% | -4.25% | 114% | 5741% | -2.22% | -3.67% | -3.65% | 118% | 8677% |
| Class A2 | -2.87% | -5.20% | -4.42% | 105% | 4594% | -2.42% | -4.83% | -3.14% | 110% | 6872% |
| Class B | -2.75% | -5.45% | -6.14% | 104% | 3765% | -2.11% | -5.92% | -5.25% | 112% | 6564% |
| Class C | -2.78% | -5.79% | -6.53% | 104% | 2439% | -2.47% | -6.17% | -5.70% | 105% | 6451% |
| Class E | -3.89% | -4.31% | -5.83% | 105% | 3896% |  |  |  |  |  |
| Overall | -2.92% | -5.05% | -5.57% | 106% | 3797% | -2.29% | -5.32% | -4.63% | 111% | 6972% |
| Class D | -3.11% | -5.68% | -7.19% | 102% | 2345% | -2.79% | -6.46% | -6.68% | 106% | 5636% |

# Conclusion

In this contribution LOP2 filter design is revised toward further complexity reduction. Separable convolution was extended toward use in fusion/transition block and LOP architecture is adjusted to accommodate the released complexity reduction budget. Stage 3 training and inference results indicate that proposed design provide acceptable complexity-performance trade-off and is suitable for a very low complexity ILF configuration. It is proposed to investigate the method in the next round of EE1.

# References

- Common test conditions and evaluation procedures for neural network-based video coding technology, JVET-AF2016, E. Alshina, R. -L. Liao, S. Liu, A. Segall, Hannover, DE, October 2023.

- JVET-AF0043: AhG11/EE1: Status of the joint EE1-0 (LOP.2) training, D. Rusanovskyy, Y. Li (Qualcomm), T. Shao, P. Yin, S. McCarthy(Dolby), J. N. Shingala, A. Shyam, A. Suneja, S. P. Badya (Ittiam), J. Li, Y. Li, C. Lin, K. Zhang, L. Zhang (Bytedance), R. Chang, L. Wang, X. Xu, S. Liu (Tencent), Hannover, DE, October 2023.

- JVET-AF0071: EE1-related: Further complexity reduction on the joint EE1-0 (LOP.2) unified filte, T. Shao, P. Yin, S. McCarthy (Dolby), J. N. Shingala, A. Shyam, A. Suneja, S. P. Badya (Ittiam), Hannover, DE, October 2023.

- JVET-AF0206: AHG11: Complexity reduction of NN-based loop-filters, J. Li, Y. Li, C. Lin, K. Zhang, L. Zhang (Bytedance), Hannover, DE, October 2023.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).