| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0174-v1 |
| --- | --- |

| Title: | EE1-1.1: Report on training with HOP architecture change for EE1-0 (variant 1) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yue Li, Chaoyi Lin, Junru Li, Kai Zhang, Li Zhang (Bytedance) Dmytro Rusanovskyy, Yun Li, Marta Karczewicz (Qualcomm) Renjie Chang, Liqiang Wang, Xiaozhong Xu, Shan Liu (Tencent) | Email: | yue.li@bytedance.com dmytror@qti.qualcomm.com renjiechang@tencent.com |
| Source: | Bytedance, Qualcomm, Tencent |  |  |

_____________________________

- Abstract

The HOP filter adopted into NNVC-7.1 shares the same architecture as its previous version in NNVC-6.0 but is obtained with an improved training procedure, where the hyper-parameters such as learning rate, batch size, as well as luma/chroma loss weight, have been finetuned.

In JVET-AF meeting, several contributions submitted in HOP category, JVET-AF0102, JVET-AF0103, JVET-AF0182, JVET-AF0192, aiming at reducing the complexities of existing HOP filter were selected for joint training within EE1-1.1. The goal of the test is to combine selected methods and compensate the resulting complexity reduction with other parameters of the architecture. To align the complexity of combined design and the existing HOP filter (477kMAC/pixel), the number of the residual blocks was increased. The training procedure and inference process have been aligned with the existing HOP filter in NNVC-7.1.

Joint training and CTC inference testing were conducted by the Bytedance, Qualcomm and Tencent. The following inference testing results are reported.

BD-rate changes on top of NNNC-7.1 HOP anchor (NN intra + HOP.2) are reportedly summarized as below:

For integer model: RA: -0.2%, -1.7%, -0.3%; AI: -0.2%, -0.3%, 0.0%.

For floating point model: RA: -0.5%, -2.0%, -1.2%;  AI: -0.3%, 0.4%, 0.4%

Additionally, run-time reduction is reported by all testers. Tested design results in RA/AI the 67% / 66% and 77% / 67% of encoder/decoder run-times, comparing to the HOP anchor in the integer model configuration.

kMAC/pixel and number of parameters of the combined design are reportedly summarized as below:

kMAC/pixel: 466

#parameters: 1.42M

Given the reported coding gain and run-time reduction at the lower kMAC/pixel complexity, it is proposed to adopt the proposed architecture for HOP.

Introduction

In JVET-AF meeting, several contributions have been submitted in HOP category: JVET-AF0102 [1], JVET-AF0103 [2], JVET-AF0182 [3], JVET-AF0192 [4].

In JVET-AF0182, NNVC-6.0 HOP architecture was simplified by applying group convolution to the first layer after spatial decomposition in the residual block, as shown in Fig. 1.

Fig. 1. Network architecture in JVET-AF0182.

In JVET-AF0102, NNVC-6.0 HOP architecture was refined by reducing decomposition rank and alternating two types of residual blocks with different decomposition order, as shown in Fig. 2.

Fig. 2. Network architecture in JVET-AF0102.

JVET-AF0103 proposed to optimize network head in NNVC-6.0 HOP architecture by applying 1x1 convolution for IPB and BS inputs, as shown in Fig. 4.

Fig. 3. Network head in JVET-AF0103.

Proposed Refined HOP Filter Architecture

In this contribution a unified filter architecture for HOP is studied in EE. Proposed architecture combines key components from abovementioned contributions and is designed to be closely aligned to the existing HOP filter architecture and to employ the existing HOP training procedure.

In network head, 1×1 convolution is applied on sparse inputs such as IPB and BS, to reduce complexity. Network backbone comprises residual blocks, which have been optimized by introducing group convolution, alternated decomposition order and channel reduction (C31=48). To align complexity, number of residual blocks is increase to 31. The filter usage is similar to current HOP in terms of interface of input and output.

Fig. 4. Proposed refined architecture for HOP EE1-1.1 test.

Training process

Training is conducted independently in Bytedance, Qualcomm, and Tencent following the procedure resulting from EE1-0 test and specified for training of the HOP.2 since NNVC7.1. Compared with the training procedure of HOP.1 in NNVC-6.0, following modification have been introduced in EE1-0: learning rate increases to 0.0002 (0.0001 in HOP.1), batch size reduces to 32 (64 in HOP.1), luma/cb/cr loss weight changes to 12:1:1 (6:1:1 in HOP.1).

Experimental results

The proposed model is integrated into NNVC-7.1 and tested according to the common test conditions defined in [5] in floating-point and int16 configuration against the NNVC7.1 HOP anchor. nnIntra tool is enabled for both, the test and the anchor.

Summary of the simulation results in the floating-point configuration are reported below:

Bytedance trained models (float)

Compared with NNVC-7.1 HOP2 anchor (nn intra + hop.2), the proposed method brings on average {%, %, %} and {-0.23%, 0.77%, 1.24%} BD-rate changes for {Y, Cb, Cr} under RA and AI configurations.

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-7.1 HOP2 Anchor |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -0.49% | -2.16% | -1.67% | 91% | 91% |
| Class A2 | -0.37% | -2.32% | 0.42% | 93% | 92% |
| Class B | -0.23% | -1.47% | -1.41% | 91% | 88% |
| Class C | -0.20% | -1.08% | -0.73% | 90% | 83% |
| Class E |  |  |  |  |  |
| Overall | -0.30% | -1.68% | -0.92% | 91% | 88% |
| Class D | -0.13% | -1.67% | -0.41% | 89% | 82% |
| Class F | -0.33% | -0.56% | -0.15% | 89% | 82% |
|  | All Intra Main10 |  |  |  |  |
|  | BD-rate Over NNVC-7.1 HOP2 Anchor |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -0.22% | 0.41% | 2.23% | 96% | 87% |
| Class A2 | -0.23% | 0.87% | 2.52% | 92% | 87% |
| Class B | -0.23% | 2.49% | 1.53% | 92% | 84% |
| Class C | -0.20% | -0.33% | -0.05% | 94% | 85% |
| Class E | -0.29% | -0.38% | 0.21% | 92% | 84% |
| Overall | -0.23% | 0.77% | 1.24% | 93% | 85% |
| Class D | -0.16% | -0.02% | -0.29% | 104% | 86% |
| Class F | -0.33% | -0.65% | 0.95% | 96% | 90% |

Qualcomm trained models (float)

Compared with NNVC-7.1 HOP2 anchor (nn intra + hop.2), the proposed method brings on average:
{-0.5%, -2.0%,-1.2%} and {-0.3%, 0.4%, 0.4%} BD-rate changes for {Y, Cb, Cr} under RA and AI configurations.

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Test vs NNVC7.1 HOP Anchor |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -0.49% | -2.52% | -0.98% | 85% | 96% |
| Class A2 | -0.49% | -2.96% | -0.86% | 85% | 94% |
| Class B | -0.42% | -2.05% | -1.80% | 85% | 99% |
| Class C | -0.41% | -1.12% | -0.80% | 82% | 89% |
| Class E |  |  |  |  |  |
| Overall | -0.45% | -2.08% | -1.18% | 84% | 95% |
| Class D | -0.44% | -1.12% | -0.71% | 82% | 87% |
| Class F | -0.43% | -1.74% | -1.00% | 84% | 86% |

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Test vs NNVC7.1 HOP Anchor |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -0.14% | -0.19% | -0.92% | 86% | 97% |
| Class A2 | -0.32% | 0.65% | 3.24% | 90% | 97% |
| Class B | -0.27% | 2.38% | 0.91% | 93% | 101% |
| Class C | -0.29% | -0.88% | -0.49% | 90% | 91% |
| Class E | -0.39% | -1.17% | -0.62% | 89% | 97% |
| Overall | -0.28% | 0.35% | 0.43% | 90% | 97% |
| Class D | -0.23% | -0.83% | -0.75% | 94% | 92% |
| Class F | -0.18% | -1.12% | 0.31% | 87% | 92% |

Tencent trained models (float)

Compared with NNVC-7.1 HOP2 anchor (nn intra + hop.2), the proposed method brings on average {-0.35%, -2.16%, -0.99%} and {-0.23%, 0.65%, 1.51%} BD-rate changes for {Y, Cb, Cr} under RA and AI configurations.

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-7.1 HOP2 Anchor |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -0.18% | -3.05% | 0.22% | 90% | 95% |
| Class A2 | -0.49% | -2.82% | -0.31% | 89% | 94% |
| Class B | -0.42% | -1.82% | -1.74% | 90% | 94% |
| Class C | -0.27% | -1.43% | -1.45% | 89% | 90% |
| Class E |  |  |  |  |  |
| Overall | -0.35% | -2.16% | -0.99% | 89% | 93% |
| Class D | -0.12% | -1.22% | -1.78% | 89% | 91% |
| Class F | -0.41% | -1.64% | -0.12% | 88% | 81% |
|  | All Intra Main10 |  |  |  |  |
|  | BD-rate Over NNVC-7.1 HOP2 Anchor |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -0.07% | -0.05% | 2.50% | 91% | 95% |
| Class A2 | -0.30% | 0.54% | 2.92% | 93% | 94% |
| Class B | -0.25% | 3.03% | 2.36% | 94% | 94% |
| Class C | -0.26% | -0.62% | -0.27% | 96% | 93% |
| Class E | -0.27% | -0.83% | 0.07% | 93% | 95% |
| Overall | -0.23% | 0.65% | 1.51% | 94% | 94% |
| Class D | -0.19% | -0.11% | -0.56% | 96% | 93% |
| Class F | 0.00% | -0.67% | 1.20% | 95% | 95% |

Summary of the simulation results in the int16 configuration are reported below:

Qualcomm trained models (int16)

Compared with NNVC-7.1 HOP2 anchor (nn intra + hop.2), the proposed method brings on average:
{-0.2%, -1.7%,-0.3%} and {-0.2%, -0.3%, 0.0%} BD-rate changes for {Y, Cb, Cr} under RA and AI configurations. The inference of this integer model has been successfully crosschecked by Bytedance.

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Test vs NNVC7.1 HOP Anchor |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | 0.06% | -2.31% | -0.11% | 65% | 66% |
| Class A2 | -0.29% | -2.62% | -0.46% | 65% | 65% |
| Class B | -0.19% | -1.38% | -0.75% | 66% | 69% |
| Class C | -0.20% | -0.79% | 0.15% | 71% | 65% |
| Class E |  |  |  |  |  |
| Overall | -0.16% | -1.65% | -0.32% | 67% | 66% |
| Class D | -0.24% | -1.08% | 0.24% | 69% | 63% |
| Class F | -0.33% | -1.48% | -0.19% | 64% | 60% |

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Test vs NNVC7.1 HOP Anchor |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -0.05% | -0.75% | -0.86% | 70% | 70% |
| Class A2 | -0.20% | -0.45% | 0.84% | 77% | 70% |
| Class B | -0.17% | 0.89% | 0.60% | 78% | 69% |
| Class C | -0.18% | -0.67% | -0.29% | 84% | 64% |
| Class E | -0.23% | -0.95% | -0.46% | 73% | 64% |
| Overall | -0.17% | -0.26% | 0.02% | 77% | 68% |
| Class D | -0.13% | -0.65% | -0.78% | 88% | 66% |
| Class F | -0.02% | -0.80% | 0.65% | 72% | 60% |

Conclusion

This contribution proposes an improved filter architecture for HOP study in EE1. Proposed architecture combines key components of EE tests in JVET-AF meeting related to HOP and is designed to be closely aligned to the HOP filter architecture. It is proposed to adopt this architecture for HOP in next version of NNVC.

References

- EE1-1.1.2: Complexity-performance trade off of decompostion, JVET-AF0102, D. Rusanovskyy, Y. Li, M. Karczewicz (Qualcomm)

- EE1-1.1.3: Study on input feature set optimization, JVET-AF0103, D. Rusanovskyy, Y. Li, M. Karczewicz (Qualcomm)

- EE1-1.1.4.b: Group convolution for HOP, JVET-AF0182, Y. Li, K. Zhang, L. Zhang (ByteDance)

- EE1-1.1.5: Combination test of EE1-1.1.1 and EE1-1.1.2, JVET-AF0192, R. Chang, L. Wang, X. Xu, S. Liu (Tencent), D. Rusanovskyy, Y. Li, M. Karczewicz (Qualcomm)

- Common test conditions and evaluation procedures for neural network-based video coding technology, JVET-AF2016, E. Alshina, R. -L. Liao, S. Liu, A. Segall, Hannover, DE, October 2023.

Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).