| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0184-v1 |
| --- | --- |

| Title: | AHG9: Text comment SEI message |  |  |  |
| --- | --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |  |
| Purpose: | Proposal |  |  |  |
| Author(s) or Contact(s): | P. de Lagrange D. Doyen E. François F. Urban C. Salmon-Legagneur | Email: | philippe.delagrange@interdigital.com |
| Source: | InterDigital |  |  |  |

_____________________________

# Abstract

This contribution proposes a new SEI message allowing to embed text comments in a video bitstream. Examples of use cases are annotations for parts of a video sequence, descriptive annotations for indexing purpose, user generated comments. In the proposed "text comment SEI message”, text comments are associated with ids to allow overlapping persistence (and potentially defining different types of comments).

# Introduction

While working on JVET-AG0182, it was noted that JPEG files can embed text comments, but that functionally is not available in video bitstreams like AVC, HEVC and VVC.

It is then proposed here to create a text comment SEI message, where text comments are associated with ids to allow overlapping persistence (and potentially defining different types of comments), as illustrated in Figure 1.

Possible use cases are:

- Annotations for parts of a video sequence, for example in a production context (e.g. identifying takes, conditions, things to do)

- Descriptive annotations, or keywords, for indexing purpose in a large collection of video sequences, where different portions of the sequences may be of specific interest

- Carry technical data (e.g. command line and/or description for conformance bitstreams)

- User generated comments, or description, for example in user generated content.

- Comments and or links related to products showcased in the video sequence

- A player could list available comments, and offer to play the relevant portion of the video

Figure 1 – Example arrangement of text comments

# Proposed SEI message

Proposed syntax and semantics are the following:

Table 1 – Proposed syntax

| text_comment( payloadSize ) { | Descriptor |
| --- | --- |
| text_comment_cancel_flag | u(1) |
| if( !text_comment_cancel_flag ) { |  |
| text_comment_id | u(8) |
| text_comment_id_cancel_flag | u(1) |
| if ( !text_comment_id_cancel_flag ) { |  |
| text_comment_id_persistence_flag | u(1) |
| while( !byte_aligned( ) ) |  |
| text_comment_alignment_zero_bit | u(1) |
| for( i = 0; more_data_in_payload( ); i++ ) |  |
| text_comment[text_comment_idx][i] | b(8) |
| } |  |
| } |  |
| } |  |

text_comment_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous text comment SEI message that is associated with one or more layers to which the text comment SEI message applies. text_comment_cancel_flag equal to 0 indicates that text comment information follows.

text_comment_id indicates the identifier of the text comment. The value of text_comment_id shall be in the range of 0 to 255, inclusive [note: we may want to provide more space (e.g. 13-bit, removing alignment zero bits)].

text_comment_id_cancel_flag equal to 1 cancels the persistence scope of the text_comment_id-th text comment. text_comment_id_cancel_flag equal to 0 indicates that text comment follows.

text_comment_id_persistence_flag specifies the persistence of the text_comment_id-th text comment for the current layer.

text_comment_id_persistence_flag equal to 0 specifies that the text comment applies to the current decoded picture only.

text_comment_id_persistence_flag equal to 1 specifies that the text comment applies to the current decoded picture and persists for all subsequent pictures of the current layer in output order until one or more of the following conditions are true:

– A new CLVS of the current layer begins.

– The bitstream ends.

– A picture in the current layer, associated with a text comment SEI message with the same text_comment_id value, is output that follows the current picture in output order.

text_comment_alignment_zero_bit shall be equal to 0.

text_comment[text_comment_id][i] contains the i-th byte of the text comment.

Notes:

- for consistency with other SEI messages in the context of AVC, a persistence time can be specified instead of a flag. In that case, text_comment_id_persistence_flag would be replaced by text_comment_id_repetition_period: value 0 meaning no persistence, value 1 meaning persistence until cancel (as with the flag), and values greater than 1 would indicate the number of POCs during which the message is valid.

# Conclusion

It is proposed to add a Text comment SEI message based on this contribution in the next version of  the “Technologies under consideration for future extensions of VSEI” document.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).