| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0200-v2 |
| --- | --- |

| Title: | Non-EE2: Inter CCP merge mode with zero luma CBF |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhipin Deng1, Kai Zhang2, Li Zhang2 1Zijinshumayuan 4th building F9 18 Zhongguancun Nansijie Haidian District, 100190, Beijing, China 28910 University Center Lane San Diego, CA 92122, USA | Tel: Email: | {zhipin.deng, zhangkai.video, lizhang.idm}@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

In ECM-11.0, inter CCP merge mode is allowed only if luma CBF is non-zero. In this contribution, it is proposed to allow inter CCP merge mode when the luma CBF is equal to 0. The proposed method is applied on SCC sequences. The impact on coding efficiency and runtimes of the proposed method over ECM-11.0 are reportedly for {Y, U, V, EncT, DecT}:

RA :

class F { 0.02%, -0.28%, -0.49%, 100.9%, 101.1% }

class TGM { -1.26%, -3.61%, -3.61%, 101.1%, 100.7% }

LDB :

class F { -0.17%, -1.43%, -1.69%, 101.5%, 100.8% }

class TGM { -0.90%, -5.74%, -5.84%, 101.2%, 100.9% }

# Introduction

At the JVET-AF meeting, an inter CCP merge mode was adopted to blend the inter motion-compensated chroma prediction with a CCP model estimated chroma prediction, and the CCP model can be either calculated from adjacent neighboring samples or inherited from a previous CCP coded block. The inter CCP merge mode follows the same restriction of inter CCCM mode that requires non-zero luma CBF. However, it may be inefficient to limit the luma CBF to non-zero for the inter CCP merge mode, as the CCP model is not calculated from inter reference blocks.

# Proposed method

In this contribution, it is proposed to allow the inter CCP merge mode to a chroma block even if the luma CBF is equal to 0. With the proposed method, the inter CCP merge mode flag can be signalled for the merge skip mode, and when the luma CBF or the rootCBF is equal to 0.

# Experimental results

The proposed method is implemented on top of ECM-11.0 [1] and tested following the common test conditions [2]. The proposed method is applied for SCC sequences. The simulation results of the proposed method as compared to ECM-11.0 are summarized in Table 1.

Table 1: Coding performance of the proposed method

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class F | 0.02% | -0.28% | -0.49% | 100.9% | 101.1% | 99.5% | 99.8% |
| Class TGM | -1.26% | -3.61% | -3.61% | 101.1% | 100.7% | 100.1% | 100.3% |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class F | -0.17% | -1.43% | -1.69% | 101.5% | 100.8% | 99.7% | 99.2% |
| Class TGM | -0.90% | -5.74% | -5.84% | 101.2% | 100.9% | 100.3% | 99.8% |

# Conclusions

The proposed method can achieve noticeable BD-rate gains with decent runtime change. It is recommended to include the proposed methods to the next EE2 for a further investigation.

# References

- ECM-11.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-11.0.

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-AF2017, Oct. 2023.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).