| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET- AG0212-v1 |
| --- | --- |

| Title: | AHG8: A post-processing algorithm for machine consumption |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Binzhe Li, Shurun Wang, Jie Chen, Yan Ye (Alibaba) Shiqi Wang (City University of Hong Kong) | Tel: Email: | {libinzhe.lbz, shurun.wsr, jiechen.cj, yan.ye}@alibaba-inc.com shiqwang@cityu.edu.hk |
| Source: | Alibaba, City University of Hong Kong |  |  |

_____________________________

- Abstract

A learning-based video post-processing algorithm in which an enhancement neural network is utilized to process the reconstructed pictures to improve machine task performance is proposed in this contribution. According to the experimental results, it is reported that the proposed algorithm achieves 18.46% (RA), 26.35% (LD), 33.70% (AI) BD-rate saving for object detection task on SFU dataset, and 27.86% (RA), 42.96% (LD), 33.70% (AI) BD-rate savings for object tracking task on TVD dataset. Additional simulation results show that the proposed algorithm can also achieve obvious performance improvement when evaluated with a different machine task model rather than the one specified in VCM CTC.

Description of post-processing algorithms

Framework

Figure 1 shows a typical post-processing pipeline. The original video is fed into the video encoder and compressed into bitstream. After the bitstream is decoded, the reconstructed video sequence is further processed for machine consumption. The post-processing is conducted with a neural network model.

Figure 1: Typical post-processing only compression framework.

Post-processing network

Denote the reconstructed pictures as Ir (input to the post-processing network) and post-processed pictures as Ip (output of the post-processing network), the architecture of the post-processing network is shown in Figure 2.  In order to achieve a multi-granularity post-processing, we use the promising U-Net architecture with parallel mirrored skip connections and multi-resolution modules. In particular, the reconstructed picture Ir is fed to feature down sampling branch. After that, the intermediate feature Fd1 of 1st resolution is extracted by the convolution layer. Then, the intermediate features Fdi are processed with the base block to F'di. After that, the F'di is downsampled to Fdi+1 by the max pooling layer with kernel size 2x2. The F'di is also copied to the up branch by the skip connection. In the feature up sampling branch, the intermediate feature F'ui+1 or F'd4 is upsampled by the deconvolution layer and pixel shuffle function with the upscaling factor of 2. Then, the unsampled feature is combined with skip connected feature F'di. The convolution layer with kernel size 1x1 reduces the channel size of the combined feature and obtains the feature Fui. The base block is also facilitated to transform the Fui to F'ui. Finally, the output visual signal Ip is inferred based on the F'u1. Specifically, the base block of the visual signal restoration network [1] is introduced to improve the visual signal enhancement performance, which could enhance the features representation with the channel attention mechanism with the adaption layer.

Figure 2: Architecture of the post-processing network.

Experimental results

Experimental results under CTC

The proposed algorithm is tested under the VCM common test conditions (CTC) specified in JVET-AF2031 [2], where the VTM version is 22.2. The overall performance for detection and tracking is shown in Table 1.

Table 1. Overview of results under VCM CTC

| Task | Dataset | RA (Pareto mAP) | LD (Pareto mAP) | AI (Pareto mAP) |
| --- | --- | --- | --- | --- |
| Object Detection | SFU-HW (Average) | -18.46% | -26.35% | -33.70% |
| Object Tracking | TVD (Average) | -27.86% | -42.96% | -38.88% |

The per-class performance on SFU-HW dataset for object detection is shown in Table 2 and the per-sequence performance on TVD dataset for object tracking is given in Table 3. (Please note “/” in the tables indicates the performance cannot be calculated due to the nonmonotonic machine task performance). The results show that the proposed algorithm achieves significant performance improvement under CTC.

Table 2 Performance of post-processing algorithm on SFU-HW dataset under RA, LD and AI configurations.

| RA | Object Detection | End-to-End BD-Rate [%] |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | mAP | mAP (low4) | Pareto mAP | End-to-End BD-mAP |
| SFU-HW | Class A | / | / | -17.39% | 0.82 |
|  | Class B | / | / | -14.80% | 1.58 |
|  | Class C | -18.17% | -35.87% | -18.17% | 2.32 |
|  | Class D | -22.66% | -24.09% | -22.66% | 2.16 |
|  | Average | / | / | -18.46% | 1.93 |
| LD | Object Detection |  |  |  |  |
|  |  | mAP | mAP (low4) | Pareto mAP | End-to-End BD-mAP |
| SFU-HW | Class A | / | -43.97% | -33.94% | 4.03 |
|  | Class B | -11.34% | -16.74% | -11.34% | 2.71 |
|  | Class C | / | / | -35.40% | 4.13 |
|  | Class D | / | / | -30.41% | 3.34 |
|  | Average | / | / | -26.35% | 3.44 |
| AI | Object Detection |  |  |  |  |
|  |  | mAP | mAP (low4) | Pareto mAP | End-to-End BD-mAP |
| SFU-HW | Class A | -39.43% | -42.51% | -39.43% | 6.10 |
|  | Class B | -30.22% | -36.72% | -30.22% | 6.15 |
|  | Class C | -38.21% | -48.39% | -38.21% | 5.63 |
|  | Class D | -31.24% | -35.16% | -31.24% | 4.28 |
|  | Average | -33.70% | -40.28% | -33.70% | 5.41 |

Table 3 Performance of post-processing algorithm on TVD dataset under RA, LD and AI configurations.

| RA | Object Tracking |  | End-to-End BD-Rate [%] |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | MOTA | MOTA (low4) | Pareto MOTA | End-to-End BD-MOTA |
| TVD | TVD-01-1 | / | / | -12.81% | 1.92 |
|  | TVD-01-2 | / | / | -24.08% | 1.64 |
|  | TVD-01-3 | / | 4.73% | 1.39% | -3.93 |
|  | TVD-02-1 | -22.38% | -17.05% | -22.38% | 1.41 |
|  | TVD-03-1 | -33.68% | -33.94% | -33.68% | 4.08 |
|  | TVD-03-2 | / | / | -50.63% | 4.02 |
|  | TVD-03-3 | -52.84% | -57.04% | -52.84% | 5.41 |
|  | Average | / | / | -27.86% | 2.08 |
| LD | Object Tracking |  |  |  |  |
|  |  | MOTA | MOTA (low4) | Pareto MOTA | End-to-End BD-MOTA |
| TVD | TVD-01-1 | -34.46% | -34.71% | -34.46% | 3.25 |
|  | TVD-01-2 | / | / | -56.88% | 2.15 |
|  | TVD-01-3 | / | -28.73% | -41.64% | 4.38 |
|  | TVD-02-1 | 4.16% | 12.93% | 4.16% | 0.09 |
|  | TVD-03-1 | / | -42.00% | -48.83% | 7.28 |
|  | TVD-03-2 | / | / | -55.34% | 4.30 |
|  | TVD-03-3 | -67.70% | -75.50% | -67.70% | 6.20 |
|  | Average | / | / | -42.96% | 3.95 |
| AI | Object Tracking |  |  |  |  |
|  |  | MOTA | MOTA (low4) | Pareto MOTA | End-to-End BD-MOTA |
| TVD | TVD-01-1 | -27.71% | -28.99% | -27.71% | 3.58 |
|  | TVD-01-2 | -34.53% | -36.05% | -34.53% | 3.90 |
|  | TVD-01-3 | -17.85% | -15.90% | -17.85% | 2.12 |
|  | TVD-02-1 | -23.08% | 9.22% | -23.08% | 1.24 |
|  | TVD-03-1 | -51.60% | -54.24% | -51.60% | 8.73 |
|  | TVD-03-2 | -57.06% | -62.18% | -57.06% | 7.34 |
|  | TVD-03-3 | -60.29% | -67.37% | -60.29% | 8.75 |
|  | Average | -38.88% | -36.50% | -38.88% | 5.09 |

Additional experimental results beyond CTC

To show the performance stability of the proposed algorithm, experiments are conducted with Faster-RCNN-R101-FPN on SFU-HW dataset for object detection (Please note Faster-RCNN-X101-FPN is used in CTC for object detection) [3]. The experiments are also conducted with JDE.864x480 on TVD dataset for object tracking (JDE.1088x608 is used in CTC for object tracking.). The overall performance for detection and tracking is shown in Table 4. The results are shown in Table 5 and Table 6. Similar with results under CTC, the proposed algorithm also achieves significant performance improvement.

Table 4. Overview of results beyond CTC

| Task | Dataset | Test Network | RA (Pareto mAP) | LD (Pareto mAP) | AI (Pareto mAP) |
| --- | --- | --- | --- | --- | --- |
| Object Detection | SFU-HW (Average) | Faster-RCNN-R101-FPN | -26.37% | -24.42% | -31.88% |
| Object Tracking | TVD (Average) | JDE.864x480 | -12.75% | -12.54% | -14.04% |

Table 5 Performance of post-processing algorithm on SFU-HW test dataset under RA, LD and AI configurations, with Faster-RCNN-R101-FPN as the task network.

| RA | Object Detection | End-to-End BD-Rate [%] |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | mAP | mAP (low4) | Pareto mAP | End-to-End BD-mAP |
| SFU-HW | Class A | / | / | -14.16% | -0.33 |
|  | Class B | / | / | -24.98% | 2.73 |
|  | Class C | / | -26.45% | -28.01% | 3.28 |
|  | Class D | / | / | -29.16% | 2.49 |
|  | Average | / | / | -26.37% | 2.59 |
| LD | Object Detection |  |  |  |  |
|  |  | mAP | mAP (low4) | Pareto mAP | End-to-End BD-mAP |
| SFU-HW | Class A | / | / | -22.76% | 1.00 |
|  | Class B | / | / | -18.06% | 3.17 |
|  | Class C | -24.51% | -28.23% | -24.51% | 3.67 |
|  | Class D | -31.12% | -37.69% | -31.12% | 3.35 |
|  | Average | / | / | -24.42% | 3.21 |
| AI | Object Detection |  |  |  |  |
|  |  | mAP | mAP (low4) | Pareto mAP | End-to-End BD-mAP |
| SFU-HW | Class A | / | -29.00% | -26.81% | 2.31 |
|  | Class B | / | / | -30.65% | 6.80 |
|  | Class C | -38.34% | -41.10% | -38.34% | 6.13 |
|  | Class D | -27.91% | -34.97% | -27.91% | 4.17 |
|  | Average | / | / | -31.88% | 5.44 |

Table 6 Performance of post-processing algorithm on TVD dataset under RA, LD and AI configurations, with JDE.864x480 as the task network.

| RA | Object Tracking | End-to-End BD-Rate [%] |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | MOTA | MOTA (low4) | Pareto MOTA | End-to-End BD-MOTA |
| TVD | TVD-01-1 | / | -10.36% | -11.28% | 0.30 |
|  | TVD-01-2 | -20.01% | -6.38% | -20.01% | 1.18 |
|  | TVD-01-3 | -20.41% | -20.71% | -20.41% | 0.80 |
|  | TVD-02-1 | -0.27% | -3.67% | -0.27% | 0.04 |
|  | TVD-03-1 | / | 0.06% | 3.60% | -0.04 |
|  | TVD-03-2 | -26.49% | -22.27% | -26.49% | 1.43 |
|  | TVD-03-3 | -14.41% | -16.45% | -14.41% | 0.64 |
|  | Average | / | -11.40% | -12.75% | 0.62 |
| LD | Object Tracking |  |  |  |  |
|  |  | MOTA | MOTA (low4) | Pareto MOTA | End-to-End BD-MOTA |
| TVD | TVD-01-1 | -17.00% | -20.65% | -17.00% | 0.08 |
|  | TVD-01-2 | / | 14.03% | 16.74% | -0.36 |
|  | TVD-01-3 | / | / | -16.62% | 1.69 |
|  | TVD-02-1 | 2.68% | 12.04% | 2.68% | -0.12 |
|  | TVD-03-1 | -13.85% | -16.00% | -13.85% | 1.75 |
|  | TVD-03-2 | -29.69% | -28.63% | -29.69% | 1.49 |
|  | TVD-03-3 | -30.05% | -32.63% | -30.05% | 1.19 |
|  | Average | / | / | -12.54% | 0.82 |
| AI | Object Tracking |  |  |  |  |
|  |  | MOTA | MOTA (low4) | Pareto MOTA | End-to-End BD-MOTA |
| TVD | TVD-01-1 | 1.35% | 4.52% | 1.35% | -0.15 |
|  | TVD-01-2 | -3.22% | 15.86% | -3.22% | 0.06 |
|  | TVD-01-3 | -16.91% | -3.04% | -16.91% | 0.76 |
|  | TVD-02-1 | 9.48% | 19.54% | 9.48% | -0.50 |
|  | TVD-03-1 | -20.62% | -24.22% | -20.62% | 2.61 |
|  | TVD-03-2 | -33.71% | -33.42% | -33.71% | 2.86 |
|  | TVD-03-3 | -34.66% | -37.04% | -34.66% | 2.30 |
|  | Average | -14.04% | -8.26% | -14.04% | 1.13 |

Figure 3 shows the rate-mAP curves for some sequences of SFU dataset and Figure 4 shows rate-MOTA curves for some sequences of TVD dataset. From these curves, it is observed that the performance improvement introduced by the proposed algorithm can be achieved both in monotonic and non-monotonic cases for object detection and tracking.

(a) RA configuration.

(b) LD configuration.

(c) AI configuration.

Figure 3: The rate-mAP curves for object detection on SFU-HW dataset sequences.

(a) RA configuration.

(b) LD configuration.

(c) AI configuration.

Figure 4: The rate-MOTA curves for object tracking on TVD dataset sequences. (a) RA configuration. (b) LD configuration. (c) AI configuration.

Conclusion

In this contribution, a post-processing algorithm is proposed for the purpose of machine consumption. Simulation results show that the proposed algorithm does not only achieve significant performance improvement under CTC but also works for other machine task model beyond CTC. Thus, it is suggested to include the proposed algorithm into the next draft of technical report (TR) on optimization of encoders and receiving systems for machine analysis of coded video content.

References

- X Chu, L Chen, W Yu. "NAFSSR: Stereo image super-resolution using NAFNet." Proceedings of the IEEE/CVF Conference on Computer Vision and Pattern Recognition. 2022.

- S. Liu, C. Hollmann, “Common test conditions for optimization of encoders and receiving systems for machine analysis of coded video content,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 output document JVET-AF2031, 32nd Meeting, Hannover, DE, 13–20 October 2023 (available online at https://jvet-experts.org/).

- Y, Wu, A. Kirillov, F. Massa, W. Lo, R. Girshick, “Detectron2” https://github.com/facebookresearch/detectron2 2019.

Patent rights declaration(s)

Alibaba group and City University of Hong Kong may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).