| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0216-v1 |
| --- | --- |

| Title: | AHG8: Multi-layer VVC for hybrid machine-human consumption |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jaakko Laitinen, Tero Partanen, Alexandre Mercat, Jarno Vanne Tampere University, Finland Alireza Aminlou, Miska M. Hannuksela, Francesco Cricri, Honglei Zhang Nokia Technologies, Tampere, Finland | Email: Email: | firstname.lastname@tuni.fi firstname.lastname@nokia.com |
| Source: | Tampere University, Nokia |  |  |

_____________________________

# Abstract

This contribution investigates VVC multi-layer coding to facilitate hybrid machine-human consumption, where machine-optimized input is coded as the base-layer and the original content is encoded as an enhancement-layer for human consumption. Machine-optimized input is generated using pre-processing by extracting Region-of-Interest (ROI) areas from the background. Two multi-layered schemes are tested by blurring or greying-out the background. For each multi-layered schemes four different spatial scaling ratios of 1x, 0.75x, 0.5x, and 0.25x are tested and compared to coding of original content using VVC (VVC only), and simulcasting of machine-optimized and original contents using VVC (simulcasting). The comparison for machine and human consumption are performed using mAP and PSNR, respectively.

In addition to BD-rate comparisons between the tested schemes, the contribution presents break-even points, defined as the fraction of time that the full multi-layer bitstream is required for human consumption, where the multi-layered schemes are beneficial over the VVC-only case. Based on the presented results, it is concluded that both the VVC multi-layer coding and simulcast are valid options for hybrid machine-human consumption. The contribution proposes to add a section on hybrid machine-human consumption in the technical report on optimization of encoders and receiving systems for machine analysis of coded video content.

# Introduction

Figure 1 illustrates the general use case for hybrid human-machine video consumption considered in this study. A set of camera sensors capture video streams that are either processed and stored at the edge layer or transmitted to the cloud layer for further processing and storage. Typically, these streams serve two purposes: 1) continuous monitoring and analysis automated by various machine vision algorithms; and 2) on-demand monitoring by human operators in some specific situations or adverse operating conditions either in real time or offline afterward. Example use cases include intelligent traffic, assembly line, and security monitoring systems.

Figure 1. General use case for human-machine video consumption.

This input document investigates VVC multi-layer coding to facilitate hybrid machine-human consumption. In this context, human compatible bitstream equals standard VVC video. Machine-optimized bitstream is also in VVC format, but it is pre-processed for machines, e.g., by extracting Region-of-Interest (ROI) areas from the background.

# Investigated coding schemes

In this contribution, we evaluate three schemes, namely the VVC only, VVC and M-VVC, and multi-layer VVC coding schemes, as illustrated in Figure 2. These schemes fulfill all the needed requirements of the addressed use cases:

- VVC only: A standard VVC bitstream is continuously sent for machine and human consumption.

- VVC and M-VVC: A machine-optimized VVC bitstream is continuously sent for machine consumption, and a standard VVC bitstream is sent if it is requested for human consumption.

- Proposed multi-layer VVC: A machine-optimized VVC bitstream is continuously sent for machine consumption, and an enhancement bitstream is sent if it is requested for human consumption.

(a) VVC only coding scheme (anchor)

(b) Simulcast (VVC and M-VVC) coding scheme

(c) Proposed multi-layer VVC coding scheme

Figure 2. Studied coding schemes

## VVC only coding scheme

Figure 2-(a) depicts the VVC only coding scheme, where the input video, referred to as YUV, is encoded using a standard VVC encoder, resulting in the bVVC bitstream. Decoding the bitstream yields the distorted output video YUV’ with no additional processing applied to it.

This VVC only approach is compatible with most machine tasks, which are often trained on regular, unprocessed images. However, it is important to note that VVC only might not always represent the most efficient solution in terms of rate for most machine tasks. Depending on the nature of these tasks, the standard VVC encoding could include information that, while necessary for human consumption, may be redundant or unnecessary for machine processing.

## Simulcast (VVC and M-VVC) coding scheme

Figure 2-(b) illustrates the dual-path (simulcast) approach of the VVC and M-VVC coding scheme. where the input video YUV undergoes two independent encoding processes: one standard and the machine optimized encodings.

- The standard encoding path mirrors the VVC only coding scheme. The input video YUV is encoded using a standard VVC encoder, resulting in the base VVC bitstream bVVC. Decoding bVVC produces the distorted output video YUV’, identical to the outcome of the VVC only scheme.

- The machine-optimized encoding path introduces a pre-processing (ROI-based) step to create YUVM, a machine-optimized version of the input video. This pre-processed video is then encoded into a machine-optimized bitstream bVVCM, which, upon decoding, yields a reconstructed machine-optimized video YUV’M.

## Proposed multi-layer VVC coding scheme

Figure 2-(c) shows the proposed multi-layer VVC coding scheme. In this scheme, the input video YUV is pre-process (ROI-based) into the machine-optimized version of the video YUVM and both YUV and YUVM are encoded utilizing multi-layer encoding. The scheme generates two distinct bitstreams: the BL bitstream bVVCM and the EL bitstream bVVCEL. These are then combined into a single multi-layer bitstream, denoted as bVVCM+EL. The decoding of this bitstream yields two reconstructed outputs: YUV’M for the BL, tailored for machine tasks, and YUV’EL for the EL, suitable for human viewing. In other words, when only the machine-optimized video is required, only the BL bitstream bVVCM is transmitted; conversely, if the full video is needed for human consumption, the complete bitstream bVVCM+EL is sent. For consistency and fair comparison across the simulcast and multi-layer VVC coding schemes, the same pre-processing method is employed to generate machine-optimized YUVM.

# Experimental setup

## Test sequences and parameters

Experiments largely follow the common test conditions (CTC) defined in document JVET-AF2031 [1]. SFU-HW dataset is used for the experiments, but only classes A, B, and C are included because ROI based processing loses efficacy on smaller resolutions and spatial scalability cannot effectively be used either. Encoding is done using the VVC reference software VTM (ver. 20.1) [2] and the random-access configuration is used for all tests. When multi-layer coding is used, the default multi-layer configuration as well as the same quantization parameters (QPs) are used for each layer. Additional encoding parameters, such as QP, follow those of the anchor defined in the CTC (see Table 1).

Table 1. Test sequences and relevant coding parameters used in the experiments

| Class | Sequence | Seq. ID | Resolution | Fps | # skipped frames | # encoded frames | Intra period | QPs |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A | Traffic | A1 | 2560x1600 | 30 | 117 | 33 | 32 | 39, 45, 48, 51, 54, 58 |
| B | ParkScene | B1 | 1920x1080 | 24 | 207 | 33 | 32 | 32, 36, 40, 44, 48, 52 |
|  | Cactus | B2 | 1920x1080 | 50 | 403 | 97 | 64 | 43, 46, 49, 52, 55, 58 |
|  | BasketballDrive | B3 | 1920x1080 | 50 | 403 | 97 | 64 | 40, 43, 46, 49, 52, 55 |
|  | BQTerrace | B4 | 1920x1080 | 60 | 471 | 129 | 64 | 40, 43, 46, 49, 52, 55 |
| C | BasketballDrill | C1 | 832x480 | 50 | 403 | 97 | 64 | 27, 31, 35, 39, 43, 47 |
|  | BQMall | C2 | 832x480 | 60 | 471 | 129 | 64 | 27, 32, 37, 42, 47, 52 |
|  | PartyScene | C3 | 832x480 | 50 | 403 | 97 | 64 | 31, 35, 39, 43, 47, 51 |
|  | RaceHorses | C4 | 832x480 | 30 | 235 | 65 | 64 | 27, 31, 35, 39, 43, 47 |

This proposal examines two distinct methods for processing the background areas as proposed in JVET-AE2030 [3]. The first one involves heavily blurring the background areas (Figure 3-(b)), which makes them more easily compressible while maintaining some general detail. A Gaussian filter is used for blurring with a kernel size of 31 and a sigma of 10. The second one is to completely remove the background areas (Figure 3-(c)), replacing them with a solid color; here grey. This greying method is more efficient in terms of encoding but results in the total loss of background details. While greying out the background is likely to provide better compressibility, employing a blur might integrate better with the multi-layer coding tools, since some background detail is still maintained for use by inter-layer referencing, resulting in better rate-distortion performance.

In the preprocessing step for the bVVCM bitstream, YOLOv8x [4] is used for object detection to generate the ROI map. Additionally, ROI maps are aggregated based on GOP length to get a more stable saliency map that is shared by the pictures in the given GOP. The boxes are used to select what areas are not modified and the resulting video is downscaled using ffmpeg [5] if scale is not 1×. Results are given for spatial scaling ratios of 1×, 0.75×, 0.5×, and 0.25×, where each spatial dimension is reduced by the given ratio.

| (a) Original content YUV | (b) Blurred background YUVM | (c) Greyed-out background YUVM |
| --- | --- | --- |

Figure 3. Input contents for different schemes

## Evaluation metrics

To evaluate the distortion caused by the encoding process for human consumption, the peak signal-to-noise ratio (PSNR) is used. On the other hand, for evaluating the machine performance, the mean average precision (mAP) is used. To calculate mAP the decoded output (i.e., YUV’ and YUV’M,) is passed to the evaluation machine task detectron2 [6] as defined in the CTC.

To quantify the difference between two solutions Bjontegaard Delta Bitrate (BD-rate) [7] is employed, giving the relative bitrate difference between them at equal mAP or PSNR. Due to mAP being prone to producing non-monotonic rate-distortion curves, cubic curve fitting is applied to the mAP values so that the BD-rate can be calculated in all tests.

A break-even point is also calculated, comparing the total bitrate of the proposed multi-layer VVC scheme to that of the VVC only scheme (R) to calculate when the multi-layer coding is more efficient. Define th as the fraction of time that the full multi-layer bitstream is required (i.e., bVVCM+EL) resulting in a bitrate of RM+EL. The rest of the time a rate of RM is required for the BL (i.e., bVVCM):

| th∙RM+EL+1-thRM≤R. | (1) |
| --- | --- |

Define RM+EL and RM in terms of the VVC only bitrate R using BD-rate (BDRM+EL and BDRM) and substituting into (1):

| th⋅BDRM+EL⋅R+1-thBDRM⋅R≤R. | (2) |
| --- | --- |

Solve for th in (2):

| th≤1-BDRMBDRM+EL-BDRM. | (3) |
| --- | --- |

# Experimental results

Table 3. Coding gain in BD-rate(PSNR) of the proposed multi-layer VVC over simulcast coding scheme for human consumption   .

| Seq. | Blurred Background |  |  |  | Greyed Background |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID | BL Spatial Scaling Ratio |  |  |  | BL Spatial Scaling Ratio |  |  |  |
|  | 1× | 0.75× | 0.5× | 0.25× | 1× | 0.75× | 0.5× | 0.25× |
| A1 | -21.8% | -11.9% | -6.1% | -1.3% | -20.1% | -11.7% | -5.7% | -1.5% |
| B1 | -14.5% | -8.5% | -4.0% | -0.7% | -14.7% | -9.3% | -3.9% | -0.5% |
| B2 | -33.0% | -19.4% | -7.9% | -1.7% | -32.7% | -19.3% | -7.6% | -1.4% |
| B3 | -39.9% | -21.4% | -8.0% | -1.1% | -39.7% | -21.5% | -8.0% | -1.0% |
| B4 | -23.6% | -13.1% | -4.1% | -0.6% | -23.6% | -13.2% | -3.7% | -1.1% |
| Avg. | -26.5% | -14.9% | -6.0% | -1.1% | -26.2% | -15.0% | -5.8% | -1.1% |
| C1 | -46.1% | -18.0% | -5.3% | 0.4% | -45.2% | -18.2% | -5.1% | 0.3% |
| C2 | -43.2% | -15.9% | -4.4% | 0.6% | -44.4% | -15.9% | -4.3% | 0.7% |
| C3 | -37.1% | -15.4% | -4.0% | 0.5% | -36.0% | -15.4% | -3.8% | 0.6% |
| C4 | -47.7% | -17.9% | -4.8% | 0.0% | -47.6% | -17.8% | -4.9% | 0.2% |
| Avg. | -43.5% | -16.8% | -4.6% | 0.4% | -43.3% | -16.8% | -4.5% | 0.5% |

Table 2.  Coding gain in BD-rate(mAP) of the proposed multi-layer VVC over VVC only coding scheme for machine only consumption

| Seq. | Blurred Background |  |  |  | Greyed Background |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID | BL Spatial Scaling Ratio |  |  |  | BL Spatial Scaling Ratio |  |  |  |
|  | 1× | 0.75× | 0.5× | 0.25× | 1× | 0.75× | 0.5× | 0.25× |
| A1 | -18.3% | -25.9% | -16.4% | -43.0% | -12.2% | -24.8% | -32.3% | -40.0% |
| B1 | -22.9% | -30.8% | -40.8% | -44.1% | -64.0% | -68.2% | -68.4% | -59.0% |
| B2 | -13.1% | -33.1% | -51.4% | -69.1% | -20.5% | -37.3% | -59.0% | -75.3% |
| B3 | -3.3% | -17.6% | -21.6% | -29.7% | -7.9% | -19.3% | -16.3% | -35.3% |
| B4 | -13.9% | -24.8% | -33.6% | -56.6% | -18.8% | -28.1% | -39.3% | -49.1% |
| Avg. | -14.3% | -26.4% | -32.8% | -48.5% | -24.7% | -35.5% | -43.1% | -51.7% |
| C1 | 4.8% | -13.6% | -12.9% | -9.6% | 1.9% | -13.2% | -14.4% | -20.2% |
| C2 | 5.3% | -3.9% | -7.6% | -12.8% | 3.7% | -9.1% | -1.9% | -8.6% |
| C3 | -16.5% | 3.3% | -7.9% | -26.3% | -10.8% | -26.9% | -18.0% | -30.9% |
| C4 | -0.8% | -19.9% | -27.3% | -9.1% | 4.9% | -21.1% | -25.3% | -9.9% |
| Avg. | -1.8% | -8.5% | -13.9% | -14.5% | -0.1% | -17.6% | -14.9% | -17.4% |

Table 2 shows the BD-rate performance of the machine optimized bVVCM bitstream when compared to the unmodified bVVC (i.e., machine only consumption of multi-layer VVC and simulcast over VVC only). For machine performance, mAP is used as the quality metric. Table 3 shows the human BD-rate performance of the multi-layer VVC scheme (i.e., bVVCM+EL) compared to the simulcast scheme (i.e., bVVCM + bVVC), using PSNR as the quality metric, where YUV’EL and YUV’ are used for the PSNR calculation, respectively.

Overall, the greyed-out background pre-processing method is up to 10% better than blurring in terms of machine performance, but the pre-processing method does not seem to have a significant effect on the EL performance. This suggests that both pre-processing methods are viable options for multi-layer coding despite them making it harder to use inter-layer references, due to background areas being blurred or greyed out. With both classes and pre-processing methods, the best machine rate-distortion performance results are achieved with a 0.25× BL scale. However, scale has the inverse effect on the EL efficiency, resulting in nearly no improvement over simulcast in the 0.25× case, likely due to the EL not able to effectively take advantage of inter-layer references because of to the small base layer resolution. In cases where the machine optimized BL cannot be effectively used with multi-layer coding, simulcast coding can be used instead. Additionally, while the higher resolution classes A and B perform well, the machine performance of class C is diminished especially with scale 1×.

Table 4 lists the overhead of the multi-layer VVC coding scheme compared to VVC only for human consumption. It can be observed that all cases result in varying degrees of overhead. For class AB, the aggressive downscaling at 0.25× scale overall causes the least amount of overhead with a ~11%–13% increase in bitrate over a standard VVC bitstream. However, the 1× scale case has the second lowest overhead with Class AB and is, on average, the most efficient for class C at around 10% BD-rate increase.

The overhead from multi-layer coding can be mitigated if the enhancement layer is only needed occasionally. Using (3), we calculate the break-even point values, shown in Table 5. When the share of time for human consumption is less that the break-even point presented in Table 5, the multi-layer VVC scheme outperforms the VVC only scheme. Table 5 shows that overall, using a smaller scale allows the EL to be used more frequently while keeping the total bandwidth lower than only using VVC. With scale 0.25× and class AB, EL could be used roughly 80% of the time and still be equal to VVC only. With no BL downscaling, this value drops to around 40%–50%. For some class C sequences however, multi-layer coding is never better than VVC only, because of the positive BD-rate of the machine optimized bitstream in those cases. Regardless, with smaller scales, class C provides break-even points ranging from 27% to 46%.

Table 4. Overhead in BD-rate(PSNR) of the proposed multi-layer VVC over VVC only coding scheme for human consumption

| Seq. | Blurred Background |  |  |  | Greyed Background |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID | BL Spatial Scaling Ratio |  |  |  | BL Spatial Scaling Ratio |  |  |  |
|  | 1× | 0.75× | 0.5× | 0.25× | 1× | 0.75× | 0.5× | 0.25× |
| A1 | 20.5% | 24.4% | 18.4% | 10.8% | 12.6% | 17.7% | 14.1% | 08.0% |
| B1 | 15.4% | 18.7% | 14.3% | 08.7% | 08.4% | 12.4% | 10.4% | 06.8% |
| B2 | 24.4% | 37.7% | 30.9% | 17.5% | 19.3% | 32.0% | 27.3% | 16.0% |
| B3 | 15.1% | 33.6% | 28.5% | 14.5% | 15.0% | 33.2% | 28.4% | 14.5% |
| B4 | 25.4% | 31.8% | 25.1% | 13.2% | 23.2% | 28.4% | 23.4% | 11.8% |
| Avg. | 20.1% | 29.2% | 23.5% | 12.9% | 15.7% | 24.7% | 20.7% | 11.4% |
| C1 | 07.4% | 42.2% | 35.7% | 18.5% | 9.5% | 41.9% | 35.9% | 18.3% |
| C2 | 11.5% | 45.0% | 37.0% | 19.7% | 10.3% | 44.8% | 36.9% | 19.6% |
| C3 | 13.3% | 31.8% | 26.1% | 13.4% | 15.0% | 31.6% | 26.2% | 13.7% |
| C4 | 04.7% | 44.6% | 38.0% | 18.7% | 05.0% | 44.9% | 38.0% | 18.9% |
| Avg. | 09.2% | 40.9% | 34.2% | 17.6% | 09.9% | 40.8% | 34.2% | 17.6% |

Table 5. Break-even point of the proposed multi-layer VVC over VVC only coding scheme

| Seq. | Blurred Background |  |  |  | Greyed Background |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID | BL Spatial Scaling Ratio |  |  |  | BL Spatial Scaling Ratio |  |  |  |
|  | 1× | 0.75× | 0.5× | 0.25× | 1× | 0.75× | 0.5× | 0.25× |
| A1 | 47.2% | 51.5% | 47.0% | 80.0% | 49.1% | 58.4% | 69.6% | 83.3% |
| B1 | 59.8% | 62.2% | 74.1% | 83.6% | 88.4% | 84.6% | 86.8% | 89.7% |
| B2 | 35.0% | 46.8% | 62.5% | 79.8% | 51.4% | 53.9% | 68.4% | 82.5% |
| B3 | 18.1% | 34.4% | 43.1% | 67.2% | 34.5% | 36.8% | 36.5% | 71.0% |
| B4 | 35.3% | 43.8% | 57.2% | 81.1% | 44.7% | 49.8% | 62.7% | 80.7% |
| Avg. | 39.1% | 47.7% | 56.8% | 78.3% | 53.6% | 56.7% | 64.8% | 81.4% |
| C1 | 00.0% | 24.4% | 26.5% | 34.3% | 00.0% | 23.9% | 28.6% | 52.5% |
| C2 | 00.0% | 07.9% | 17.1% | 39.5% | 00.0% | 16.8% | 04.8% | 30.5% |
| C3 | 55.3% | 00.0% | 23.2% | 66.2% | 41.7% | 46.0% | 40.7% | 69.3% |
| C4 | 13.8% | 30.9% | 41.8% | 32.7% | 00.0% | 32.0% | 39.9% | 34.4% |
| Avg. | 17.3% | 15.8% | 27.2% | 43.2% | 10.4% | 29.7% | 28.5% | 46.6% |

# Proposal

[The proposed text to be added to the TR as a new encoding sub-section 7.3 in the next version]

# Conclusion

It is recommended to include simulcast and multi-layer VVC encoding methods and their software implementation in the next version of the technical report on a description of optimization of encoders and receiving systems for machine analysis of coded video content (JVET-AG2030).

# Patent rights declaration(s)

Nokia and Tampere University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- S. Liu and C. Hollman, “Common test conditions for optimization of encoders and receiving systems for machine analysis of coded video content,” document JVET-AF2031-v1, Hannover, Germany, Oct. 2023

- “VVC Reference Software Version 20.1,” Accessed: Nov. 2023. [Online]. Available: https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/-/tree/VTM-20.1

- J. Chen, C. Hollmann, and S. Liu, “Optimization of encoders and receiving systems for machine analysis of coded video content (draft 3),” document JVET-AE2030-v1, Geneva, Switzerland, Jul. 2023.

- G. Y. Jocher, A. Chaurasia, and J. Qiu, “YOLOv8,” Ultralytics, 2023. [Online]. https://github.com/ultralytics/ultralytics

- “FFmpeg,” Accessed: Nov. 2023. [Online]. Available: https://ffmpeg.org

- Y. Wu, A. Kirillov, F. Masa, W.-Y. Lo, and R. Girschick, “Detectron2,” 2019. Accessed: Dec. 2023. [Online]. Available: https://github.com/facebookresearch/detectron2

- G. Bjøntegaard, “Improvements of the BD-PSNR model,” document VCEG-AI11, Berlin, Germany, Jul. 2008.