| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0222 |
| --- | --- |

| Title: | Crosscheck of JVET-AG0104 (EE2-1.17: IBC GPM with block vector difference and split mode reordering) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Crosscheck Report |  |  |
| Author(s) or Contact(s): | Yang Wang Fangheng Building, No.27, 3rd Ring West Rd, Haidian District, 100098, Beijing, China | Tel: Email: | wangyang.cs@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

- Abstract

This contribution reports the crosscheck results of JVET-AG0104 [1], which proposes IBC-GPM with block vector difference, and IBC-GPM with split mode reordering. The simulation results match with those provided in JVET-AG0104.

Experimental results

JVET-AG0104 was implemented on top of ECM-11.0. The simulations were conducted under the common test conditions of EE2 [2]. The crosscheck results are shown in Table 1, Table 2, and Table 3.

Table 1. Results of Test #1

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class F | -0.10% | -0.23% | 0.14% | 102.3% | 102.9% | 100.1% | 100.0% |
| Class TGM | -0.29% | -0.23% | -0.28% | 102.2% | 99.7% | 100.0% | 99.9% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class F | -0.06% | -0.09% | 0.01% | 102.8% | 101.4% | 100.1% | 99.8% |
| Class TGM | -0.15% | -0.11% | -0.17% | 106.2% | 105.5% | 101.2% | 100.1% |

Table 2. Results of Test #2

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class F | -0.03% | -0.12% | -0.01% | 99.5% | 103.8% | 100.0% | 100.0% |
| Class TGM | -0.06% | -0.09% | -0.07% | 100.4% | 102.8% | 100.0% | 99.9% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class F | -0.09% | -0.18% | 0.09% | 101.4% | 102.0% | 100.0% | 100.0% |
| Class TGM | -0.03% | -0.06% | -0.02% | 104.6% | 106.3% | 101.0% | 100.3% |

Table 3. Results of Test #3

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class F | -0.12% | -0.26% | 0.15% | 102.1% | 102.8% | 100.1% | 100.0% |
| Class TGM | -0.36% | -0.33% | -0.32% | 103.5% | 100.2% | 100.1% | 99.9% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class F | -0.11% | -0.16% | -0.04% | 103.2% | 103.2% | 100.0% | 100.1% |
| Class TGM | -0.23% | -0.23% | -0.20% | 106.1% | 105.6% | 101.4% | 99.7% |

References

C. Ma, X. Xiu, W. Chen, H.-J. Jhu, C.-W. Kuo, N. Yan, and X. Wang, “EE2-1.16: IBC LIC with slope adjustment,” Document of Joint Video Experts Team, JVET-AG0104, by teleconference, January 2024.

M. Karczewicz, and Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, Document of Joint Video Experts Team, JVET-AF2017, Hannover, DE, 13–20 October 2023.