| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0303-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-AG0186 (Non-EE2: FIBC Extension) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Woong Lim Sung-Chang Lim 218 Gajeong-ro, Yuseong-gu, Daejeon, 34129, KOREA Name(s) Jiyoung Kim Haechul Choi (HNU) 125 Dongseodae-ro, Yuseong-gu, Daejeon, 34158, KOREA | Tel: Email: | +82-42-860-1836 woong.lim@etri.re.kr sclim@etri.re.kr jiyeongk34@gmail.com choihc@hanbat.ac.kr |
| Source: | Electronics and Telecommunications Research Institute (ETRI) and Hanbat National University (HNU) |  |  |

_____________________________

- Abstract

This contribution reports a crosscheck result for JVET-AG0186 contribution which proposes support multiple template shapes and multiple models for the FIBC. The crosschecking is conducted on implementation on top of ECM-11.0. It is confirmed that the partial crosscheck results are matched with those provided by the proponent.

Introduction

In JVET-AG0186 [1], supporting multiple template shapes and multiple models for FIBC was proposed. FIBC method in the current ECM is limited to the use of L-shaped template and single model for deriving linear model parameters. In this contribution, two modifications are proposed to the FIBC to improve its coding efficiency on screen content sequences (SCC): multiple template shapes and multiple models.

Firstly, besides the existing L-shaped template, two new FIBC modes are introduced to allow using top-only or left-only neighboring reconstructed samples, respectively, for the filter derivation.

Secondly, similar to MMLM, two linear models are proposed for the FIBC which are derived based on the classification of the template samples of the current CU according to their corresponding DC value.

Experimental results

The experiments were performed based on ECM-11.0 software [2] under JVET ECM common test conditions [3]. And, the experiments were conducted on the platform of Intel Xeon Gold 6256 (3.6GHz) and 256GB RAM with GCC 9.4.0 compiler under Ubuntu 20.04.4 LTS. The partial experimental results of JVET-AG0186 is summarised in Table 1. And, the detailed results can be found in accompanied Excel file.

Table 1. Simulation result of JVET-AG0186

|  | All Intra |  |  |  |  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-11.0 CTC |  |  |  |  | Over ECM-11.0 CTC |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class D |  |  |  |  |  | -0.13% | -0.16% | -0.31% | 100.7% | 103.7% |
| Class F | -0.06% | -0.02% | -0.09% | 101.4% | 101.2% | -0.08% | -0.13% | -0.05% | 99.7% | 104.6% |

References

- Hong-Jheng Jhu, Xiaoyu Xiu, Wei Chen, Che-Wei Kuo, Ning Yan, Changyue Ma, Xianglin Wang, "Non-EE2: FIBC extension," JVET-AG0186, Jan. 2024

- ECM-11.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-11.0

- M. Karczewicz and Y. Ye, "Common test conditions and evaluation procedures for enhanced compression tool testing," Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, JVET-AF2017, 32nd Meeting, Hannover, DE, Oct. 2023.