| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0323-v1 |
| --- | --- |

| Title: | Cross-check of JVET-AG0042 (AHG 16: Proposed Common Software Tools and Testing Conditions for Generative Face Video Compression) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Jing Yuan Thong Han Boon Teo | Tel: Email: | jingyuan.thong@sg.panasonic.com hanboon.teo@sg.panasonic.com |
| Source: | Panasonic |  |  |

_____________________________

# Abstract

This contribution seeks to report the cross-check results of experiments performed in JVET-AG0042 on proposed common software tools and training conditions for Generative Face Video Compression (GFVC).

# Introduction

Cross-check was performed on the source code and simulation tests of the VVC anchor (VTM22.2) and the 3 proposed GFVC methods.

As the 3 extra sequences with complex head and camera motion (in JVET-AG0042-v3) was not provided in the link to the shared folder of currently proposed test sequences, this cross-check was performed on JVET-AG0042-v2 instead.

In particular, the following workflow was adopted during the cross-check phase:

- Download the provided software, checkpoints to the related models, and testing data of 15 VOXCELEB sequences (denoted as Class A) and 15 CFVQA sequences (denoted as Class B).

- Installation of relevant packages and libraries

- Run on VVC anchor to simulate pipeline of using VVC as baseline medium for GFVC

- Run on each of the 3 proposed GFVC methods (FOMM, FV2V, CFTE) to simulate pipeline of using each method for GFVC

- Evaluation on the following metrics:

- Bitrate required during transmission

- DISTS (based on VGG16), LPIPS (based on VGG16), PSNR, SSIM in the RGB444 domain

- PSNR, SSIM in the YUV420 domain

- Calculations for rate-distortion

It was observed that the software provided by the proponents of JVET-AG0042 match the description provided in the corresponding contribution. Some excel sheet formulae were fixed during this crosscheck.

The cross-check was first performed on the following hardware configuration (with software environment listed as Config 1 below).

- OS: Ubuntu 20.04.6 LTS

- CPU: Intel Xeon W-2265

- GPU: NVIDIA RTX-3090

- RAM: 128 GB

Owing to the presence of slight deviations from the reported results in JVET-AG0042, the software configuration was requested from the proponents for exact replication of results. Yet, as CUDA version 10.2 was incompatible with GPU NVIDIA RTX-3090 during installation of CUDA libraries, a second environment was created (listed as Config 2 below) to try to match the library packages as much as possible. Config 2 uses the same hardware configuration as Config 1, except for the OS being Ubuntu 18.04.6 LTS.

In order to match the exact CUDA library used in JVET-AG0042, a 3rd configuration was setup which uses GPU NVIDIA RTX-2070 that is able to support CUDA 10.2. The software libraries are kept exactly the same as Config 2, except for PyTorch to account for the CUDA version. This is reflected as Config 3.

The overall software environment for cross-check tabulated as follows. The software environment provided by proponents of JVET-AG0042 during email exchanges are listed for comparison.

Table 1: Comparison of Hardware-Software Configurations for Cross-check against JVET-AG0042

| Hardware/Python Library | JVET-AG0042 | Config 1 | Config 2 | Config 3 |
| --- | --- | --- | --- | --- |
| OS | - | Ubuntu 20.04.6 | Ubuntu 18.04.6 | Ubuntu 20.04.5 |
| CPU | - | Intel Xeon W-2265 | Intel Xeon W-2265 | Intel i7-10870H |
| GPU | - | RTX-3090 | RTX-3090 | RTX-2070 |
| RAM | - | 128 GB | 128 GB | 32 GB |
| Python 3 | 3.6.12 | 3.10.13 | 3.6.12 | 3.6.12 |
| compressai | 1.2.4 | 1.2.4 | 1.2.4 | 1.2.4 |
| dists-pytorch | 0.1 | 0.2 | 0.1 | 0.1 |
| imageio | 1.19.5 | 2.31.3 | 2.3.0 | 2.3.0 |
| lpips | 0.1.4 | 0.1.4 | 0.1.4 | 0.1.4 |
| matplotlib | 3.3.4 | 3.7.2 | 3.3.4 | 3.3.4 |
| opencv-python | 4.6.0.66 | 4.8.0.76 | 4.6.0.66 | 4.6.0.66 |
| opencv-python-headless | 4.6.0.66 | 4.8.0.76 | 4.6.0.66 | 4.6.0.66 |
| pandas | 1.1.5 | 2.0.3 | 1.1.5 | 1.1.5 |
| Pillow | 8.4.0 | 10.0.0 | 8.4.0 | 8.4.0 |
| plotly | 5.14.1 | 5.9.0 | 5.14.1 | 5.14.1 |
| pytorch-msssim | 0.2.1 | 1.0.0 | 0.2.1 | 0.2.1 |
| PyYAML | 5.4.1 | 6.0 | 5.4.1 | 5.4.1 |
| scikit-image | 0.17.2 | 0.17.2 | 0.17.2 | 0.17.2 |
| scikit-learn | 0.24.2 | 1.3.0 | 0.24.2 | 0.24.2 |
| scipy | 1.5.3 | 1.12.0 | 1.5.3 | 1.5.3 |
| torch | 1.10.1+cu102 | 2.0.1+cu117 | 1.10.1+cu111 | 1.10.1+cu102 |
| torchvision | 0.11.2+cu102 | 0.15.2+cu117 | 0.11.2+cu111 | 0.11.2+cu102 |
| tqdm | 4.64.1 | 4.65.0 | 4.64.1 | 4.64.1 |

# Results on VVC Anchor

This section reports the results obtained from Step 3 of the workflow pipeline introduced in Section 1, where VVC is used as the transmission medium for GFVC. In particular, experiments on the VVC anchor were performed and then compared against the corresponding reported VVC results.

Table 2: Comparison of VVC Results Obtained against VVC Results in JVET-AG0042

|  | Over AG0042 VVC (VTM 22.2) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | DISTS | LPIPS | RGB444_PSNR | RGB444_SSIM | YUV420_PSNR | YUV420_SSIM |
| Class A | 0.00% | 0.00% | 0.00% | 0.0% | 0.0% | 0.0% |
| Class B | 0.00% | 0.00% | 0.00% | 0.0% | 0.0% | 0.0% |
| Overall | 0.00% | 0.00% | 0.00% | 0.0% | 0.0% | 0.0% |

Table 2 illustrates the comparison of results obtained for VVC anchor, by testing the obtained results against the results reported in JVET-AG0042 as reference. As the bit-rate savings have values of 0.00% across all metrics, this illustrates an exact match in results obtained compared to JVET-AG0042.

# Results on GFVC methods in JVET-AG0042

## Results on GFVC methods for Config 1

This section reports the results obtained from Step 4 of the workflow pipeline introduced in Section 1 for Config 1, where each proposed GFVC method is used as transmission medium. In particular, experiments on each method were performed and then compared against the corresponding reported results. Note that the result “#VALUE!” means the curve overleap for some sequences so that BD rate cannot be calculated.

Table 3: Comparison of CFTE Results Obtained against CFTE Results in JVET-AG0042 for Config 1

|  | CFTE |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over AG0042 GFVC |  |  |  |  |  |
|  | DISTS | LPIPS | RGB444_PSNR | RGB444_SSIM | YUV420_PSNR | YUV420_SSIM |
| Class A | 0.03% | 0.03% | 0.03% | 1.9% | 0.0% | #VALUE! |
| Class B | -0.01% | -0.01% | -0.04% | 0.2% | -4.2% | 0.0% |
| Overall | 0.01% | 0.01% | -0.01% | 1.1% | -2.1% | #VALUE! |

Table 4: Comparison of FOMM Results Obtained against FOMM Results in JVET-AG0042 for Config 1

|  | FOMM |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over AG0042 GFVC |  |  |  |  |  |
|  | DISTS | LPIPS | RGB444_PSNR | RGB444_SSIM | YUV420_PSNR | YUV420_SSIM |
| Class A | -0.01% | -0.01% | -0.03% | -0.1% | 0.0% | 0.0% |
| Class B | -0.02% | -0.03% | 0.14% | 0.0% | -0.1% | #VALUE! |
| Overall | -0.01% | -0.02% | 0.06% | 0.6% | 0.0% | #VALUE! |

Table 5: Comparison of FV2V Results Obtained against FV2V Results in JVET-AG0042 for Config 1

|  | FV2V |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over AG0042 GFVC |  |  |  |  |  |
|  | DISTS | LPIPS | RGB444_PSNR | RGB444_SSIM | YUV420_PSNR | YUV420_SSIM |
| Class A | -0.01% | -0.01% | -0.02% | 0.0% | 0.5% | 0.0% |
| Class B | 0.00% | 0.00% | -0.06% | 0.0% | 0.0% | 0.0% |
| Overall | -0.01% | -0.01% | -0.04% | 0.0% | 0.2% | 0.0% |

Tables 3, 4 and 5 illustrate the comparison of results obtained for CFTE, FOMM and FV2V respectively, by testing the obtained results against the results reported in JVET-AG0042 as reference. Some deviations across the different metrics between the obtained results and the results in JVET-AG0042 were obtained.

## Results on GFVC methods for Config 2

This section reports the results obtained from Step 4 of the workflow pipeline introduced in Section 1 for Config 2, where each proposed GFVC method is used as transmission medium. In particular, experiments on each method were performed and then compared against the corresponding reported results.

It should be mentioned that the result “#VALUE!” means the curve overleap for some sequences so that the BD rate cannot be calculated.

Table 6: Comparison of CFTE Results Obtained against CFTE Results in JVET-AG0042 for Config 2

|  | CFTE |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over AG0042 GFVC |  |  |  |  |  |
|  | DISTS | LPIPS | RGB444_PSNR | RGB444_SSIM | YUV420_PSNR | YUV420_SSIM |
| Class A | 0.02% | 0.02% | 0.02% | 0.2% | 0.0% | #VALUE! |
| Class B | -0.01% | -0.01% | -0.04% | -1.6% | 1621.2% | -0.4% |
| Overall | 0.01% | 0.00% | -0.01% | -0.7% | 810.6% | #VALUE! |

Table 7: Comparison of FOMM Results Obtained against FOMM Results in JVET-AG0042 for Config 2

|  | FOMM |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over AG0042 GFVC |  |  |  |  |  |
|  | DISTS | LPIPS | RGB444_PSNR | RGB444_SSIM | YUV420_PSNR | YUV420_SSIM |
| Class A | -0.02% | -0.02% | -0.01% | -0.1% | 0.0% | 0.0% |
| Class B | 0.00% | 0.00% | 0.05% | 0.0% | 0.0% | -0.1% |
| Overall | -0.01% | -0.01% | 0.02% | 0.0% | 0.0% | -0.1% |

Table 8: Comparison of FV2V Results Obtained against FV2V Results in JVET-AG0042 for Config 2

|  | FV2V |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over AG0042 GFVC |  |  |  |  |  |
|  | DISTS | LPIPS | RGB444_PSNR | RGB444_SSIM | YUV420_PSNR | YUV420_SSIM |
| Class A | 0.01% | 0.02% | -0.08% | 0.0% | 0.8% | 0.1% |
| Class B | -0.03% | -0.03% | 0.01% | 0.0% | -0.2% | -0.1% |
| Overall | -0.01% | -0.01% | -0.03% | 0.0% | 0.3% | 0.0% |

Tables 6, 7 and 8 illustrate the comparison of results obtained for CFTE, FOMM and FV2V respectively, by testing the obtained results against the results reported in JVET-AG0042 as reference. Some deviations across the different metrics between the obtained results and the results in JVET-AG0042 were obtained.

## Results on GFVC methods for Config 3

This section reports the results obtained from Step 4 of the workflow pipeline introduced in Section 1 for Config 3, where each proposed GFVC method is used as transmission medium. In particular, experiments on each method were performed and then compared against the corresponding reported results. Note that the result “#VALUE!” means the curve overleap for some sequences so that BD rate cannot be calculated.

Table 9: Comparison of CFTE Results Obtained against CFTE Results in JVET-AG0042 for Config 3

|  | CFTE |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over AG0042 GFVC |  |  |  |  |  |
|  | DISTS | LPIPS | RGB444_PSNR | RGB444_SSIM | YUV420_PSNR | YUV420_SSIM |
| Class A | 0.00% | 0.00% | 0.00% | 0.0% | 0.0% | #VALUE! |
| Class B | 0.00% | 0.00% | 0.00% | 0.0% | 0.0% | 0.0% |
| Overall | 0.00% | 0.00% | 0.00% | 0.0% | 0.0% | #VALUE! |

Table 10: Comparison of FOMM Results Obtained against FOMM Results in JVET-AG0042 for Config 3

|  | FOMM |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over AG0042 GFVC |  |  |  |  |  |
|  | DISTS | LPIPS | RGB444_PSNR | RGB444_SSIM | YUV420_PSNR | YUV420_SSIM |
| Class A | 0.00% | 0.00% | 0.00% | 0.0% | 0.0% | 0.0% |
| Class B | 0.00% | 0.00% | 0.00% | 0.0% | 0.0% | 0.0% |
| Overall | 0.00% | 0.00% | 0.00% | 0.0% | 0.0% | 0.0% |

Table 11: Comparison of FV2V Results Obtained against FV2V Results in JVET-AG0042 for Config 3

|  | FV2V |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over AG0042 GFVC |  |  |  |  |  |
|  | DISTS | LPIPS | RGB444_PSNR | RGB444_SSIM | YUV420_PSNR | YUV420_SSIM |
| Class A | 0.00% | 0.00% | 0.00% | 0.0% | 0.0% | 0.0% |
| Class B | 0.00% | 0.00% | 0.00% | 0.0% | 0.0% | 0.0% |
| Overall | 0.00% | 0.00% | 0.00% | 0.0% | 0.0% | 0.0% |

Tables 9, 10 and 11 illustrate the comparison of results obtained for CFTE, FOMM and FV2V respectively, by testing the obtained results against the results reported in JVET-AG0042 as reference. As the bit-rate savings have values of 0.00% across all metrics, this illustrates an exact match in results obtained.

## Discussions in Deviations for Config 1 and Config 2

Some slight differences in the pixel values of reconstructed videos were observed due to possible slight differences in computations across different hardware-software configurations during generation of videos by GANs.

With these small deviations in pixels of the reconstructed videos during computation by GANs, metrics that are more sensitive to changes in pixel values such as PSNR and SSIM would observe larger deviations. In contrast, neural-network-based metrics such as DISTS and LPIPS that seek to measure similarity of the reconstructed frames would be less sensitive to slight changes in pixel values, leading to very small differences across different configurations.

Furthermore, as bitrate calculations are dependent on neural-network-based computations of key aspects across the frames of same sequence, slight differences were obtained due to differences in calculating movements of unsupervised frame-based key points for FOMM and FV2V or compact features for CFTE.

Since the goal of GFVC is to reconstruct talking-face videos at low bitrates by GANs through the extraction of key features across frames, these small differences in pixel values between original and reconstructed videos may be tolerable.

# Overall Obtained Results

In the spirit of JVET-AG0042, this section seeks to cover the overall results obtained during cross-check.

## Overall Results for Config 1

For Rate-DISTS, compared to VVC codec, CFTE, FOMM and FV2V achieve 60.24%, 34.05% and 47.52% average bit-rate savings respectively. For Rate-LPIPS, compared to VVC codec, CFTE, FOMM and FV2V achieve 58.09%, 30.19% and 45.39% average bit-rate savings respectively.

Table 12: Bit-rate Savings for CFTE compared against Traditional VVC Codec for Config 1

|  | CFTE |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over VVC (VTM 22.2) |  |  |  |  |  |
|  | DISTS | LPIPS | RGB444_PSNR | RGB444_SSIM | YUV420_PSNR | YUV420_SSIM |
| Class A | -64.75% | -63.65% | -5.34% | 6.7% | -2.2% | -28.8% |
| Class B | -55.73% | -52.52% | -3.29% | -23.3% | -3.1% | -22.0% |
| Overall | -60.24% | -58.09% | -4.32% | -8.3% | -2.7% | -25.4% |

Table 13: Bit-rate Savings for FOMM compared against Traditional VVC Codec for Config 1

|  | FOMM |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over VVC (VTM 22.2) |  |  |  |  |  |
|  | DISTS | LPIPS | RGB444_PSNR | RGB444_SSIM | YUV420_PSNR | YUV420_SSIM |
| Class A | -35.57% | -31.00% | 0.00% | 0.2% | 0.0% | 1.7% |
| Class B | -32.53% | -29.39% | 16.99% | -0.8% | 0.0% | #VALUE! |
| Overall | -34.05% | -30.19% | 8.49% | -0.3% | 0.0% | #VALUE! |

Table 14: Bit-rate Savings for FV2V compared against Traditional VVC Codec for Config 1

|  | FV2V |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over VVC (VTM 22.2) |  |  |  |  |  |
|  | DISTS | LPIPS | RGB444_PSNR | RGB444_SSIM | YUV420_PSNR | YUV420_SSIM |
| Class A | -51.87% | -50.72% | 3.01% | -21.0% | 1.0% | -20.0% |
| Class B | -43.18% | -40.05% | 0.84% | -12.2% | -0.9% | -9.6% |
| Overall | -47.52% | -45.39% | 1.92% | -16.6% | 0.0% | -14.8% |

The rate-distortion curves for DISTS and LPIPS of the GFVC methods against VVC are displayed below.

| Figure 1: Rate-Distortion Performance for CFTE compared against Traditional VVC Codec for Config 1 |  |
| --- | --- |
| Figure 2: Rate-Distortion Performance for FOMM compared against Traditional VVC Codec for Config 1 |  |
| Figure 3: Rate-Distortion Performance for FV2V compared against Traditional VVC Codec for Config 1 |  |

## Overall Results for Config 2

For Rate-DISTS, compared to VVC codec, CFTE, FOMM and FV2V achieve 60.24%, 34.04% and 47.52% average bit-rate savings respectively. For Rate-LPIPS, compared to VVC codec, CFTE, FOMM and FV2V achieve 58.09%, 30.19% and 45.38% average bit-rate savings respectively.

Table 15: Bit-rate Savings for CFTE compared against Traditional VVC Codec for Config 2

|  | CFTE |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over VVC (VTM 22.2) |  |  |  |  |  |
|  | DISTS | LPIPS | RGB444_PSNR | RGB444_SSIM | YUV420_PSNR | YUV420_SSIM |
| Class A | -64.75% | -63.65% | -5.34% | 6.7% | -2.2% | -28.8% |
| Class B | -55.73% | -52.52% | -3.29% | -23.3% | -3.1% | -22.0% |
| Overall | -60.24% | -58.09% | -4.32% | -8.3% | -2.7% | -25.4% |

Table 16: Bit-rate Savings for FOMM compared against Traditional VVC Codec for Config 2

|  | FOMM |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over VVC (VTM 22.2) |  |  |  |  |  |
|  | DISTS | LPIPS | RGB444_PSNR | RGB444_SSIM | YUV420_PSNR | YUV420_SSIM |
| Class A | -35.57% | -31.00% | 0.00% | 0.2% | 0.0% | 1.7% |
| Class B | -32.52% | -29.38% | 17.39% | -0.9% | 0.0% | 1.7% |
| Overall | -34.04% | -30.19% | 8.69% | -0.4% | 0.0% | 1.7% |

Table 17: Bit-rate Savings for FV2V compared against Traditional VVC Codec for Config 2

|  | FV2V |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over VVC (VTM 22.2) |  |  |  |  |  |
|  | DISTS | LPIPS | RGB444_PSNR | RGB444_SSIM | YUV420_PSNR | YUV420_SSIM |
| Class A | -51.85% | -50.71% | 2.97% | -21.0% | 0.7% | -20.0% |
| Class B | -43.19% | -40.06% | 0.79% | -12.0% | -0.9% | -9.7% |
| Overall | -47.52% | -45.38% | 1.88% | -16.5% | -0.1% | -14.8% |

The rate-distortion curves for DISTS and LPIPS of the GFVC methods against VVC are displayed below.

| Figure 4: Rate-Distortion Performance for CFTE compared against Traditional VVC Codec for Config 2 |  |
| --- | --- |
| Figure 5: Rate-Distortion Performance for FOMM compared against Traditional VVC Codec for Config 2 |  |
| Figure 6: Rate-Distortion Performance for FV2V compared against Traditional VVC Codec for Config 2 |  |

## Overall Results for Config 3

For Rate-DISTS, compared to VVC codec, CFTE, FOMM and FV2V achieve 60.25%, 34.04% and 47.52% average bit-rate savings respectively. For Rate-LPIPS, compared to VVC codec, CFTE, FOMM and FV2V achieve 58.09%, 30.18% and 45.38% average bit-rate savings respectively.

Table 18: Bit-rate Savings for CFTE compared against Traditional VVC Codec for Config 3

|  | CFTE |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over VVC (VTM 22.2) |  |  |  |  |  |
|  | DISTS | LPIPS | RGB444_PSNR | RGB444_SSIM | YUV420_PSNR | YUV420_SSIM |
| Class A | -64.76% | -63.66% | -5.35% | -28.1% | -2.2% | #VALUE! |
| Class B | -55.73% | -52.52% | -3.31% | -23.3% | -3.1% | -22.0% |
| Overall | -60.25% | -58.09% | -4.33% | -25.7% | -2.7% | #VALUE! |

Table 19: Bit-rate Savings for FOMM compared against Traditional VVC Codec for Config 3

|  | FOMM |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over VVC (VTM 22.2) |  |  |  |  |  |
|  | DISTS | LPIPS | RGB444_PSNR | RGB444_SSIM | YUV420_PSNR | YUV420_SSIM |
| Class A | -35.56% | -30.99% | 0.00% | 0.2% | 0.0% | 1.7% |
| Class B | -32.51% | -29.37% | 16.90% | -0.8% | -0.1% | 1.9% |
| Overall | -34.04% | -30.18% | 8.45% | -0.3% | 0.0% | 1.8% |

Table 20: Bit-rate Savings for FV2V compared against Traditional VVC Codec for Config 3

|  | FV2V |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over VVC (VTM 22.2) |  |  |  |  |  |
|  | DISTS | LPIPS | RGB444_PSNR | RGB444_SSIM | YUV420_PSNR | YUV420_SSIM |
| Class A | -51.86% | -50.72% | 3.00% | -21.0% | 0.6% | -20.0% |
| Class B | -43.18% | -40.05% | 0.81% | -12.1% | -0.9% | -9.6% |
| Overall | -47.52% | -45.38% | 1.91% | -16.6% | -0.2% | -14.8% |

# Patent rights declaration(s)

Panasonic does not have any current or pending patent rights relating to the technology described in this contribution.