| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0333-v1 |
| --- | --- |

| Title: | BoG report on ECM expert viewing preparations |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | M. Wien | Tel: Email: | wien@lfb.rwth-aachen.de |
| Source: | BoG |  |  |

_____________________________

# Agenda

- Develop a tentative plan for testing ECM performance by expert viewing tests at the 34th JVET meeting in April 2024

- Selection of test sequences / categories / resolution(s)

- Selection of configuration(s) to be tested

- Consideration of potential subsets of ECM tools to be tested.

- Proposal of a time line for AhG work between this and the next meeting.

# BoG Discussion

Thu 0600h.

Brief review of JVET-AB0041 “AHG4, 7, 12: Report on AHG meetings on ECM performance evaluation preparation” for checking the preparations for ECM viewing which have taken place before the Mainz meeting in October 2022 (reported in JVET-AB2029).

Discussion on the possible size of the test:

- About 30s of test time per PVS (processed video sequence)

- Separate sessions for different resolutions

- Recommended duration of one test session: 15min (20min max)

- In the JVET-AB2029 tests

  - 7 UHD sequences: all RA

  - 6 HD sequences: 3 RA, 3 LD

It was suggested to follow the basic layout of the test reported in JVET-AB2029. The size of the test could be extended a bit, but keeping it at about this size might be good. Rough calculation: 8 sequences × 4 rate points × 3 variants = 96 PVS under test, i.e. 48 of testing time with the DCR protocol. It was noted that using ACR-HR (Absolute Category Rating with Hidden Reference) can be done with about half of the testing time.

Approach: Start with a set of about 10 candidate sequences per resolution, narrow this down to the set to be used in the AhG phase.

It was suggested to focus on non-CTC sequences in the viewing tests. An overlap of the test sequences of the new test and the JVET-AB2029 test was considered useful to study the progression between the two tests. It was noted that testing non-CTC sequences would be more interesting than CTC sequences which already have been assessed in various previous tests during the development.

It was noted that the configuration files provided with the JVET CTC, which have been used in the visual tests, are significantly tuned towards optimization of PSNR. This also holds for the encoder decision making in the reference software packages.

It was noted that consideration of different testing methods might be helpful for different question: e.g., the comparison of VTM vs. ECM could be performed with an DCR test while ECM tool group tests could be done using A-B comparisons.

It was noted that using non-CTC sequences which appear to be challenging for the ECM would be interesting. It might be good to have a mix of sequences, i.e. to include ‘corner case’ sequences w.r.t. RD performance evaluated using PSNR.

Options for testing are:

- DCR, providing good differentiability of difference coding schemes, and comparability with the previous ECM test

- ACR-HR, being much faster than DCR, potentially at slightly less differentiability

- A-B comparisons, useful if smaller difference between versions are expected.

It was suggested to use ECM 11 for preparations as it is available for directly starting preparations for the tests. Using the CTC RA configuration and CTC LDB configuration is recommended to maintain comparability with the previous test.

Time line (meeting start on April 17th)

- April 18: Tests to be conducted in Rennes before the start of the MPEG week, latest on Sat

- April 14: Final bitstreams available

- April 01: Crosscheck of bitstreams complete

Volunteers for encoding and other preparation efforts:

- Google (X. Li)

- Alibaba (X. Li)

- HHI (A. Wieckowski)

- Ericsson (K. Andersson)

- Huawei (E. Alshina) (to be confirmed)

# BoG Recommendations

- Set up AhG calls for determining the rate points. 3 AhG meetings are suggested. Meeting slots accommodating the volunteers for encoding are suggested

  - Meeting in week of Feb 12: Determination of settings and sequence candidate list, determination of ECM group test layout

  - Meeting in week of Mar 18: review of results, determination of selection process to be viewed

  - Meeting in week of Apr 8: set of tentative candidate sequences

- Use of ECM-11

- Use CTC configurations

Session closed at 0714h.