| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes,FR, 17–24 April 2024 | Document: JVET-AH0010-v1 |
| --- | --- |

| Title: | JVET AHG report: encoding algorithm optimization (AHG10) |  |  |
| --- | --- | --- | --- |
| Status: | AHG report input to JVET |  |  |
| Purpose: | AHG report |  |  |
| Author(s) or Contact(s): | A. Duenas P. de Lagrange R. Sjöberg A. Tourapis | Email: | alberto.duenas@wbd.com philippe.delagrange@interdigital.com rickard.sjoberg@ericsson.com atourapis@apple.com |
| Source: | AHG |  |  |

_____________________________

# Abstract

This document summarizes the activities of AHG10 on encoding algorithm optimization, between the 33rd meeting (teleconference, 17–26 January 2024) and the 34th meeting (Rennes, FR, 17–24 April 2024).

# Mandates

At the 33nd meeting of the ITU-T/ISO/IEC Joint Video Experts Team (JVET), an ad hoc group on encoding algorithm optimization was established with the following mandates:

- Study the impact of using techniques such as tool adaptation and configuration, and perceptually optimized adaptive quantization for encoder optimization.

- Study the impact of non-normative techniques of preprocessing for the benefit of encoder optimization.

- Study encoding techniques of optimization for objective quality metrics and their relationship to subjective quality.

- Study optimized encoding for reference picture resampling and scalability modes in VTM, and coordinate with AHG4 on improving encoders and test settings for multi-layer verification testing.

- Study optimized encoding and suitable test settings for noisy materials, such as sequences containing film grain.

- Study optimized encoding and tool combinations for low latency and low complexity.

- Consider neural network-based encoding optimization technologies for video coding standards.

- Investigate other methods of improving objective and/or subjective quality, including adaptive coding structures and multi-pass encoding.

- Study methods of rate control and rate-distortion optimization and their impact on performance, subjective and objective quality.

- Study the potential of defining default or alternate software configuration settings and test conditions optimized for either subjective quality, or higher objective quality, and coordinate such efforts with AHG3 and AHG6.

- Study the effect of varying configuration parameters depending on temporal layer, such as those related to deblocking, partitioning, chroma QP.

The regular JVET e-mail reflector was used for discussions (jvet@lists.rwth-aachen.de). No e-mail related to AHG10 activity was sent to the JVET reflector during the AHG period.

# Related contributions

Seven contributions, not including cross-checks, are identified relating to AHG10, and summarized in the following sections.

## Adaptive resolution

### JVET-AH0130 - AHG8: RPR for machine consumption

This contribution proposes to reduce resolution by 1.25x for chunks of 32 pictures, with a decision driven by object detection loss in reduced resolution. Gains are reported for machine tasks.

### JVET-AH0171 - AHG10/AHG12 GOP-based RPR encoder control results on UHD test data

This contribution is a follow-up of JVET-AF0058 and JVET-AG0116, measuring the performance of GOP-based RPR encoding (using the “GOPBasedRPR” configuration parameter) with VTM-23.0 and ECM-12.0, on UHD sequences considered for ECM visual performance assessment (see JVET-AH0041).

BD-rate differences around -3.0% (Y-PSNR) and -6.6% (Y-MSSIM) are reported for VTM, and respectively around -0.15% and -3.4% for ECM. Visual quality improvement is also reported. It is suggested to include GOP-based RPR feature into VTM11_ANC too for consistency.

### JVET-AH0249 - AHG8: An adaptive spatial resampling method for machine vision

This contribution proposes adaptive 2x reduced resolution coding for machine tasks, with downscaling and upscaling performed by CNN models (3 and 20 layers respectively). Performance improvements were observed.

## Local QP optimization

### JVET-AH0078 – AhG10: Distortion Propagation Factor for VVC Low-Delay Configuration

This contribution is a follow-up of JVET-AF0089 and JVET-AG0055, that propose a method that tries to give more quality weight to local areas that are likely to be reused in other picture (through temporal prediction). It does so by estimating a “distortion propagation factor” with a fast 1st inter pass.

During the 33rd JVET meeting, progress was acknowledged, and it was suggested to further investigate about the non-homogeneous behaviour over different sequences.

In this follow-up version, a propagation length was introduced to differentiate quality propagation from key and non-key frames. BD-rate difference around -4.1% are reported for VVC low-delay configurations (class B and below). Speed optimization (1st pass only for key frames) reduce the BD-rate to around -3.8% while cutting the extra encoder time by more than 2.

Compared to JVET-AG0055 per-sequence performance differences have been reduced, with around 1% BD-rate change for sequences with losses.

As a reminder, the gains of BIM (comparable technique, but single-pass relying on MCTF) are around 2.3% in the low-delay test conditions.

## Residual spatial weighting

### JVET-AH0248 – Reduced residual encoding in VVC for machine consumption

This contribution is a follow-up of JVET-AG0217 where a residual spatial weighting was proposed. The residuals are scaled by 70% in the center of a coding block -when larger than 4 in either dimension- but preserving DC). Compared to previous contribution, combination with Region of Interest was tested. BD-rate (PSNR) loss is around 0.5% but the method can bring gains for machine tasks.

## Low latency and low complexity

### JVET-AH0180 – AHG12: Modified CTC for encoding time reduction under low-delay configuration

This contribution is a follow-up of JVET-AE0163, and proposes further encoder configuration changes (MTT depth reduction for some temporal layer at low QP) in low-delay B common test conditions to reduce the worst case encoding time of ECM-12 (+2 days compared to ECM-11). Minor losses with significant reduction of encoding times are reported (including a plot showing several trade-offs and comparison with ECM-11).

### JVET-AH0199 – On the requirements and use cases of new-generation video coding standard

This contribution advocates for more focus on low-delay and low-complexity in the study of next-generation coding, based on market forecast of related applications. It is suggested to include updated HDR and low-delay test material (gaming, user generated, remote screens) in the CTCs, and potentially include complexity constraints for low-delay. Comparison with both HEVC and VVC is also suggested.

# Recommendation

The AHG recommends that the related input contributions are reviewed and to further continue the study of encoding algorithm optimizations in JVET.