| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 17–24 April 2024 | Document: JVET- AH0100-v2 |
| --- | --- |

| Title: | EE1-4.2-related: Multiple scaling ratios coding for NNVC with NNSR |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhuoyi Lv Chuan Zhou | Email: | zhuoyi.lv@vivo.com chuan.zhou@vivo.com |
| Source: | vivo Mobile Communication Co., Ltd |  |  |

_____________________________

- Abstract

EE1-4.2 investigates the use of a unified CNN-based super-resolution method to handle both scaling ratios of 2x and 1.5x. Based on EE1-4.2, this contribution proposes a more generic and flexible method for NNVC.

Specifically, in order to handle multi-scaling ratios in one coding process, firstly encoder adaptively chooses one of the three scaling ratios, i.e. 2x, 1.5x and 1.0x, at GOP level. Then adjust QPs corresponding to the selected scaling ratio (2x and 1.5x). The scaling ratios are input to the unified CNN-based super resolution, which is same as EE1-4.2

The overall BD rate savings are {-0.94%, 1.84%, 1.78%} and {-1.01%, 1.22%, 1.14%} for AI and RA configurations, respectively. The average BD rate savings for ClassA1 and ClassA2 are reported as follows:

- AI: {-2.07%,	4.00%, 2.95%} for Class A1, {-3.60%, 7.06%, 7.75%} for Class A2;

- RA:{-3.59%,	0.40%, 0.27%} for Class A1, {-1.46%, 5.68%, 5.45%} for Class A2.

Introduction

Current NNVC8.0 allows the encoder to adaptively decide whether i-th GOP will be down-scaled with a ratio of 2 by comparing the down-up scaled PSNR for the first picture of i-th GOP with a threshold.

In EE1-4.2, two tests are investigated, sharing the same CNN-based super resolution model, using scaling factor s=2 (EE1-4.2.1) and s=1.5 (EE1-4.2.2) and different QP offsets correspondingly.

Proposed method

Encoder strategy for supporting multi-scaling ratios

The PSNR is calculated between original and down-up scaled picture using down-sampling filters in RPR with a ratio of 2.0, where the picture is the first picture of i-th GOP.

Figure 1 Flowchart of encoder strategy

- For RA, if PSNRU>thrUV and PSNRV>thrUV, and PSNRY>thrY, scaling ratio is set equal to 2.0.

For AI, if PSNRU>thrUV and PSNRV>thrUV, scaling ratio is set equal to 2.0.

- else

  - if PSNRY>(basePSNR)-QPi-BaseQP*0.5, scaling ratio is set equal to 2.0.

  - else if PSNRY>(basePSNR-offsetPSNR)-QPi-BaseQP*0.5, scaling ratio is set equal to 1.5.

  - else, scaling ratio is set equal to 1.0.

In this contribution, basePSNR is set equal to 47 and 44 for RA and AI, respectively. baseQP is set equal to 32. OffsetPSNR is set equal to 2 and 3 for RA and AI, respectively.

According to selected scaling ratio for i-th GOP, QP offset is set equal to -5 and -4 for 2.0x and x1.5x respectively.

Network structure

As shown in Fig. 2, the CNN-based super-resolution model in EE1-4.2 [1] is utilized, except that the stride of convolution is set to 2 for all input feature extraction layers and the stride of 2 in the transition layer is removed to further reduce complexity.

Scaling ratio (Sr) is set as an input to the model:

- For scaling ratio is 2.0x, Sr = 1;

- For scaling ratio is 1.5x, Sr = 2;

- For scaling ratio is 1.0x, Sr = 0.

Figure 2 Network structure

Training details

Table 1 Network information for models in training stage

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | A100 |
|  | Framework: | PyTorch v2.1.1 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | 60 |
|  | Batch size: | 128 |
|  | Training time: | ~10 days |
|  | Training data information: | DIV2K, BVI_DVC, TVD |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | Turn on NNLF-LOP2 and NNIntra |
|  | Loss function: | L1, L2 |
| Optional | Number of iterations |  |
|  | Patch size | 128 |
|  | Learning rate: | 4e-4 |
|  | Optimizer: | ADAM |
|  | Preprocessing: |  |
|  | Other information: |  |

Inference

The network information in the inference stage is provided in Table 2.

Table 2 Network information for the proposed models in inference stage

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: |  |
|  | Framework: | SADL |
|  | Number of GPUs per Task |  |
|  | Number of Parameters (Each Model) | 0.05M |
|  | Total Parameter Number | 0.05M |
|  | Parameter Precision (Bits) | 32 (F) |
|  | Memory Parameter (MB) | 0.2MB |
|  | Multiply Accumulate (MAC) | 12kMAC/pixel |
| Optional | Total Conv. Layers |  |
|  | Total FC Layers | 0 |
|  | Batch size: | 1 |
|  | Patch size | 256 |

Experimental results

The proposed method is tested on top of NNVC 8.0, according to the common test condition [2].

Table 3 Multiple scaling ratios coding with RPR

|  | AI Main10 |  |  | RA Main10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | -1.75% | 4.40% | 4.35% | -3.13% | 1.19% | 2.40% |
| Class A2 | -1.64% | 12.17% | 12.84% | -0.51% | 8.57% | 8.37% |
| Overall | -0.56% | 2.76% | 2.86% | -0.73% | 1.95% | 2.15% |

Table 4 Multiple scaling ratios coding with NNSR (EE1-4.2 model)

|  | AI Main10 |  |  | RA Main10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | -2.15% | 3.83% | 3.12% | -3.70% | 0.28% | 0.60% |
| Class A2 | -3.56% | 6.80% | 7.68% | -1.47% | 5.49% | 5.43% |
| Overall | -0.95% | 1.77% | 1.80% | -1.03% | 1.16% | 1.21% |

Table 5 Multiple scaling ratios coding with NNSR (simplified EE1-4.2 model)

|  | AI Main10 |  |  | RA Main10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | -2.07% | 4.00% | 2.95% | -3.59% | 0.40% | 0.27% |
| Class A2 | -3.60% | 7.06% | 7.75% | -1.46% | 5.68% | 5.45% |
| Overall | -0.94% | 1.84% | 1.78% | -1.01% | 1.22% | 1.14% |

Conclusion

In this contribution, a more generic and flexible method for NNVC is proposed, which handles multi-scaling ratios in one coding process. It is proposed to investigate the method in the next round of EE1.

References

- C. Zhou, Z. Lv, “EE1-4.2: Adaptive resolution coding for NNVC-8”, JVET-AH0104

- E. Alshina, R.-L. Liao, S. Liu, A. Segall, “Common test conditions and evaluation procedures for neural network-based video coding technology”, JVET-AF2016

Patent rights declaration(s)

vivo Mobile Communication Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).