| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 17–24 April 2024 | Document: JVET-AH0109-v1 |
| --- | --- |

| Title: | AHG16: Removal of the analysis model on the decoder side and lightweight design |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Renjie Zou Ru-Ling Liao Bolin Chen Yan Ye Jie Chen | Email: | {zourenjie.zrj, ruling.lrl, chenbolin.chenboli, yan.ye jiechen.cj}@alibaba-inc.com |
| Source: | Alibaba Group |  |  |

_____________________________

- Abstract

This contribution proposes two changes to the Generative Face Video Compression (GFVC) methods supported in the AHG16 software. Firstly, the analysis model is removed from the decoder and the features of the base pictures are directly coded in the bitstream instead. This is applied to all three GFVC methods, CFTE, FOMM and FV2V, in the AHG16 GFVC software. The complexity (in terms of number of parameters and kMACs per pixel) and performance (in terms of {DISTS, LPIPS, RGB444_PSNR, RGB444_SSIM, YUV420_PSNR, YUV420_SSIM}) of this proposed element are reported as follows:

- For CFTE, complexity is reduced by 24.1% and 1.8%, coding performance is {1.32%, 1.39%, 2.16%, -0.1%, 9.9%, #VALUE!} compared with the current CFTE, and {-59.66%, -57.47%, -3.05%, -27.4%, -4.3%, #VALUE!} compared with VTM-22.2.

- For FOMM, complexity is reduced by 23.7% and 2.3%, coding performance is {2.37%, 2.50%, 8.24%, 2.7%, 18.3%, #VALUE!} compared with the current FOMM, and {-32.37%, -28.48%, 2.98%, 1.3%, 1.4%, #VALUE!} compared with VTM-22.2.

- For FV2V, complexity is reduced by 24.1% and 1.6%, coding performance is {1.45%, 1.54%, 1.54%, 2.3%, -0.8%, 2.0%} compared with the current FV2V, and {-46.69%, -44.54%, 5.37%, -14.0%, 4.3%, -12.8%} compared with VTM-22.2.

In the second element of this proposal, the Lightweight CFTE in AHG16 software is further simplified by selectively replacing 3x3 convolution layers with depthwise separable convolution, resulting in a 50% reduction in the number of channels in the generative model. Combined with the first element, the proposed lightweight CFTE achieves complexity reduction of 58.6% and 76.3%, respectively. Its coding performance is {0.38%, 2.99%, 0.45%, 9.9%, 4.4%, 262409.8%} compared with the current lightweight CFTE in the AHG16 software, and {-62.48%, -59.54%, -4.91%, -26.9%, -2.0%, -27.3%} compared with VTM-22.2. It should be noted that the abnormal number observed in YUV420_SSIM is attributable to a specific sequence.

Introduction

Figure 1 illustrates the current general block diagram of generative face video compression (GFVC) software used in AHG16 activities. As shown in Figure 1, the base picture is encoded using VTM software, whereas the subsequent pictures are analyzed using the analysis model and the features are coded on the encoder side. On the decoder side, the base picture is firstly decoded using VTM software and the reconstructed basic picture is fed into the analysis model to get its reconstructed base features. Then, the dense motion model uses the reconstructed base features and the decoded features of the subsequent pictures as input to generate motion information. Finally, the generative model generates reconstructed subsequent pictures using the generated motion information and reconstructed base picture.

The current AHG16 GFVC software supports four GFVC methods, including FOMM[6], FV2V[7], CFTE[1] and lightweight CFTE[4]. The lightweight CFTE is proposed in JVET-AG0139 [4] which replaces partial 3x3 convolution layers by depthwise separable convolution in dense motion model to reduce the number of parameters and computation complexity. In this proposal, we propose to directly code the features of the base pictures into the bitstream rather than relying on the decoder to perform analysis to obtain the base picture features. This allows for the elimination of the analysis model for all these four GFVC methods at the decoder side. Moreover, we propose to further reduce the computation complexity of lightweight CFTE by reducing number of channels and selectively replacing some of the 3x3 convolution layers within the generative model with depthwise separable convolution layers.

Figure 1. Illustration of general block diagram of GFVC.

Proposed methods

Eliminating the analysis model from the decoder

The analysis model is removed in all four GFVC methods and the proposed block diagram is depicted in Figure 2. Instead, the features of the base pictures are first directly encoded into the bitstream by the encoder and subsequently decoded by the decoder, obliviating the need for the analysis model at the decoder side. This is asserted to be a software change only, and does not require any changes to the GFV SEI message under development, as it already supports coding of features of the base pictures.

Figure 2. Illustration of the streamlined decoding process and corresponding encoding process.

Complexity reduction of generative model in lightweight CFTE

An analysis is performed to measure the number of parameters and kMACs per pixel of each model in the lightweight CFTE method, and the results is shown in Table 1. As can be seen, the generative model has the highest kMACs per pixel among all three models.

Table 1. Number of parameters and kMACs/pixel of each model in the lightweight CFTE method

| Model | Analysis model | Dense motion model | Generative model |
| --- | --- | --- | --- |
| Parameters | 14.1M | 12.3M | 7.9M |
| kMACs/pixel | 15.1 | 47.9 | 756.1 |

To reduce the complexity of generative model, we implement this by halving the channel count in the generative model and replacing 3x3 convolutions with depthwise separable convolutions in the down-sample blocks. Figure 3 illustrates these modifications.

Figure 3. Illustration of modifications in the generative model.

Simulation results

The proposed methods are implemented on top of the AHG16 GFVC software, and are evaluated using the test conditions and evaluation procedures specified in JVET-AG2035 [5]. Table 3 and Table 4 show the simulation results of removing the analysis model in CFTE over VTM-22.2 and the current CFTE in AHG16 GFVC software, respectively. Similarly, Table 4 to Table 7 show the simulation results of removing the analysis model in FOMM and FV2V. It can be seen that overall, removing analysis model in CFTE, FOMM and FV2V introduces small loss over the original design. It should be mentioned that the result “#VALUE!” means the RD curves cross over each other for some sequences so that the BD-rate cannot be calculated.

Table 2. Simulation results of removing analysis model in CFTE over VTM-22.2

|  | Over VTM-22.2 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | DISTS | LPIPS | RGB444_PSNR | RGB444_SSIM | YUV420_PSNR | YUV420_SSIM |
| Class A | -64.40% | -63.28% | -4.73% | -33.0% | -2.1% | -27.2% |
| Class B | -54.92% | -51.66% | -1.36% | -21.9% | -6.4% | #VALUE! |
| Overall | -59.66% | -57.47% | -3.05% | -27.4% | -4.3% | #VALUE! |

Table 3. Simulation results of removing analysis model in CFTE over the current CFTE

|  | Over CFTE |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | DISTS | LPIPS | RGB444_PSNR | RGB444_SSIM | YUV420_PSNR | YUV420_SSIM |
| Class A | 1.00% | 1.04% | 0.97% | 0.3% | 1.1% | #VALUE! |
| Class B | 1.63% | 1.75% | 3.36% | -0.5% | 18.8% | #VALUE! |
| Overall | 1.32% | 1.39% | 2.16% | -0.1% | 9.9% | #VALUE! |

Table 4. Simulation results of removing analysis model in FOMM over VTM-22.2

|  | Over VTM-22.2 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | DISTS | LPIPS | RGB444_PSNR | RGB444_SSIM | YUV420_PSNR | YUV420_SSIM |
| Class A | -34.49% | -29.84% | 0.00% | 1.3% | 0.0% | #VALUE! |
| Class B | -30.25% | -27.12% | 5.96% | 1.3% | 2.8% | 10.2% |
| Overall | -32.37% | -28.48% | 2.98% | 1.3% | 1.4% | #VALUE! |

Table 5. Simulation results of removing analysis model in FOMM over the current FOMM

|  | Over FOMM |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | DISTS | LPIPS | RGB444_PSNR | RGB444_SSIM | YUV420_PSNR | YUV420_SSIM |
| Class A | 1.60% | 1.70% | 4.07% | 2.5% | 24.7% | #VALUE! |
| Class B | 3.14% | 3.31% | 12.42% | 3.0% | 11.8% | 3.6% |
| Overall | 2.37% | 2.50% | 8.24% | 2.7% | 18.3% | #VALUE! |

Table 6. Simulation results of removing analysis model in FV2V over VTM-22.2

|  | Over VTM-22.2 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | DISTS | LPIPS | RGB444_PSNR | RGB444_SSIM | YUV420_PSNR | YUV420_SSIM |
| Class A | -51.36% | -50.23% | 0.11% | -20.4% | -0.8% | -19.5% |
| Class B | -42.03% | -38.85% | 10.63% | -7.6% | 9.4% | -6.1% |
| Overall | -46.69% | -44.54% | 5.37% | -14.0% | 4.3% | -12.8% |

Table 7 Simulation results of removing analysis model in FV2V over the current FV2V

|  | Over FV2V |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | DISTS | LPIPS | RGB444_PSNR | RGB444_SSIM | YUV420_PSNR | YUV420_SSIM |
| Class A | 1.02% | 1.02% | 0.30% | 0.7% | -3.2% | 0.8% |
| Class B | 1.89% | 2.06% | 2.78% | 3.9% | 1.7% | 3.2% |
| Overall | 1.45% | 1.54% | 1.54% | 2.3% | -0.8% | 2.0% |

Table 8 presents a comparative overview of the number of parameters and kMACs per pixel on the decoder side. It can be seen that the number of parameters and kMACs per pixel are decreased roughly by 24% and 2% for all three CFTE, FOMM and FV2V methods, respectively.

Table 8. Number of parameters and kMACs/pixel of analysis model elimination

|  | Total parameters number | kMACs/pixel |
| --- | --- | --- |
| Current CFTE | 58.0M | 851.7 |
| Proposed CFTE | 43.9M | 836.6 |
| Current FOMM | 59.8M | 838.0 |
| Proposed FOMM | 45.6M | 818.5 |
| Current FV2V | 125.1M | 7562.3 |
| Proposed FV2V | 95.0M | 7443.2 |

Table 9 and Table 10 show the simulation results of removing the analysis model and simplifying generative model in lightweight CFTE over VTM-22.2 and the current lightweight CFTE in AHG16 GFVC software, respectively. Table 11 summarizes the number of parameters and kMACs per pixel of the original and proposed lightweight CFTE. As can be seen, the proposed lightweight CFTE introduces minor coding loss while the number of parameters is reduced by 59% and kMACs per pixel is decreased by 76%. It should be noted that the abnormal number observed in YUV420_SSIM is attributable to a specific sequence CFVQA_Seq009, in comparison with the current lightweight CFTE. Figure 4 shows the RD curves of CFVQA_Seq009.

Table 9. Simulation results of removing analysis model and complexity reduction in lightweight CFTE over VTM-22.2

|  | Over VTM-22.2 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | DISTS | LPIPS | RGB444_PSNR | RGB444_SSIM | YUV420_PSNR | YUV420_SSIM |
| Class A | -67.02% | -65.46% | -3.12% | -34.6% | 0.0% | -29.7% |
| Class B | -57.94% | -53.63% | -6.71% | -19.2% | -4.0% | -25.0% |
| Overall | -62.48% | -59.54% | -4.91% | -26.9% | -2.0% | -27.3% |

Table 10. Simulation results of removing analysis model and complexity reduction in lightweight CFTE over lightweight CFTE

|  | Over lightweight CFTE |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | DISTS | LPIPS | RGB444_PSNR | RGB444_SSIM | YUV420_PSNR | YUV420_SSIM |
| Class A | 1.04% | 2.01% | -2.51% | 2.8% | 0.8% | 1.4% |
| Class B | -0.28% | 3.97% | 3.41% | 16.9% | 8.0% | 524818.2% |
| Overall | 0.38% | 2.99% | 0.45% | 9.9% | 4.4% | 262409.8% |

Table 11. Comparison of the proposed lightweight CFTE on number of parameters and kMACs/pixel

|  | Total parameters number | kMACs/pixel |
| --- | --- | --- |
| Current lightweight CFTE | 34.3M | 819.1 |
| Proposed lightweight CFTE | 14.2M | 194.2 |

Figure 4. RD curves of CFVQA_Seq009

Conclusions

This contribution proposes two modifications to the AHG16 GFVC software aimed at reducing the complexity. The first modification involves removing the analysis model from the decoder, which results in a decrease in both parameters and complexity. It is also asserted that this modification introduces only a software change, and allows a more flexible design of the analysis model to be used by the encoder. The second modification includes adopting depthwise separable convolutions and halving channel counts in the generative model of the lightweight CFTE framework. The overall reductions in complexity makes a compelling case for their implementation in resource-constrained scenarios. We recommend to adopt both proposed modifications to the AHG16 GFVC software.

References

- B. Chen, et al., “Beyond keypoint coding: Temporal evolution inference with compact feature representation for talking face video compression,” Proc. Data Compression Conference (DCC), Mar. 2022.

- B. Chen, et al., “AHG16: Proposed Common Software Tools and Testing Conditions for Generative Face Video Compression,” JVET-AG0042, Jan. 2024.

- F. Chollet, “Xception: Deep learning with depthwise separable convolutions,” IEEE Conference on Computer Vision and Pattern Recognition (CVPR), Nov. 2017.

- R. Zou, et al., “AHG16: Depthwise separable convolution for generative face video compression,” JVET-AG0139, Jan. 2024.

- S. McCarthy and B. Chen, “Test conditions and evaluation procedures for generative face video coding,” JVET-AG2035, Jan. 2024.

- Siarohin A, Lathuilière S, Tulyakov S, et al. First order motion model for image animation[J]. Advances in Neural Information Processing Systems, 2019, 32.

- Wang T C, Mallya A, Liu M Y. One-shot free-view neural talking-head synthesis for video conferencing[C]//Proceedings of the IEEE/CVF conference on computer vision and pattern recognition. 2021.

Patent rights declaration(s)

Alibaba group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).