| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 17–24 April 2024 | Document: JVET-AH0110-v1 |
| --- | --- |

| Title: | AHG 16: Scalable Representation and Layered Reconstruction for Generative Face Video Compression |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Bolin Chen, Yan Ye, Jie Chen, Ru-Ling Liao, Shanzhi Yin, Shiqi Wang | Email: | chenbolin.chenboli@alibaba-inc.com, yan.ye@alibaba-inc.com, jiechen.cj@alibaba-inc.com, ruling.lrl@alibaba-inc.com shanzhyin3-c@my.cityu.edu.hk, shiqwang@cityu.edu.hk |
| Source: | Alibaba and City University of Hong Kong |  |  |

_____________________________

- Abstract

At the 33th JVET meeting, a unified software package as well as test conditions for generative face video coding (GFVC) were established for JVET AHG16 experimentation. The AHG16 software incorporates various GFVC methods and allows coding to be performed using VVC Main 10 profile. However, it has been observed that the current GFVC software can only achieve promising face video compression in ultra-low bitrate ranges, and cannot provide a more comprehensive coverage for mid- to high bitrate ranges.

In this contribution, we propose a scalable representation and layered reconstruction (SRLR) architecture to extend the bitrate and quality range that GFVC methods in AHG16 software can cover. Experimental results show that the proposed SRLR architecture can extend the quality and bitrate ranges for all three GFVC methods supported in the AHG16 software for all quality metrics in the GFVC test conditions. Further, visual examples are shown to verify that the proposed SRLR architecture can reduce some annoying visual artifacts caused by GFVC methods.

Introduction

Generative face video compression (GFVC) [2]-[7] can achieve promising rate-distortion performance especially at ultra-low bitrate ranges thanks to its ability to use compact features to achieve high-quality image reconstruction via strong inference capabilities of deep generative models [1]. Inspired by this, a series of technical proposals [8]- [25] on GFVC have been submitted to the JVET regarding carrying GFVC parameters using SEI messages, GFVC software implementation, test conditions, coordinated experimentation, interoperability study and so on. More specifically, JVET-AG0042 [17] proposed a unified software tool for experimentation with different GFVC methods and JVET-AG2035 [25] specified test conditions and software reference configurations for GFVC testing. At the 33rd meeting, it was agreed to establish GFVC software and test conditions as part of AHG16 activities.

The current AHG16 GFVC software can only achieve promising rate-distortion performance at ultra-low bitrate ranges. In this contribution, we propose scalable representation and layered reconstruction (SRLR) to extend the bitrate range coverage for all existing GFVC algorithms in the AHG16 software. In particular, we develop a multi-granularity facial description strategy to regularize long-term dependencies between video frames and compensate for motion estimation errors caused by compact representations of motion information. By exploiting the visual information at different granularities, the enhanced coding framework could achieve satisfactory reconstruction visual quality within a wider range of coded bitrates.

Problem Statements

Lossy video compression aims at achieving the lowest possible perceived distortion (i.e., D) given the bitrate budget (i.e., R). Alternatively, it can be relaxed as a rate-distortion optimization problem regarding how to minimize the overall cost ȷcost with a trade-off coefficient between R and D as follows,

ȷcost=D+λR,

where λ is the Lagrange multiplier representing the R-D relationship for a particular quality level. Though the RD relationship that distortion decreases along with the increase of bitrate has been regarded as a golden principle in video coding, it is argued that the current generative coding does not follow this rule, as shown in Figure 1. The specific phenomena and analyses are listed below,

- Limited bitrate coverage. Figure 1 (a)&(b) provide the RD performance for the VVC codec and GFVC algorithms in terms of perceptual-level measures (i.e., Deep Image Structure and Texture Similarity (DISTS) and Learned Perceptual Image Patch Similarity (LPIPS)), revealing that allocating more coding bits to base pictures can only improve perceptual quality within a limited bitrate range. Towards mid to high bitrate ranges, allocsting more bits to the base pictures no long improves the perceptual quality of the reconstructed (i.e. generated) images. As a result, the reconstructed quality and coded bitrates of the current GFVC algorithms in AHG16 software are constrained within a limited range, whereas in comparsion, the quality and rate coverage of VVC is much wider and not saturated.

(a) Rate – DISTS                                 (b) Rate -LPIPS

Figure 1 – Rate-distortion performance comparisons for VVC codec and the AHG16 software [17] in terms of perceptual-level measures (DISTS and LPIPS), where the VVC codec adopts LDB configuration with the QPs of 22/27/32/37/42/47/52 and the base picture of GFVC is compressed via VVC intra mode with the QPs of 2/7/12/17/22/32/42/52.

- Unstable reconstruction quality. Although deep generative models possess powerful inference capabilities, their robustness in complex motion and long-term dependency scenes cannot always be guaranteed. As a result, GFVC is prone to failure cases with annoying artifacts, missing details and temporal inconsistency.  Figure 2 provides examples where annoying artifacts exist in the reconstructed (i.e. generated) images using the existing GFVC approaches, including  low face fidelity, inaccurate local motion in eye/mouth regions, temporal inconsistency, and occlusion artifacts. These unacceptable visual reconstruction results can be easily perceived by humans. However, it is worth mentioning that allocating more bits to the base pictures cannot alleviate such distortions.

(a) Reference        (b) Generated Occlusions    (c) Low Face Fidelity     (d) Poor Local Motion

Figure 2 – Limitations of visual reconstruction quality for GFVC algorithms, where these generated faces are sourced from FOMM [3], CFTE [5] and FV2V [4].

Proposed SRLR Algorithm

Algorithm Overview

The proposed SRLR-based algorithm is rooted in the view that face frames own prior statistics and can be automatically characterized into meaningful representations, such as latent code (i.e., keypoints, facial semantics and compact feature) and enriched signal (i.e., residual signal, segmentation map and dense flow). The proposed SRLR-based algorithm is shown in Figure 3, consisting of base and enhancement layers.

As shown in Figure 3, the base layer can be one of any existing GFVC algorithms in AHG16 software [17], which can achieve ultra-low bitrate face video communication with compact representations. The face data is characterized with latent code (i.e., keypoints, facial semantics and compact feature) via the analysis model at the encoder side, and the extracted information is further compressed and conveyed in the bitstream to reconstruct face via the synthesis model at the decoder side.

Afterwards, the enhancement layer is our proposed SRLR-based algorithm that is capable of providing different-granularity feature space with enriched facial motion information and support high-quality face video communication when bandwidth permit. The encoder can further characterize visual face data with different-granularity facial feature space and compress them into the bitstream, whilst the decoder receives the bitstream and exploits these decoded auxiliary facial signals to improve the reconstruction quality of the base-layer reconstructed pictures. Specifically, the base-layer resonctructed subsequent pictures Pt and the original subsequent pictures Pt are separately fed into the multi-granularity feature descriptor to obtain the corresponding learned feature space, and then they do a difference operation to remove spatial redundancy. After undergoing a series of coding operations (i.e., quantization, parameter encoding/decoding, inverse quantization), the reconstructed feature map will be further input into the face enhancement model with the combination of reconstructed base picture P0 and resonctructed subsequent pictures Pt, such that the enhanced subsequent pictures Pt can be obtained. It should be mentioned that the different-granularity feature space only can be produced by the corresponding fixed multi-granularity feature descriptor.

Figure 3 – Overview of proposed algorithm based on scalable representation and layered reconstruction for bandwidth-intelligent face video communication, which can be compatible with all existing GFVC algorithms.

Algorithm Details

Herein, Figure 4 provides the details of the proposed enhanced coding framework based on scalable representation and layered reconstruction, aiming to improve the reconstruction quality and signal fidelity of base-layer face frames via enriched facial signal description. It should be further clarified that all different-granularity models are trained separately within the same architecture design instead of a unified training.

- Multi-granularity feature descriptor: can hierarchically compensate for motion estimation errors by delivering multi-granularity feature representation to describe the important motion information (e.g., expression and headpose) and complex background changes in a more scalable and flexible manner.

- The input is base-layer reconstructed inter frames and the original inter frames.

- The output is the residual of multi-granularity feature space with the possible size of 8*8*1, 16*16*1, 24*24*1, 32*32*1 and etc.

- Learning-based feature compression model: can improve the compression efficiency of multi-granularity feature residuals via the joint autoregressive and hierarchical priors-based entropy model [27].

- The input is multi-granularity feature residuals.

- The output is the multi-granularity feature bitstream.

- Attention-guided face enhancement model: can reconstruct more reliable and robust face video. Specifically, based on the attention mechanism, the decoded multi-granularity feature representation can better compensate for the motion estimation error and provide extra facial structure and background information for the base-layer reconstructed face signal. In addition, to avoid the error accumulation of inaccurate base-layer texture representation, the decoded base picture is further provided with a coarse-to-fine generative strategy to ensure that the final generation result has high-quality texture reference.

- The input is base-layer reconstructed inter frames (subsequent pictures), the decoded multi-granularity feature residuals and the decoded base picture.

- The output is the enhanced inter frames (subsequent pictures).

Figure 4 – Illustration of the proposed SRLR-based architecture regarding how to achieve multi-granularity facial description, entropy-based feature compression and attention-guided face signal enhancement. In particular, Q, AE, AD and Conv denote the quantization, arithmetic encoding, arithmetic decoding and convolution processes, respectively.

Experiment

In this section, we provide the experimentation and performance analysis using proposed SRLR-based algorithms. The proposed SRLR is implemented on top of the GFVC software of AHG16.

Testing Conditions

Herein, we follow the test conditions and software reference configurations for GFVC testing specified in JVET-AG2035 [26]. It should be noted that since the proposed SRLR-based algorithms is designed for high bitrate coverage, we propose to modify test settings as follows,

- Low bitrate ranges:

- For VVC, VTM 22.2 is used and the test is conducted under low-delay B (LDB) configuration, and the quantization parameters (QP) is set from 37/42/47/52.

- For GFVC, its base picture is compressed as an I picture using the VTM reference software 22.2, and other pictures are further represented with a series of facial parameters and compressed via a context-adaptive arithmetic coder. The QPs of the base picture are set from 22/32/42/52.

- High bitrate ranges:

- For VVC, VTM 22.2 is used and the test is conducted under low-delay B (LDB) configuration, and the quantization parameters (QP) is set from 27/32/37/42.

- For SRLR-based algorithm, it includes two layers. Specifically, the base picture of base-layer GFVC algorithm is compressed using the VTM reference software 22.2 with the QP of 17. Besides, the enhancement layer includes 3 different feature granularities with the feature size of 8*8, 16*16, 24*24 and 32*32.

Rate-distortion Performance

Figure 5 shows the rate distortion performance (i.e., Rate-DISTS and Rate-LPIPS) of the VVC codec, three different GFVC methods in the AHG16 software [17] and our proposed SRLR-based algorithm. It is clear that, in terms of DISTS, the proposed method clearly have better rate distortion performance compared with VVC at relatively high bitrate ranges (10~50 kbps). As for the LPIPS, the proposed algorithm also can achieve performance gains at middle bitrate ranges (10~30 kbps) compared with VVC. For rate distortion plots for individual sequences in terms of DISTS and LPIPS, readers are referred to the attached xls file.

(a) Overall – DISTS                            (b) Overall -LPIPS

(c) Class A – DISTS                            (d) Class A -LPIPS

(e) Class B – DISTS                            (f) Class B -LPIPS

Figure 5 – Rate distortion performance of VVC, GFVC and our porposed SRLR-based algorithm measured by two perceptual quality metrics DISTS and LPIPS in terms of overall dataset, Class A and Class B.

Figure 6 shows the rate distortion performance (i.e., Rate-PSNR and Rate-SSIM) of the VVC codec, three different GFVC methods in the AHG16 software [17] and our proposed SRLR-based algorithm. It is clear that, the proposed method can improve the fidelity of signal reconstruction. Though the overall RD performance is still not as good as VVC, this is expected as all generative methods do not optimize for sample-level fidelity. For rate distortion plots for individual sequences in terms of PSNR and SSIM, readers are referred to the attached xls file.

(a) PSNR-RGB444                            (b) SSIM-RGB444

(c) PSNR-YUV420                           (d) SSIM-YUV420

Figure 6 – Rate distortion performance of VVC, GFVC in AHG16 software and our porposed SRLR-based algorithm measured by two pixel-level quality metrics PSNR and SSIM in RGB444 and YUV420 domains for overall dataset.

BD-Rate Savings

As shown in Table 1, in comparison with the latest VVC codec at high bitrate ranges, the proposed SRLR-based algorithm can achieve 32.91%, 31.43% and 27.14% average bit-rate savings in terms of Rate-DISTS for FOMM, CFTE and FV2V, respectively. As for Rate-LPIPS, nearly half of 30 sequences have performance gains, but on average, there are no gains. It may be attributed that these testing models are optimized by DISTS loss function rather than LPIPS loss function, thereby the testing results are more inclined towards DISTS meaurement.

Table 1 - Average bit-rate savings for SRLR-based algorithm compared with the traditional VVC codec evaluated in RGB 444 domain.

| Dataset |  | SRLR-FOMM |  | SRLR-CFTE |  | SRLR-FV2V |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Rate-DISTS | Rate-LPIPS | Rate-DISTS | Rate-LPIPS | Rate-DISTS | Rate-LPIPS |
| Class A (VoxCeleb) | Seq001 | -72.97% | -42.20% | -68.42% | -36.19% | -71.89% | -43.08% |
|  | Seq002 | -68.83% | -46.14% | -67.53% | -44.03% | -72.44% | -47.35% |
|  | Seq003 | -59.91% | -22.60% | -61.15% | -26.76% | -56.90% | -22.56% |
|  | Seq004 | -54.91% | -12.27% | -51.57% | -24.76% | -41.55% | -18.89% |
|  | Seq005 | -53.63% | -14.19% | -49.49% | -18.75% | -53.52% | -13.37% |
|  | Seq006 | -40.19% | 17.89% | -46.44% | 14.54% | -37.81% | 12.74% |
|  | Seq007 | -19.24% | 126.12% | -13.00% | 113.63% | 1.71% | 89.89% |
|  | Seq008 | 16.72% | 81.74% | -4.13% | 44.35% | -1.91% | 40.30% |
|  | Seq009 | -59.85% | -7.57% | -57.92% | 1.10% | -54.79% | 1.62% |
|  | Seq010 | -75.92% | -37.09% | -80.10% | -42.49% | -73.75% | -40.82% |
|  | Seq011 | -65.74% | -44.53% | -65.27% | -40.31% | -62.07% | -35.71% |
|  | Seq012 | -72.36% | -30.71% | -76.14% | -32.01% | -70.88% | -34.50% |
|  | Seq013 | -57.99% | -26.31% | -55.87% | -28.60% | -59.16% | -27.70% |
|  | Seq014 | 28.79% | 149.38% | 21.60% | 96.28% | 33.82% | 145.34% |
|  | Seq015 | -34.83% | 19.05% | -41.33% | 2.82% | -21.72% | 11.51% |
| Class B (CFVQA) | Seq001 | -29.28% | 64.85% | -26.63% | 99.50% | -37.68% | 72.46% |
|  | Seq002 | -64.68% | 56.65% | -67.44% | 48.70% | 0.00% | 45.91% |
|  | Seq003 | -28.77% | 55.46% | -16.79% | 66.59% | -21.36% | 69.50% |
|  | Seq004 | -65.10% | -3.92% | -69.03% | 3.37% | 0.00% | -2.27% |
|  | Seq005 | -19.68% | -2.80% | -1.00% | 13.75% | -23.39% | 16.95% |
|  | Seq006 | -41.05% | 34.68% | -38.23% | 34.10% | -50.24% | 35.76% |
|  | Seq007 | -48.61% | 2.99% | -44.52% | -3.00% | -31.86% | 13.07% |
|  | Seq008 | -2.32% | 104.69% | -1.27% | 106.05% | 8.31% | 97.25% |
|  | Seq009 | 42.00% | 126.53% | 93.20% | 145.79% | 28.48% | 98.99% |
|  | Seq010 | 0.00% | -35.97% | 0.00% | -19.17% | 0.00% | -21.50% |
|  | Seq011 | -38.77% | -4.05% | -47.00% | -0.44% | -44.89% | 6.06% |
|  | Seq012 | -0.22% | 36.06% | -7.34% | 51.98% | 1.16% | 48.99% |
|  | Seq013 | 0.00% | -29.72% | 0.00% | -26.03% | 0.00% | -32.22% |
|  | Seq014 | 0.00% | -28.68% | 0.00% | -28.20% | 0.00% | -28.08% |
|  | Seq015 | 0.00% | -25.82% | 0.00% | -27.89% | 0.00% | -24.80% |
| Average | Class A | -46.06% | 7.37% | -47.78% | -1.41% | -42.86% | 1.16% |
|  | Class B | -19.77% | 23.40% | -15.07% | 31.01% | -11.43% | 26.40% |
|  | Class A+B | -32.91% | 15.38% | -31.43% | 14.80% | -27.14% | 13.78% |

Quailty Improvement Comparisons

Herein, visual examples are shown in Figure 7/8/9 to verify that the proposed SRLR architecture can reduce some annoying visual artifacts or provide more missing details caused by GFVC methods. Video demos using more test sequences will be shown during presentation at the meeting, and will emphasize very similar observations.

Figure 7 – Comparison of reconstruction quality using GFVC (FOMM) as base layer vs. SRLR with different granularities as enhancemet layer.

Figure 8 – Comparison of reconstruction quality using GFVC (FV2V) as base layer vs. SRLR with different granularities as enhancemet layer.

Figure 8 – Comparison of reconstruction quality using GFVC (CFTE) as base layer vs. SRLR with different granularities as enhancemet layer.

Subjective Quailty Comparisons

Reconstructed face images are shown in Figure 10 to provide comparison of reconstruction quality using VVC and a few SRLR-based schemes at similar bitrates. It can be seen that at high bitrate ranges, VVC reconstructed face images existing blurring distortions, whereas GFVC methods can still provide good face details. Figure 11 compares coded bitrates of different coding schemes when reconstructed quality is approximately the same. It can be seen that VVC needs almost more coding bits than the generative methods in order to deliver similar quality. Video demos using more test sequences will be shown during presentation at the meeting, and will emphasize very similar observations.

Figure 10 – Comparison of reconstruction quality using VVC vs. SRLR-based methods at similar coded bitrates. The bitrate and DISTS value are provided.

Figure 11 – Comparison of coded bitrates using VVC vs. SRLR-based methods at similar DISTS reconstruction quality. The bitrate and DISTS value are provided.

Conclusion

This contribution proposes a scalable representation and layered reconstruction (SRLR) algorithm for all existing GFVC approaches, which can provide a wider coverage of bitrate ranges for face video compression. It is suggested to consider the proposed SRLR architecture in AHG16 software.

References

- Goodfellow I, Pouget-Abadie J, Mirza M, et al. “Generative adversarial nets,” Advances in neural information processing systems, vol. 27, 2014.

- Konuko G, Valenzise G, Lathuilière S. Ultra-low bitrate video conferencing using deep image animation[C]//ICASSP 2021-2021 IEEE International Conference on Acoustics, Speech and Signal Processing (ICASSP). IEEE, 2021: 4210-4214.

- Siarohin A, Lathuilière S, Tulyakov S, et al. First order motion model for image animation[J]. Advances in Neural Information Processing Systems, 2019, 32.

- Wang T C, Mallya A, Liu M Y. One-shot free-view neural talking-head synthesis for video conferencing[C]//Proceedings of the IEEE/CVF conference on computer vision and pattern recognition. 2021: 10039-10049.

- Chen B, Wang Z, Li B, et al. Beyond keypoint coding: Temporal evolution inference with compact feature representation for talking face video compression[C]//2022 Data Compression Conference (DCC). IEEE, 2022: 13-22.

- Chen B, Wang Z, Li B, et al. Interactive Face Video Coding: A Generative Compression Framework[J]. arXiv preprint arXiv:2302.09919, 2023.

- Konuko G, Lathuilière S. and Valenzise G., "A Hybrid Deep Animation Codec for Low-Bitrate Video Conferencing," 2022 IEEE International Conference on Image Processing (ICIP), Bordeaux, France, 2022, pp. 1-5.

- Chen B, Chen J, et al. “AHG9: Generative Face Video SEI Message,” The Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, doc. no. JVET-AC0088, January 2023.

- Chen B, Chen J, et al. “AHG9: Common SEI Message for Generative Face Video,” The Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, doc. no. JVET-AD0051, April 2023.

- McCarthy S, et al. “AHG9: Generative Face Video SEI message,” The Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, doc. no. JVET-AE0080, July 2023.

- Chen B, Chen J, et al. “AHG9: Common SEI Message of Generative Face Video,” The Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, doc. no. JVET-AE0083, July 2023.

- Chen B, Chen J, et al. “AHG9: Common Text for Generative Face Video SEI Message,” The Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, doc. no. JVET-AE0280, July 2023.

- Han Boon Teo, Jing Yuan Thong, Karlekar Jayashree, et al. “AHG9: On Face Motion Information for Generative Face Video,” The Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, doc. no. JVET- AF0146, October 2023.

- Chen B, Yin S, Chen J, et al. “A Study on Decoder Interoperability of Generative Face Video Compression,” The Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, doc. no. JVET- AF0048, October 2023.

- Chen B, Chen J, et al. “AHG9: Common text for proposed generative face video SEI message,” The Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, doc. no. JVET- AF0234, October 2023.

- Ye Y et al., “On VVC-assisted ultra-low rate generative face video coding,” MPEG ISO/IEC JTC 1/SC 29/WG 2 doc. no. m64987, October 2023.

- Chen B, Chen J, et al. “AHG16: Proposed Common Software Tools and Testing Conditions for Generative Face Video Compression,” The Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, doc. no. JVET- AG0042, January 2024.

- Yin S, Chen B, et al. “AHG16: Interoperability Study on Parameter Translator of Generative Face Video Coding,” The Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, doc. no. JVET- AG0048, January 2024.

- Yin S, Chen B, et al. “AHG16: Interoperability Study on Parameter Translator of Generative Face Video Coding,” The Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, doc. no. JVET- AG0048, January 2024.

- Hannuksela M. M., Cricri F, Zhang H. “AHG9: On the generative face video SEI message,” The Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, doc. no. JVET- AG0087, January 2024.

- Hannuksela M. M., Cricri F, Zhang H. “AHG9: Usage of the neural-network post-filter characteristics SEI message to define the generator NN of the generative face video SEI message,” The Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, doc. no. JVET- AG0088, January 2024.

- Zou R, Liao R-L, et al. “AHG16: Depthwise separable convolution for generative face video compression,” The Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, doc. no. JVET- AG0139, January 2024.

- McCarthy S, Yin P et al. “AHG16: Study text for common test conditions and evaluation procedures for generative face video coding (draft 1) ,” The Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, doc. no. JVET- AG0187, January 2024.

- Chen J, Chen B et al. “AHG9/AHG16: Common text for proposed generative face video SEI message  ,” The Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, doc. no. JVET- AG0203, January 2024.

- McCarthy S, Chen B et al. “Test conditions and evaluation procedures for generative face video coding ,” The Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, doc. no. JVET- AG2035, January 2024.

- Hénaff O J, Goris R L T, Simoncelli E P. Perceptual straightening of natural videos[J]. Nature neuroscience, 2019, 22(6): 984-991.

- Minnen D, Ballé J, Toderici G D. Joint autoregressive and hierarchical priors for learned image compression[J]. Advances in neural information processing systems, 2018, 31.

Patent rights declaration(s)

Alibaba and City University of Hong Kong may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).