| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 17–24 April 2024 | Document: JVET-AH0130-v1 |
| --- | --- |

| Title: | AHG8: RPR for machine consumption |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Christopher Hollmann Jacob Ström Per Wennersten Kenneth Andersson | Email: | {christopher.hollmann, jacob.strom, per.wennersten, kenneth.r.andersson} @ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

In this contribution the reference picture resampling tool present in the VVC standard has been tested in the context of machine consumption of coded video content. In a first test, entire sequences have been coded at lower resolution and the following results are reported:

- 1.25x resampling: SFU−HW: −6.8% | −8.6% [RA | LD], TVD: −4.2% | −5.6% [RA | LD]

- 1.5x resampling: SFU−HW: −8.1% | −9.2% [RA | LD], TVD: −6.6% | 4.3% [RA | LD]

- 2.0x resampling: SFU−HW: −4.0% | −2.0% [RA | LD], TVD: 1.3% | 19.4% [RA | LD]

In a second test, a keyframe analysis-based method has been tested for random access. This method works by limiting resampling to only be applied if the number of objects in a keyframes does not exceed a threshold and by switching between full resolution and 1.25x resampling for 32-frame segments based on the share of objects detected in both the full resolution and a downscaled version of the keyframe. The method is reported to give a benefit of −9.1% for SFU-HW and −4.7% for TVD.

It is proposed to include a section on RPR in the next version of JVET-AG2030.

# Introduction

JVET has been working on a technical report regarding optimizations for encoders and receiving systems for machine analysis of coded video content. The latest version of this report, which is also registered as ISO/IEC TR 23888-3, was made available after the January 2024 JVET meeting in JVET-AG2030 [1].

VVC has a feature referred to as reference picture resampling (RPR), which allows coding frames at lower resolution. In this contribution, this feature is utilized as a tool to improve the coding efficiency for machine consumption of the decoded video.

It is proposed to include a section on RPR in the next version of ISO/IEC TR 23888-3. The exact proposed text can be found in the attached version with tracked changes.

# Reduced resolution coding using RPR

In the VVC reference software there exist a RPR functionality testing feature that can be invoked by setting the configuration parameter RPRFunctionalityTesting to 1. This allows defining which resolution and quantization parameter is to be used for encoding pictures in reduced resolution. This is controlled by setting the RPRSwitchingSegmentSize parameter to the length of segments with a pre-defined resolution and listing which resolution to use for each segment in the argument of the configuration parameter RPRSwitchingResolutionOrderList. If the same reduced resolution is desired for the entire sequence to be encoded, two options are to either set RPRSwitchingSegmentSize  to the length of the sequence or to list only identical rescaling factors in the RPRSwitchingResolutionOrderList. We tested the three default rescaling factors in VTM, i.e., 1.25x, 1.5x and 2.0x. As in the default settings of the RPR functionality testing, we set the QP offset to -2 for factor 1.25x, -4 for factor 1.5x and -6 for factor 2.0x. The QP offsets are set with the parameter RPRSwitchingQPOffsetOrderList.

The results we obtained are summarized in Table 1 for both random access and low delay, using the common test conditions (CTC) defined in JVET-AF2031 [2]. We used VTM-22.0 for our tests. The reconstruction produced by the encoder and decoder is upsampled to the original resolution using the default upscaling filter in VTM by setting the encoder and decoder parameter UpscaledOutput to 2 to ensure that the machine consumption using the object detection or object tracking network uses the same resolution as the anchor results.

To determine the results we utilized BD-rate based on bit rate and task performance. From these results it can be seen that coding in reduced resolution can give a benefit in terms of BD-rate. One exception here is the Traffic sequence (class A in the SFU-HW dataset), where a significant loss in random access can be observed. In general, 1.5x performs slightly better than 1.25x (except for TVD low delay).

Table 1. Test results for reduced resolution coding

| Dataset |  | Random access |  |  | Low delay |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1.25x | 1.5x | 2.0x | 1.25x | 1.5x | 2.0x |
| SFU-HW | Class A | 34.97% | 37.93% | 24.84% | −17.93% | −15.59% | 9.31% |
|  | Class B | −22.02% | −16.77% | −18.53% | −6.70% | −8.35% | −16.48% |
|  | Class C | −1.13% | −9.06% | −9.93% | −10.42% | −10.84% | 0.26% |
|  | Class D | −7.75% | −9.86% | 9.35% | −6.16% | −6.74% | 7.31% |
|  | Average | −6.81% | −8.07% | −3.97% | −8.55% | −9.17% | −2.03% |
| TVD | Average | −4.23% | −6.59% | 1.26% | −5.62% | 4.28% | 19.44% |

# Keyframe analysis-based resolution switching

One aspect of RPR is that it can adjust the resolution of coded frames for pre-defined segments of a sequence, i.e., encoding a segment of a sequence at one resolution before encoding the following segment at a different resolution. In our proposed method we utilize a keyframe analysis to determine which resolution to use for a segment. An overview of the method is shown in Figure 1.

Keyframe extraction

Analysis

Encoder

Input video

keyframes

Downscale

Analysis

Comparison, RPR decision

Figure 1. Overview of the pre-processing

## Detailed description

From the input video we extract keyframes in regular intervals. We select every 32nd frame, but this could be adjusted as a trade-off between amount of pre-processing and adaptability of the method. This means that the keyframe and the next 31 frames until the next keyframe are coded using the same resolution.

The keyframes are then fed into the analysis network in two variants, in full resolution and a reduced resolution. In our test we utilize the downscaling factor 1.25x in both horizontal and vertical direction for resizing the keyframes. For the analysis network we utilize the YOLOv3 object detection network [3]. This will result in two outputs, one for full resolution and one for low resolution. The output consists of a list of objects described by their bounding boxes that were found in the keyframes.

We apply an object threshold of 20, indicating that if at least 20 objects are found in the full resolution keyframe, no downscaling is applied. The same is done if there are no objects in the full resolution keyframe.

The two outputs of the analysis are then compared and for each object in the full resolution it is checked whether there is a corresponding object of the same class and a sufficiently large intersection over union (IoU) in the downscaled resolution. In our test we set the threshold to 0.75, i.e., only objects where a corresponding object in the low resolution with at least 0.75 IoU are marked as found. If more than 75% of all objects in the full resolution are found in the reduced resolution, the current segment is coded with downscaling factor 1.25x. If at most 75% of objects are found in the reduced resolution, the segment is encoded at full resolution.

In the encoder we use the RPR functionality testing feature to encode different parts of each sequence at different resolutions. To counteract the quality loss by downscaling, we decrease the QP by 2 for segments coded using downscaling factor 1.25x. This offset is commonly used in RPR tests.

To summarize, a segment can be coded in two different resolutions:

- 1.25x downscaled: the keyframe has at least one and less than 20 objects and more than 75% of objects were found in the downscaled version, or

- 1x (original resolution): the keyframe has at least 20 objects, zero objects, or at most 75% of objects were found in the downscaled version

## Results

We tested our method according to the CTC [2] based on VTM-22.0. Detailed per-sequence results can be found in the attached Excel document. A summary is shown in Table 2. Additional results are included for the case where the object threshold is not applied, i.e., if at least 75% of objects are found in the downscaled version 1.25x is used and otherwise full resolution. This affects only two sequences, Traffic and BQTerrace. The results for all other sequences are identical in both cases. In addition, results for coding the entire sequence at downscaling factor 1.25x are included as reference.

This method has primarily been tested under random access conditions. It shows benefit for the SFU-HW dataset, outperforming always downscaling with factor 1.25x. For TVD, results are mostly identical to always using 1.25x, with the difference being TVD-01_03 which has an improved performance. It likely shows a positive effect on low delay as well, but it is possible that other thresholds should be used.

Table 2. Test results for the proposed method in random access

| Dataset |  | With object threshold | Without object threshold | Reference: coding at 1.25x |
| --- | --- | --- | --- | --- |
| SFU-HW | Class A | 0.00% | 34.97% | 34.97% |
|  | Class B | −22.00% | −22.02% | −22.02% |
|  | Class C | −1.47% | −1.47% | −1.13% |
|  | Class D | −6.15% | −6.15% | −7.75% |
|  | Average | −9.11% | −6.43% | −6.81% |
| TVD | Average | −4.75% | −4.75% | −4.23% |

# Conclusion

In this contribution we have studied the effects of reference picture resampling and coding in reduced resolution on machine consumption performance. We have shown that a keyframe analysis-based method can provide an additional benefit over always coding at lower resolutions. It is proposed to add a description of RPR into the technical report on optimizations for encoders and receiving systems for machine analysis of coded video content according to the attached document.

Depending on the interest of the group, we would also be willing to share the code for our keyframe analysis method on the jvet-ahg-ofm Gitlab.

# Patent rights declaration

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

| [1] | C. Hollmann, J. Chen and S. Liu, "Optimization of encoders and receiving systems for machine analysis of coded video content (draft 4)," JVET-AG2030, Online, January 2024. |
| --- | --- |
| [2] | S. Liu and C. Hollmann, "Common test conditions for optimization of encoders and receiving systems for machine analysis of coded video content," JVET-AF2031, Hannover, DE, October 2023. |
| [3] | J. Redmon and A. Farhadi, "YOLOv3: An Incremental Improvement," arXiv, 2018. |