| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 17–24 April 2024 | Document: JVET-AH0132-v2 |
| --- | --- |

| Title: | AHG9: On use of PON SEI messages |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Lulin Chen Olena Chubach Yu-Wen Huang Shawmin Lei | Email: | {lulin.chen, olena.chubach, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek, Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to add a constraint when the same SEI message payload type and processing order of a nested SEI message are indicated in more than one PON SEI messages for a particular value of po_id.

# Introduction

In JVET-AG2027, the updated semantics for the flag of po_sei_wrapping_flag[ i ] is as follows.

po_sei_wrapping_flag[ i ] equal to 1 specifies that one or more processing order nesting SEI messages with both of the following constraints should be present:

–	pon_target_po_id[ j ] with any value of j is equal to po_id.

–	There is a k-th loop entry in the processing order nesting SEI message such that the payloadType of the k-th nested SEI message is equal to po_sei_payload_type[ i ] and pon_processing_order[ k ] is equal to po_sei_processing_order[ i ].

When po_sei_wrapping_flag[ i ] is equal to 0, an SEI message with payloadType equal to po_sei_payload_type[ i ] (and, when po_sei_prefix_flag[ i ] equal to 1, prefix data that matches the values of po_sei_prefix_data_bit[ i ][ j ]) should be present outside of the processing order nesting SEI message.

NOTE 2 – po_sei_wrapping_flag[ i ] equal to 1 enables SEI messages to be carried within the processing order nesting SEI message to prevent such SEI messages from being incorrectly interpreted by decoders that do not process the SPO SEI message. Thus, po_sei_wrapping_flag[ i ] equal to 1 is intended to be used when po_sei_wrapping_flag[ i ] equal to 0 can lead to unintended results being produced by such decoders.

# Proposal

There are cases, when po_sei_wrapping_flag[ i ] equal to 1, the same payload type and processing order of a nested SEI message may be indicated in more than one PON SEI messages, e.g., there may be one or more NNPF characteristic SEI messages with update on an NNPF characteristic SEI message nested in a PON SEI message and the NNPF characteristic SEI messages with update are also nested SEI messages that may be nested in one or more PON SEI messages.

This contribution proposes to add a constraint that when the same SEI message payload type and preferred processing order are indicated in more than one PON SEI messages, the last one of those PON SEI messages is used. The proposed text as below highlighted in green.

po_sei_wrapping_flag[ i ] equal to 1 specifies that one or more processing order nesting SEI messages with all both of the following constraints should be present:

–	pon_target_po_id[ j ] with any value of j is equal to po_id.

–	There is a k-th loop entry in the processing order nesting SEI message such that the payloadType of the k-th nested SEI message is equal to po_sei_payload_type[ i ] and pon_processing_order[ k ] is equal to po_sei_processing_order[ i ].

–	For any value of k, when  payloadType of a k-th nested SEI message equal to po_sei_payload_type[ i ] and pon_processing_order[ k ] equal to po_sei_processing_order[ i ] are indicated in more than one PON SEI messages for a particular value of po_id, the last PON SEI message that contains the k-th loop entry for that particular value of po_id is applied to the processing chain.

…

# References

- “SEI processing order and processing order nesting SEI messages in VVC (draft 7)”, JVET-AG2027, JVET 33rd (AG) meeting, January 2024, online.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).