| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes,FR, 17–24 April 2024 | Document: JVET-AH0141-v3 |
| --- | --- |

| Title: | AHG12: Sharp motion compensation filter for bi-prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jonatan Samuelsson-Allendes | Tel: Email: | +46721882169 samuelssonj@sharplabs.com |
| Source: | Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

This input contribution relates to motion compensation (MC) filters in ECM and proposes an additional luma filter to be used for bi-predicted blocks, while the existing MC filter is still used for uni-predicted blocks. No changes are proposed to chroma filters, affine filters or RPR filters. The proposed filter has slightly sharper filter properties compared to the existing MC filters, and it is asserted that this sharpness helps compensate for the additional filtering that occurs during bi-prediction. The proposed change gives an average BD-Rate(PSNR-Y/U/V) impact of -0.07%/-0.03%/-0.05% for RA and -0.07%/-0.21%/-0.39% for LDB. There is no impact on LDP since uni-prediction is not changed. No impact on encoding time and decoding time is expected. In the second version of this document, results for BQTerrace LDB QP 22 were added. The third version of the contribution adds a presentation slide deck.

# Problem Statement

Motion compensation (MC) filtering in video coding standards is generally realised with fixed Finite Impulse Response (FIR) filters and in ECM, the motion compensation filter for luma is a 12-tap filter with 8-bit filter coefficients and 16 phases and looks as follows:

ecmFilter = {

{   0,   0,   0,   0,   0, 256,   0,   0,   0,   0,   0,   0},

{  -1,   2,  -3,   6, -14, 254,  16,  -7,   4,  -2,   1,   0},

{  -1,   3,  -7,  12, -26, 249,  35, -15,   8,  -4,   2,   0},

{  -2,   5,  -9,  17, -36, 241,  54, -22,  12,  -6,   3,  -1},

{  -2,   5, -11,  21, -43, 230,  75, -29,  15,  -8,   4,  -1},

{  -2,   6, -13,  24, -48, 216,  97, -36,  19, -10,   4,  -1},

{  -2,   7, -14,  25, -51, 200, 119, -42,  22, -12,   5,  -1},

{  -2,   7, -14,  26, -51, 181, 140, -46,  24, -13,   6,  -2},

{  -2,   6, -13,  25, -50, 162, 162, -50,  25, -13,   6,  -2},

{  -2,   6, -13,  24, -46, 140, 181, -51,  26, -14,   7,  -2},

{  -1,   5, -12,  22, -42, 119, 200, -51,  25, -14,   7,  -2},

{  -1,   4, -10,  19, -36,  97, 216, -48,  24, -13,   6,  -2},

{  -1,   4,  -8,  15, -29,  75, 230, -43,  21, -11,   5,  -2},

{  -1,   3,  -6,  12, -22,  54, 241, -36,  17,  -9,   5,  -2},

{   0,   2,  -4,   8, -15,  35, 249, -26,  12,  -7,   3,  -1},
{   0,   1,  -2,   4,  -7,  16, 254, -14,   6,  -3,   2,  -1}}

The same filter is used for both uni-prediction and bi-prediction. Other filters are used for chroma, affine, and RPR.

When a block is predicted using bi-prediction, two different reference blocks will be blended together to form the final prediction of the block. One or both of these reference blocks might be located at a non-integer position, and thereby being the result of a filtering operation. Our hypothesis has been that the blending and the filtering can result in that the predicted block is smoother than would be ideal and that a slight level of sharpening would be beneficial to improve the prediction. However, rather than introducing a separate sharpening filter stage in the prediction loop, we propose to use a different MC-filter for bi-predicted blocks compared to uni-predicted blocks.

# Proposal

We propose to introduce an additional MC filter to be used for bi-predicted blocks, while the existing MC filter is still used for uni-predicted blocks. No changes are proposed to chroma filters, affine filters or RPR filters.

The proposed filter is specified as follows:

proposedBiPredFilter = {

{   0,   0,   0,   0,   0, 256,   0,   0,   0,   0,   0,   0},

{  -1,   3,  -5,   8, -16, 256,  15,  -6,   3,  -1,   0,   0},

{  -2,   5,  -9,  15, -30, 253,  32, -12,   5,  -2,   1,   0},

{  -3,   7, -12,  21, -40, 244,  50, -18,   9,  -4,   2,   0},

{  -3,   8, -15,  25, -48, 235,  71, -25,  12,  -6,   2,   0},

{  -3,   9, -16,  28, -53, 220,  93, -32,  16,  -8,   3,  -1},

{  -3,   8, -16,  28, -54, 204, 116, -39,  19, -10,   4,  -1},

{  -2,   8, -15,  27, -53, 183, 139, -45,  22, -12,   5,  -1},

{  -2,   6, -13,  25, -50, 162, 162, -50,  25, -13,   6,  -2},

{  -1,   5, -12,  22, -45, 139, 183, -53,  27, -15,   8,  -2},

{  -1,   4, -10,  19, -39, 116, 204, -54,  28, -16,   8,  -3},

{  -1,   3,  -8,  16, -32,  93, 220, -53,  28, -16,   9,  -3},

{   0,   2,  -6,  12, -25,  71, 235, -48,  25, -15,   8,  -3},

{   0,   2,  -4,   9, -18,  50, 244, -40,  21, -12,   7,  -3},

{   0,   1,  -2,   5, -12,  32, 253, -30,  15,  -9,   5,  -2},

{   0,   0,  -1,   3,  -6,  15, 256, -16,   8,  -5,   3,  -1},

Just like the current ECM MC filters, the proposed filter is based on a cos-windowed sinc function. However, a more generalized form has been used, and unlike the current ECM MC filter, a and b have been selected to be slightly lower than 1 (instead of equal to 1). The formula is shown in Equation (1), where A is a scaling factor to provide the desired number of filter taps.

fx=cosaAxsincxb								(1)

The normalized sinc function shown in Equation (2) is used.

sincx=   sinπxπ,       x≠01,                 x=0								(2)

Figure 1 illustrates the effect on the sinc function of using an a smaller than 1, and figure 2 illustrates the effect on the cos-window of using a b smaller than 1. Figure 3 illustrates the combined impact of using a and b both slightly lower than 1.

Figure 1. An illustration of the effect of a scaled sinc function as in the proposal.

Figure 2. An illustration of the effect of a modified cos window as in the proposal.

Figure 3. An illustration of the effect of a modified cos-windowed sinc function as in the proposal.

It can be noted that no filtering is introduced for the 0th phase (integer position motion vectors) and that the 8th phase (half-pel position) filter is the same as the current ECM MC filter. It can also be noted that the difference between the proposed filter and the current ECM MC filter is not larger than 5 for any coefficient, in absolute terms, as shown below:

ecmFilter - proposedBiPredFilter = {

{   0,   0,   0,   0,   0,   0,   0,   0,   0,   0,   0,   0},
{   0,  -1,   2,  -2,   2,  -2,   1,  -1,   1,  -1,   1,   0},
{   1,  -2,   2,  -3,   4,  -4,   3,  -3,   3,  -2,   1,   0},
{   1,  -2,   3,  -4,   4,  -3,   4,  -4,   3,  -2,   1,  -1},
{   1,  -3,   4,  -4,   5,  -5,   4,  -4,   3,  -2,   2,  -1},
{   1,  -3,   3,  -4,   5,  -4,   4,  -4,   3,  -2,   1,   0},
{   1,  -1,   2,  -3,   3,  -4,   3,  -3,   3,  -2,   1,   0},
{   0,  -1,   1,  -1,   2,  -2,   1,  -1,   2,  -1,   1,  -1},
{   0,   0,   0,   0,   0,   0,   0,   0,   0,   0,   0,   0},
{  -1,   1,  -1,   2,  -1,   1,  -2,   2,  -1,   1,  -1,   0},
{   0,   1,  -2,   3,  -3,   3,  -4,   3,  -3,   2,  -1,   1},
{   0,   1,  -2,   3,  -4,   4,  -4,   5,  -4,   3,  -3,   1},
{  -1,   2,  -2,   3,  -4,   4,  -5,   5,  -4,   4,  -3,   1},
{  -1,   1,  -2,   3,  -4,   4,  -3,   4,  -4,   3,  -2,   1},
{   0,   1,  -2,   3,  -3,   3,  -4,   4,  -3,   2,  -2,   1},
{   0,   1,  -1,   1,  -1,   1,  -2,   2,  -2,   2,  -1,   0},

# Results

The proposed filter has been tested in ECM-12 using the Common Test Conditions described in JVET-AF2017.

|  | Random Access |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-12.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 | -0.05% | 0.00% | -0.12% | 101.6% | 101.4% | 100.0% |
| Class A2 | 0.01% | 0.00% | -0.12% | 98.1% | 98.6% | 99.7% |
| Class B | -0.07% | -0.03% | 0.03% | 99.2% | 99.7% | 100.0% |
| Class C | -0.15% | -0.07% | -0.03% | 99.9% | 100.2% | 100.0% |
| Class E |  |  |  |  |  |  |
| Overall | -0.07% | -0.03% | -0.05% | 99.7% | 99.9% | 99.9% |
| Class D | -0.35% | -0.41% | -0.28% | 101.0% | 102.2% | 99.9% |
| Class F | -0.05% | -0.03% | -0.17% | 99.0% | 99.6% | 99.4% |
| Class TGM | -0.05% | -0.02% | -0.07% | 99.3% | 99.4% | 99.7% |

Table 1. Results for Random Access Configuration

|  | Low delay B |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-12.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | -0.08% | -0.16% | -0.34% | 101.2% | 100.5% | 100.0% |
| Class C | -0.11% | -0.30% | -0.12% | 100.0% | 98.9% | 98.9% |
| Class E | -0.02% | -0.17% | -0.83% | 96.2% | 97.8% | 99.9% |
| Overall | -0.07% | -0.21% | -0.39% | 99.6% | 99.3% | 99.6% |
| Class D | -0.06% | -0.27% | 0.29% | 101.7% | 100.9% | 100.6% |
| Class F | 0.11% | -0.42% | -0.24% | 95.9% | 96.9% | 98.1% |
| Class TGM | -0.09% | -0.20% | -0.06% | 96.9% | 97.6% | 101.3% |

Table 2. Results for Low Delay B configuration

# Discussion

One question that is relevant to ask is, if the proposed sharper filter can be beneficial also for uni-prediction. We have performed experiments applying the proposed filter also for uni-prediction and it turns out that the gain is lower than applying it only to bi-prediction and furthermore there is a clear loss in performance in LDP. Thus, we have concluded that applying the sharp filter for both uni-prediction and bi-prediction is not a viable option.

# Conclusion

A new, sharper MC filter is proposed to be used for bi-prediction in ECM. The proposed change gives an average BD-Rate(PSNR-Y/U/V) impact of -0.07%/-0.03%/-0.05% for RA and -0.07%/-0.21%/-0.39% for LDB. There is no impact on LDP since uni-prediction is not changed.

It is suggested to include the proposed technology in the next round of Exploratory Experiments for ECM.

# Patent rights declaration(s)

Sharp Labs of America, Inc., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Acknowledgement
These research results were obtained from the commissioned research (No.05101) by National Institute of Information and Communications Technology (NICT), Japan.

# Annex A. Filter response.

The filter response for the proposed filter is shown below.