| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 17–24 April 2024 | Document: JVET-AH0153-v2 |
| --- | --- |

| Title: | EE2-6.3 related: Combination of EE2-6.1 and EE2-6.2 on entropy coding extension – retraining |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | F. Galpin, F. Lo Bianco C. Salmon-Legagneur M. Balcilar Ru-Ling Liao Yan Ye Jie Chen Xinwei Li Pavel Nikitin Marta Karczewicz | Tel: Email: | franck.galpin@interdigital.com ruling.lrl@alibaba-inc.com pnikitin@qti.qualcomm.com |
| Source: | InterDigital Inc., Alibaba group, Qualcomm |  |  |

_____________________________

# Abstract

In this contribution, compared to EE2-6.3, a more reliable training using CTC is performed. Indeed, by introducing a new model for low delay slices (L-slice) which was not present before, the training between ECM version should be adapted. The B-slice and L-slice should use only the RA and LDB/LDP sequences respectively in order to emulate the presence of an L-slice model in the model. Furthermore, two fixes have been performed related to the CABAC models.

The full retraining shows the following results:

|  | Y-BDR | U-BDR | V-BDR |
| --- | --- | --- | --- |
| All Intra | -0.04% | -0.08% | -0.01% |
| Random Access | xxx |  |  |
| Low Delay B |  |  |  |

# Introduction

In the EE2 test 6.1, a new type of model (hereinafter referred to as L model) for B slices is proposed to differentiate between low-delay B and non-low delay B slices. Each B slice initializes its context model according to the low delay condition. The L model is used for low delay B slices, whereas the B model is used for non-low delay B slices.

In the EE2 test 6.2, a new entropy model called NEP (non EP) between EP (equiprobable) and CABAC (contextual, adaptive, arithmetic coding) is proposed. Furthermore, previously EP coded bins are given an associated context and can therefore uses the new NEP model.

In this contribution, the combination of both aspects was fully retrained on ECM-12.0. The CABAC/NEP/EP models are taken as follow:

- B type model uses RA only sequences (since LDx sequences now have their own L model)

- P/I model is taken from EE2-6.3: it uses RA/AI/LDB/LDP sequences as before.

- L model is retrained from B-type slice in LDB/LDP only sequences.

The criteria to use the L-model is changed so that only low-delay slice (all references in the past AND only 1 POC of difference with the last decoded POC) is used.

Furthermore, a fix has been applied for the residual coding process affecting the I-slice. Only the gtX parameters for the I model have been updated compared to 6.3.

The code reflecting the changes has been uploaded in the branch 6.3 of the EE1 repository.

# Simulations

In both tests, full encoding in AI, RA, LDB and LDP configurations using the 4 CTCs QPs is used.

All the CTC sequences including F and TGM are used.

## Test 6.3 related – using CTC sequences

Note: encoding/decoding were done on heterogeneous cluster. Timing is assumed to be the same as 6.3 (no complexity added) since only entropy model parameters are changing.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.04% | 0.02% | -0.06% | 106.1% | 102.0% | 99.3% | 101.2% |
| Class A2 | -0.08% | -0.10% | -0.06% | 109.2% | 104.5% | 100.8% | 101.9% |
| Class B | -0.04% | -0.09% | -0.02% | 103.6% | 103.8% | 98.9% | 100.6% |
| Class C | -0.02% | -0.13% | 0.04% | 104.5% | 104.4% | 99.0% | 100.2% |
| Class E | -0.02% | -0.07% | 0.01% | 103.4% | 104.8% | 102.0% | 100.8% |
| Overall | -0.04% | -0.08% | -0.01% | 105.1% | 103.9% | 99.8% | 100.8% |
| Class D | -0.08% | 0.04% | -0.25% | 101.0% | 99.3% | 100.0% | 100.1% |
| Class F | -0.09% | -0.03% | -0.07% | 102.7% | 102.1% | 102.4% | 100.4% |
| Class TGM | -0.29% | -0.24% | -0.33% | 104.9% | 102.9% | 97.4% | 100.8% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.19% | 0.17% | -0.01% | 105.5% | 105.7% | 100.1% | 99.8% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class B | -0.25% | 0.01% | -0.39% | 103.6% | 101.4% | 100.1% | 100.0% |
| Class C | -0.15% | -0.17% | -0.06% | 101.3% | 101.7% | 99.6% | 99.8% |
| Class E |  |  |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class D | -0.25% | -0.38% | -0.27% | 98.5% | 99.1% | 100.2% | 100.1% |
| Class F | -0.18% | -0.41% | -0.15% | 100.6% | 99.8% | 100.1% | 100.2% |
| Class TGM | -0.97% | -0.99% | -0.94% | 102.9% | 107.5% | 99.5% | 100.1% |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class C | -0.10% | -0.30% | -0.58% | 98.7% | 99.5% | 100.0% | 100.7% |
| Class E | -0.13% | 0.50% | -0.83% | 98.5% | 99.6% | 99.9% | 99.8% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class D | -0.30% | 0.11% | -0.67% | 93.7% | 97.3% | 100.2% | 99.9% |
| Class F | -0.03% | -0.07% | -0.10% | 98.7% | 98.5% | 100.4% | 101.4% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
|  | Low delay P Main 10 |  |  |  |  |  |  |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.07% | -0.56% | -0.10% | 103.8% | 101.2% | 100.0% | 100.0% |
| Class C | -0.08% | -0.42% | -0.59% | 101.1% | 100.8% | 100.1% | 100.1% |
| Class E | -0.05% | 0.50% | -1.15% | 100.6% | 104.9% | 95.9% | 100.1% |
| Overall | -0.07% | -0.25% | -0.53% | 102.1% | 102.0% | 99.0% | 100.1% |
| Class D | -0.31% | -2.05% | -0.96% | 99.6% | 100.5% | 99.6% | 100.0% |
| Class F | -0.28% | -0.38% | 0.16% | 100.0% | 98.0% | 101.1% | 100.2% |
| Class TGM | -0.99% | -1.05% | -1.13% | 106.0% | 101.4% | 100.2% | 100.0% |

## Additional test – without the I-slice correction

An additional test similar to the one above was performed to show the results without the correction of the residual coding.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.01% | 0.00% | 0.02% |  |  | 100.2% | 100.6% |
| Class A2 | -0.04% | -0.05% | -0.06% |  |  | 100.3% | 101.4% |
| Class B | -0.03% | -0.03% | 0.08% |  |  | 96.8% | 101.0% |
| Class C | -0.03% | -0.17% | -0.10% |  |  | 100.5% | 100.2% |
| Class E | -0.03% | -0.04% | -0.03% |  |  | 99.8% | 100.6% |
| Overall | -0.03% | -0.06% | -0.01% |  |  | 99.3% | 100.8% |
| Class D | -0.07% | 0.05% | -0.40% |  |  | 100.0% | 100.1% |
| Class F | -0.09% | -0.19% | -0.20% |  |  | 101.1% | 100.3% |
| Class TGM | -0.26% | -0.16% | -0.32% |  |  | 99.7% | 100.5% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.18% | -0.03% | -0.02% |  |  | 100.2% | 99.7% |
| Class A2 | -0.09% | -0.13% | -0.28% |  |  | 99.9% | 100.1% |
| Class B | -0.24% | -0.12% | -0.52% |  |  | 99.9% | 100.1% |
| Class C | -0.14% | -0.21% | 0.05% |  |  | 100.0% | 99.8% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.17% | -0.13% | -0.22% |  |  | 100.0% | 100.0% |
| Class D | -0.27% | -0.30% | -0.23% |  |  | 100.3% | 100.0% |
| Class F | -0.17% | -0.46% | -0.35% |  |  | 99.9% | 100.2% |
| Class TGM | -0.93% | -0.96% | -0.81% |  |  | 99.9% | 100.3% |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! |  |  | #NUM! | #NUM! |
| Class C | -0.14% | -0.38% | -0.22% |  |  | 100.0% | 100.3% |
| Class E | -0.29% | 0.81% | -0.02% |  |  | 100.3% | 99.6% |
| Overall | #VALUE! | #VALUE! | #VALUE! |  |  | #NUM! | #NUM! |
| Class D | -0.25% | -0.06% | -1.35% |  |  | 100.1% | 100.0% |
| Class F | #VALUE! | #VALUE! | #VALUE! |  |  | #NUM! | #NUM! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! |  |  | #NUM! | #NUM! |
|  | Low delay P Main 10 |  |  |  |  |  |  |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.05% | -0.58% | -0.12% |  |  | 100.0% | 99.9% |
| Class C | -0.07% | -0.92% | -0.35% |  |  | 100.1% | 100.1% |
| Class E | -0.27% | -0.37% | -0.02% |  |  | 99.0% | 100.1% |
| Overall | -0.11% | -0.64% | -0.17% |  |  | 99.8% | 100.0% |
| Class D | -0.36% | -1.75% | -0.13% |  |  | 99.7% | 100.0% |
| Class F | -0.19% | -0.25% | 0.51% |  |  | 101.2% | 100.2% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! |  |  | #NUM! | #NUM! |

# Conclusion

This contribution is similar to EE2-6.3 but a full retraining has been performed to be consistent with the new L model and NEP model. It allows a more reliable result for future CABAC training. It is proposed to adopt the new parameters and fixes in the next version of ECM.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Alibaba may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).