| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes,FR, 17–24 April 2024 | Document: JVET-AH0160-v2 |
| --- | --- |

| Title: | AHG9: On persistence cancellation of an object mask information SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry Junghak Nam Seung Hwan Kim Jaehyun Lim | Email: | dr.hendry@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

It is asserted that the while the current design of object mask information (OMI) SEI message allows the presence of multiple OMI SEI messages to be active in the CVS, the persistence cancelling mechanism is only able to cancel the persistency of any previous OMI SEI messages. A better design would be to be able to precisely cancel persistence of a specific OMI SEI message, rather than blindly cancelling persistency of all previous OMI SEI messages.

This contribution proposes to enable OMI SEI message to cancel the persistency of a specific OMI SEI message with the following modifications:

- Move the signalling of omi_aux_id_minus128 to be before syntax element omi_cancel_flag.

- Change the coding of syntax element omi_aux_id_minus128 from ue(v) to become u(5).

- Update the semantics of omi_cancel_flag such that when it is equal to 1, it cancels previous OMI SEI message with the same omi_aux_id_minus128 value, if present.

In the first revision of this contribution, the coding of the syntax element omi_aux_id_minus128 is changed from ue(v) to become u(5).

# Problem description

The following snippet shows the syntax table and semantics of syntax element omi_cancel_flag and omi_aux_id_minus128 in the object mask information (OMI) SEI message.

--------------------- Start of snippet ---------------------

| object_mask_info( payloadSize ) { | Descriptor |
| --- | --- |
| omi_cancel_flag | u(1) |
| if( !omi_cancel_flag ) { |  |
| omi_aux_id_minus128 | ue(v) |
| … |  |
| } |  |
| } |  |

…

omi_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous object mask information SEI message in output order. omi_cancel_flag equal to 0 indicates that object mask information follows.

omi_aux_id_minus128 plus 128 indicates the value of sdi_aux_id of object mask auxiliary picture layer. om_aux_id_minus128 shall be in the range of 0 to 31, inclusive.

When a CVS does not contain an SDI SEI message with sdi_aux_id[ i ] equal to omi_aux_id_minus128 + 128 for at least one value of i, no picture in the CVS shall be associated with an OMI SEI message.

When an AU contains both an SDI SEI message with sdi_aux_id[ i ] equal to omi_aux_id_minus128 + 128 for at least one value of i and an OMI SEI message, the SDI SEI message shall precede the OMI SEI message in decoding order.

…

--------------------- End of snippet ---------------------

Based on authors’ understanding, the value of syntax element omi_aux_id_minus128 shall be equal to one of the value of sdi_aux_id[ i ] that is present in a scalability dimension information (SDI) SEI message. Multiple auxiliary pictures may be present in different layers so that it is possible there may be multiple sdi_aux_id[ i ] with values greater in between 128 and 159. Consequently, it is also possible to have multiple active OMI SEI messages in the CVS.

While it is possible to have multiple active OMI SEI messages in the CVS, the current design is asserted to have problem with design of cancelling the persistency of OMI SEI message. When omi_cancel_flag is equal to 1, currently it is specified that it cancel the persistency of any previous OMI SEI messages. In authors’ opinion, the better design would be to be able to precisely cancel persistence of a specific OMI SEI message, rather than blindly cancelling persistency of all previous OMI SEI messages.

# Proposal

To overcome the problem described above, we propose the following modifications:

- Move the signalling of omi_aux_id_minus128 to be before syntax element omi_cancel_flag.

- Change the coding of syntax element omi_aux_id_minus128 from ue(v) to become u(5). As the consequence of the above item, the position of omi_aux_id_minus128 moves up to become the first syntax element in the SEI, it is better to signal it as a fixed-length syntax element rather than variable length coding method like ue(v).

- Update the semantics of omi_cancel_flag such that when it is equal to 1, it cancels previous OMI SEI message with the same omi_aux_id_minus128 value, if present.

# Text implementation

The text implementation of the above proposed points are as shown below:

| object_mask_info( payloadSize ) { | Descriptor |
| --- | --- |
| omi_aux_id_minus128 | u(5) |
| omi_cancel_flag | u(1) |
| if( !omi_cancel_flag ) { |  |
| omi_aux_id_minus128 | ue(v) |
| … |  |
| } |  |
| } |  |

…

When an access unit contains an auxiliary picture picA in a layer, with nuh_layer_id equal to nuhLayerIdA, that is indicated as an object mask auxiliary layer by an OMI SEI message, and a primary picture picB in a layer, with nuh_layer_id equal to nuhLayerIdB, that is indicated as a primary layer by the OMI SEI message, OMI SEI message persists in output order until one or more of the following conditions are true:

–	The persistence of the OMI SEI message is cancelled (i.e., by the presence of an OMI SEI message with the same value of omi_aux_id_minus128 and omi_cancel_flag equal to 1).

–	A CLVS containing the auxiliary picture picA ends.

–	A CLVS containing the primary picture picB ends.

–	A CVS ends.

–	The bitstream ends.

omi_aux_id_minus128 plus 128 indicates the value of sdi_aux_id of object mask auxiliary picture layer. om_aux_id_minus128 shall be in the range of 0 to 31, inclusive.

When a CVS does not contain an SDI SEI message with sdi_aux_id[ i ] equal to omi_aux_id_minus128 + 128 for at least one value of i, no picture in the CVS shall be associated with an OMI SEI message.

When an AU contains both an SDI SEI message with sdi_aux_id[ i ] equal to omi_aux_id_minus128 + 128 for at least one value of i and an OMI SEI message, the SDI SEI message shall precede the OMI SEI message in decoding order.

omi_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous object mask information SEI message, if present, in output order with the same value of omi_aux_id_minus128. omi_cancel_flag equal to 0 indicates that object mask information follows.

omi_aux_id_minus128 plus 128 indicates the value of sdi_aux_id of object mask auxiliary picture layer. om_aux_id_minus128 shall be in the range of 0 to 31, inclusive.

When a CVS does not contain an SDI SEI message with sdi_aux_id[ i ] equal to omi_aux_id_minus128 + 128 for at least one value of i, no picture in the CVS shall be associated with an OMI SEI message.

When an AU contains both an SDI SEI message with sdi_aux_id[ i ] equal to omi_aux_id_minus128 + 128 for at least one value of i and an OMI SEI message, the SDI SEI message shall precede the OMI SEI message in decoding order.

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).