| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes,FR, 17–24 April 2024 | Document: JVET-AH0168-v1 |
| --- | --- |

| Title: | Template Matching Picture Boundary Padding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Nicolas Neumann, Mathias Wien | Tel: Email: | Nicolas.Neumann@lfb.rwth-aachen.de Mathias.Wien@lfb.rwth-aachen.de |
| Source: | RWTH Aachen University |  |  |

_____________________________

# Abstract

This contribution proposes template matching padding for reference picture boundary handling. It uses template matching structure prediction to generate the outside samples of the reference picture from content inside the reference picture. For this purpose, the outside area is partitioned into so-called target blocks. Then a template matching search is performed for each target block and the search results are used to fill the target block. The method can be used as a replacement for motion compensated padding (JVET-AA0096) or can be combined with this method. The combination first applies motion compensated padding and then fills the remaining area with template matching boundary padding instead of repetitive padding.

The proposed has been implemented on top of VTM 23.0 and the test results are summarized as follows. The percentual encoder runtime increases are not reported because the simulations ran on a heterogenous cluster but are considered to be much lower than the percentual decoder runtime increases since the complexity is equivalent for encoder and decoder side. The results for the template matching padding are:

RA configuration: -0.07% (Y), 0.04% (U), -0.05% (V), 102% (DecT)

LDB configuration: -0.06% (Y), 0.17% (U), 0.06% (V), 102% (DecT)

For comparison the motion compensated padding results are:

RA configuration: -0.28% (Y), -0.22% (U), -0.23% (V), 106% (DecT)

LDB configuration: -0.02% (Y), -0.05% (U), 0.07% (V), 103% (DecT)

And the results for the combination of motion compensated padding and template matching padding are:

RA configuration: -0.29% (Y), -0.22% (U), -0.19% (V), 107% (DecT)

LDB configuration: -0.07% (Y), -0.01% (U), -0.17% (V), 105% (DecT)

Additionally, a study on the capabilities for reference subpicture padding was performed. The experiment follows the CE 12 on tile set boundary handling defined in JVET-M1032 with a subpicture partitioning into 384x384 subpictures. Class A1 and A2 of the CTC (JVET-AB2010 ) are simulated. The proposed method was implemented on top of VTM 13.0 and the test results are summarized as follows:

RA configuration: -0.41% (Y) / -0.43% (U) / -0.40% (V), 101% (DecT)

LDB configuration: -0.35% (Y) / -0.33% (U) / -0.3% (V), 103% (DecT)

For comparison the motion compensated padding results are:

RA configuration: -1.09% (Y) / -0.95% (U) / -1.01% (V), 126% (DecT)

LDB configuration: -0.08% (Y) / -0.17% (U) / -0.00% (V), 112% (DecT)

And the results for the combination of motion compensated padding and template matching padding are:

RA configuration: -1.21% (Y) / -1.09% (U) / -1.16% (V), 134% (DecT)

LDB configuration: -0.31% (Y) / -0.19% (U) / -0.27% (V), 121% (DecT)

Additionally, a study on the capabilities for reference subpicture padding was performed. The experiment follows the CE 12 on tile set boundary handling defined in JVET-M1032 with a subpicture partitioning into 384x384 subpictures. All sequences of the 360 CTC (JVET-L1012) were simulated. The proposed method was implemented on top of VTM 13.0 and the test results are summarized as follows:

The results for the template matching padding are (BDR Y/Cb/Cr, decoder time):

RA configuration: -0.21% (Y) / -0.14% (U) / -0.30% (V), 101% (DecT)

LDB configuration: -0.22% (Y) / -0.12% (U) / -0.15% (V), 103% (DecT)

For comparison the motion compensated padding results are:

RA configuration: -0.61% (Y) / -0.48% (U) / -0.57% (V), 125% (DecT)

LDB configuration: -0.16% (Y) / -0.08% (U) / -0.12% (V), 114% (DecT)

And the results for the combination of motion compensated padding and template matching padding are:

RA configuration: -0.68% (Y) / -0.60% (U) / -0.77% (V), 131% (DecT)

LDB configuration: -0.25% (Y) / -0.25% (U) / -0.44% (V), 122% (DecT)

# Introduction

In VTM repetitive padding is used for reference picture padding. For ECM motion compensated padding [1] was proposed which yields improved compression efficiency. Motion compensated padding uses content from other reference pictures to pad the current reference picture for applicable areas. The remaining outside area is still padded with repetitive padding.

The proposed method describes a reference picture padding algorithm, which is applicable for the whole outside area of the reference picture and can be combined with motion compensated padding to substitute the repetitive padding.

# Proposed Method

The proposed method divides the outside area of the reference picture into target blocks, which are predicted by template matching. First, a template is defined adjacent to the target block. Then a search is performed in a defined area for matching structures with the sum of absolute differences (SAD) as similarity metric. For the best matching structures, so called source blocks are extracted and averaged to form the prediction for the target block. The general algorithm is described in [2]. The search only considers the luma component to reduce complexity.

Figure 1. Illustration of Template Matching. Search area (blue), Template (green), matching structure (yellow), target block (red) and source block (purple) are shown

The best matching structures are selected by adaptive candidate selection [2]. First, the structure with the lowest SAD is determined. Then the lowest SAD is multiplied with a factor 1.5 and the result is used as a threshold for the other structures, which are called candidates. This excludes outliers from the averaging process. Additionally, a maximum number of candidates is set to 4. The parameters were found empirically and were fixed for all experiments. They could potentially be signalled in the bitstream for more flexibility.

The search area is defined by the L1 distance to the template, which results in a triangle. The search starts with distance of 3 and is gradually increases until the stopping criterion is met or the maximum distance of 12 is reached. The stopping criterion is met when one of the candidates in the search area has an SAD value of 24 or lower. This search area shape and parameters were found as a good compromise between compression efficiency and complexity.

Figure 2. Template Matching Search Area: Colors mark different steps in the process

It was fixed for all experiments but could potentially be signalled in the bitstream for more flexibility. This procedure reduces the complexity of the search by early stopping if a good candidate is found. The early stopping procedure is described in more detail in [3].

The target block size is set to 12x1 in the direction parallel and perpendicular to the picture boundary respectively. The template size is set to 16x2 in those dimensions, and it is centered adjacent to the target block. For the corners of the picture, the target block size is increased by 2 pixels so that the template is fully available (see Figure 3). The search is performed for all target blocks adjacent to the picture boundary. The search results are then reused for outward target blocks. The outward target blocks are aligned perpendicular to the picture boundary and target blocks with the same alignment share the search results. Reusing the search results was found to reduce complexity significantly without reducing compression efficiency and was initially proposed in [4]. The template and target block sizes influence the complexity of the method. The described parameters were found to reduce compression efficiency only minimally compared to other configurations and simultaneously result in very reasonable complexity. Different configurations could be defined to better adapt to different content.

Figure 3. Template and Target Block Size: light area marks virtual target block increase, blue and green area show extended target block size at the corners of the picture.

The luma and chroma components need to be processed jointly to avoid color artifacts. To cope with chroma subsampled material, a process is applied that emulates and up- and downsampling step without performing the actual up- and downsampling, as described in [4]. This reduces complexity of the algorithm compared to actual up- and downsampling. The search results are described by displacement vectors, which indicate the shift from source block to target block. To derive the chroma, the virtual (upsampled) chroma values are derived by following the displacement vectors of the luma component. The value for the shifted virtual chroma position is found by nearest neighbor lookup from the downsampled domain. If the derived value is undefined because it is not yet padded, the displacement vector of the new position is followed until a defined value can be retrieved from the downsampled domain. The final value in the downsampled domain is derived by averaging the virtual upsampled chroma values corresponding to it. The process is executed, starting from the chroma values at the picture border moving outward.

Figure 4. Chroma Handling

The process pads the reference picture by only 16 pixels to the outside to reduce complexity and memory demands. It was found that compression efficiency does not increase if the template matching padding is executed further outward. This observation is in line with the findings for motion compensated padding [1]. Repetitive padding is used further out.

# Simulation Results

This section reports the simulation results of the template matching padding (a) motion compensated padding (b) and the combination (c), where the repetitive padding of motion compensated padding is substituted with template matching padding for the common test conditions for VTM [5], further denoted as test 1. Results are reported over VTM 23.0. The encoder runtimes are not reported because simulations ran on a heterogenous cluster. Decoder runtimes were determined on a single machine. Encoder runtimes are expected to be around 100% for all simulations since the complexity of the algorithms is equal for encoder and decoder side. The motion compensated padding implementation was taken from ECM with a padding width of 16 pixels. The consecutive repetitive padding was done with the same margin as used in the anchor for fair comparison for all methods.

Table 1: performances of test 1(a) template matching padding over VTM-23.0

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-23.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.08% | -0.04% | -0.14% |  | 101% |
| Class A2 | -0.01% | 0.07% | -0.08% |  | 101% |
| Class B | -0.07% | 0.10% | 0.01% |  | 101% |
| Class C | -0.11% | -0.02% | -0.03% |  | 103% |
| Class E |  |  |  |  |  |
| Overall | -0.07% | 0.04% | -0.05% |  | 102% |
| Class D | -0.19% | 0.06% | -0.09% |  | 101% |
| Class F | -0.02% | 0.05% | 0.00% |  | 101% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-23.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.07% | 0.15% | 0.33% |  | 101% |
| Class C | -0.08% | -0.09% | -0.03% |  | 103% |
| Class E | -0.03% | 0.56% | -0.28% |  | 103% |
| Overall | -0.06% | 0.17% | 0.06% |  | 102% |
| Class D | -0.52% | -0.38% | -0.97% |  | 108% |
| Class F | -0.28% | 0.08% | -0.50% |  | 102% |

Table 2: performances of test 1(b) motion compensated padding over VTM-23.0

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-23.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.24% | -0.25% | -0.19% |  | 103% |
| Class A2 | -0.18% | -0.11% | -0.19% |  | 103% |
| Class B | -0.34% | -0.20% | -0.26% |  | 105% |
| Class C | -0.29% | -0.29% | -0.24% |  | 110% |
| Class E |  |  |  |  |  |
| Overall | -0.28% | -0.22% | -0.23% |  | 106% |
| Class D | -0.74% | -0.59% | -0.53% |  | 119% |
| Class F | -0.04% | -0.02% | -0.06% |  | 105% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-23.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | 0.02% | 0.04% |  | 102% |
| Class C | -0.07% | 0.00% | 0.11% |  | 106% |
| Class E | 0.02% | -0.25% | 0.05% |  | 103% |
| Overall | -0.02% | -0.05% | 0.07% |  | 103% |
| Class D | -0.06% | -0.12% | 0.06% |  | 107% |
| Class F | -0.22% | 0.56% | -0.45% |  | 101% |

Table 3: performances of test 1(c) combined padding over VTM-23.0

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-23.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.28% | -0.19% | -0.12% |  | 104% |
| Class A2 | -0.18% | -0.10% | -0.14% |  | 103% |
| Class B | -0.36% | -0.26% | -0.20% |  | 106% |
| Class C | -0.31% | -0.27% | -0.28% |  | 112% |
| Class E |  |  |  |  |  |
| Overall | -0.29% | -0.22% | -0.19% |  | 107% |
| Class D | -0.79% | -0.46% | -0.63% |  | 122% |
| Class F | -0.02% | 0.00% | -0.02% |  | 108% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-23.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.07% | 0.21% | -0.27% |  | 103% |
| Class C | -0.07% | 0.02% | 0.07% |  | 107% |
| Class E | -0.09% | -0.43% | -0.33% |  | 105% |
| Overall | -0.07% | -0.01% | -0.17% |  | 105% |
| Class D | -0.40% | -0.49% | -1.56% |  | 110% |
| Class F | -0.15% | 0.66% | -0.48% |  | 103% |

Further, a study on the capabilities for reference subpicture padding is performed for regular video, further denoted as test 2. The experiment follows the CE 12 on tile set boundary handling [6] with a subpicture partitioning into 384x384 independently decodable subpictures. The methods (a), (b) and (c) are used at all subpicture boundaries. The class A1 and A2 sequences of the CTC [5] are simulated. Subpictures are simulated individually, and the BD-rate numbers averaged for the overall result. FoodMarket4 is excluded from the set because of problems with the simulations. The partitioning starts from the top left corner resulting in smaller subpictures at the bottom. The methods are implemented on top of VTM 13.0 for (a), (b) and (c). Results are reported with VTM 13.0 anchor because the simulations have been conducted at an earlier stage and simulations could not be repeated in time for the newer VTM version.

Table 8: performances of test 2(a) template matching padding over VTM-13.0

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-13.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Campfire | -0.03% | -0.13% | -0.14% |  |  |
| CatRobot | -0.52% | -0.38% | -0.50% |  |  |
| DaylightRoad2 | -0.40% | -0.13% | -0.33% |  |  |
| ParkRunning3 | -0.25% | -0.44% | -0.31% |  |  |
| Tango2 | -0.87% | -1.08% | -0.74% |  |  |
| Overall | -0.41% | -0.43% | -0.40% |  | 101% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-13.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Campfire | -0.10% | 0.17% | -0.07% |  |  |
| CatRobot | -0.57% | -0.56% | -0.67% |  |  |
| DaylightRoad2 | -0.20% | -0.39% | -0.02% |  |  |
| ParkRunning3 | -0.33% | -0.28% | -0.37% |  |  |
| Tango2 | -0.53% | -0.60% | -0.51% |  |  |
| Overall | -0.35% | -0.33% | -0.33% |  | 103% |

Table 9: performances of test 2(b) template matching padding over VTM-13.0

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-13.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Campfire | -0.22% | -0.28% | -0.30% |  |  |
| CatRobot | -0.86% | -0.69% | -0.81% |  |  |
| DaylightRoad2 | -1.85% | -1.46% | -1.59% |  |  |
| ParkRunning3 | -1.12% | -1.15% | -1.15% |  |  |
| Tango2 | -1.39% | -1.18% | -1.19% |  |  |
| Overall | -1.09% | -0.95% | -1.01% |  | 126% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-13.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Campfire | 0.03% | 0.00% | -0.01% |  |  |
| CatRobot | -0.20% | 0.13% | -0.13% |  |  |
| DaylightRoad2 | -0.17% | -0.50% | 0.14% |  |  |
| ParkRunning3 | -0.05% | -0.13% | -0.09% |  |  |
| Tango2 | -0.02% | -0.35% | 0.07% |  |  |
| Overall | -0.08% | -0.17% | 0.00% |  | 112% |

Table 10: performances of test 2(c) template matching padding over VTM-13.0

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-13.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Campfire | -0.28% | -0.34% | -0.37% |  |  |
| CatRobot | -1.01% | -0.84% | -1.00% |  |  |
| DaylightRoad2 | -1.97% | -1.52% | -1.75% |  |  |
| ParkRunning3 | -1.24% | -1.35% | -1.18% |  |  |
| Tango2 | -1.56% | -1.42% | -1.48% |  |  |
| Overall | -1.21% | -1.09% | -1.16% |  | 134% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-13.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Campfire | -0.14% | 0.49% | -0.22% |  |  |
| CatRobot | -0.59% | -0.77% | -0.82% |  |  |
| DaylightRoad2 | -0.24% | -0.01% | 0.34% |  |  |
| ParkRunning3 | -0.24% | -0.28% | -0.26% |  |  |
| Tango2 | -0.35% | -0.38% | -0.37% |  |  |
| Overall | -0.31% | -0.19% | -0.27% |  | 121% |

Table 11: summary table for test 2

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-13.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| a | -0.41% | -0.43% | -0.40% |  | 101% |
| b | -1.09% | -0.95% | -1.01% |  | 126% |
| c | -1.21% | -1.09% | -1.16% |  | 134% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-13.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| a | -0.35% | -0.33% | -0.33% |  | 103% |
| b | -0.08% | -0.17% | 0.00% |  | 112% |
| c | -0.31% | -0.19% | -0.27% |  | 121% |

Additionally, a study on the capabilities for reference subpicture padding is performed for 360 video, further denoted as test 3. The experiment follows the CE 12 on tile set boundary handling [6] with a subpicture partitioning into 384x384 independently decodable subpictures. The methods (a), (b) and (c) are used at all subpicture boundaries. All sequences of the 360 CTC [7] are converted into projected Cubemap format (CMP) and simulated. Subpictures are simulated individually, and the BD-rate numbers averaged for the overall result. The methods are implemented on top of VTM 13.0 for (a) motion compensated padding (b) and the combination (c).

Table 4: performances of test 3(a) template matching padding over VTM-13.0

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-13.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Balboa | -0.37% | -0.25% | -0.32% |  |  |
| BranCastle2 | -0.17% | -0.19% | -0.37% |  |  |
| Broadway | -0.60% | -0.42% | -0.73% |  |  |
| ChairliftRide | -0.36% | -0.38% | -0.33% |  |  |
| Gaslamp | 0.00% | 0.01% | -0.06% |  |  |
| Harbor | -0.02% | 0.03% | -0.14% |  |  |
| KiteFlite | 0.00% | -0.01% | 0.00% |  |  |
| Landing2 | -0.33% | -0.30% | -0.43% |  |  |
| SkateboardInLot | -0.20% | 0.15% | -0.61% |  |  |
| Trolley | -0.01% | -0.05% | 0.03% |  |  |
| Overall | -0.21% | -0.14% | -0.30% |  | 101% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-13.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Balboa | 0.32% | -0.68% | -0.23% |  |  |
| BranCastle2 | -0.15% | -0.25% | 0.05% |  |  |
| Broadway | -0.89% | -0.09% | -0.08% |  |  |
| ChairliftRide | -0.52% | -0.75% | -1.19% |  |  |
| Gaslamp | -0.09% | -0.05% | 0.02% |  |  |
| Harbor | -0.12% | -0.08% | -0.42% |  |  |
| KiteFlite | -0.03% | -0.12% | 0.22% |  |  |
| Landing2 | -0.35% | -0.04% | -0.30% |  |  |
| SkateboardInLot | -0.26% | 1.13% | 0.28% |  |  |
| Trolley | -0.12% | -0.24% | 0.12% |  |  |
| Overall | -0.22% | -0.12% | -0.15% |  | 103% |

Table 5: performances of test 3(b) template matching padding over VTM-13.0

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-13.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Balboa | -1.15% | -1.03% | -1.01% |  |  |
| BranCastle2 | -0.70% | -0.91% | -0.89% |  |  |
| Broadway | -1.17% | -1.02% | -1.04% |  |  |
| ChairliftRide | -1.69% | -1.47% | -1.23% |  |  |
| Gaslamp | -0.04% | -0.02% | -0.05% |  |  |
| Harbor | -0.03% | -0.01% | -0.08% |  |  |
| KiteFlite | -0.03% | -0.03% | -0.03% |  |  |
| Landing2 | -0.56% | -0.70% | -0.85% |  |  |
| SkateboardInLot | -0.74% | 0.43% | -0.56% |  |  |
| Trolley | -0.02% | -0.03% | 0.02% |  |  |
| Overall | -0.61% | -0.48% | -0.57% |  | 125% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-13.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Balboa | -0.18% | -0.43% | -0.29% |  |  |
| BranCastle2 | -0.35% | -0.54% | -0.42% |  |  |
| Broadway | -0.28% | -0.15% | -0.02% |  |  |
| ChairliftRide | -0.27% | -0.41% | -0.77% |  |  |
| Gaslamp | -0.03% | 0.17% | 0.04% |  |  |
| Harbor | -0.02% | 0.12% | -0.07% |  |  |
| KiteFlite | -0.02% | 0.11% | 0.30% |  |  |
| Landing2 | -0.25% | -0.03% | -0.46% |  |  |
| SkateboardInLot | -0.21% | 0.34% | 0.19% |  |  |
| Trolley | -0.03% | -0.01% | 0.31% |  |  |
| Overall | -0.16% | -0.08% | -0.12% |  | 114% |

Table 6: performances of test 3(c) template matching padding over VTM-13.0

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-13.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Balboa | -1.27% | -1.23% | -1.22% |  |  |
| BranCastle2 | -0.75% | -0.98% | -0.98% |  |  |
| Broadway | -1.32% | -1.15% | -1.14% |  |  |
| ChairliftRide | -1.82% | -1.66% | -1.43% |  |  |
| Gaslamp | -0.06% | -0.08% | -0.13% |  |  |
| Harbor | -0.06% | -0.02% | -0.15% |  |  |
| KiteFlite | -0.04% | -0.04% | -0.02% |  |  |
| Landing2 | -0.66% | -0.89% | -0.84% |  |  |
| SkateboardInLot | -0.77% | 0.08% | -1.77% |  |  |
| Trolley | -0.05% | -0.08% | -0.03% |  |  |
| Overall | -0.68% | -0.61% | -0.77% |  | 131% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-13.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Balboa | 0.02% | -0.71% | -0.21% |  |  |
| BranCastle2 | -0.42% | -0.72% | -0.82% |  |  |
| Broadway | -0.51% | -0.58% | -0.27% |  |  |
| ChairliftRide | -0.52% | -0.70% | -0.85% |  |  |
| Gaslamp | -0.10% | -0.05% | -0.10% |  |  |
| Harbor | -0.14% | 0.01% | -0.25% |  |  |
| KiteFlite | -0.04% | -0.07% | 0.13% |  |  |
| Landing2 | -0.39% | -0.31% | -0.37% |  |  |
| SkateboardInLot | -0.31% | 0.90% | -1.70% |  |  |
| Trolley | -0.08% | -0.20% | 0.06% |  |  |
| Overall | -0.25% | -0.24% | -0.44% |  | 122% |

Table 7: summary table for test 3

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-13.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| a | -0.21% | -0.14% | -0.30% |  | 101% |
| b | -0.61% | -0.48% | -0.57% |  | 125% |
| c | -0.68% | -0.61% | -0.77% |  | 131% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-13.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| a | -0.22% | -0.12% | -0.15% |  | 103% |
| b | -0.16% | -0.08% | -0.12% |  | 114% |
| c | -0.25% | -0.24% | -0.44% |  | 122% |

# Conclusion

The proposed combination method (c) results in a BD-rate gain of -0.07% on top of VTM for luma (test 1) compared to -0.02% for motion compensated padding (b) for the CTC simulations.

It can be seen that the two methods involving motion compensated padding (b) and (c) show improvements over VTM in the random access configuration.

Test 2 involves a test case involving independently decodable subpictures and is therefore more demanding in terms of reference (sub)picture padding. Test 2 results show luma BD-rate gains of -1.21% / -0.31% for the combination method (c) compared to -1.09% / -0.08% for motion compensated padding (b) while increasing decoder runtime from 126% / 112% to 134% / 121% for the random access / low delay B configuration respectively for reference subpicture coding on 360 video. The template matching padding can improve luma BD-rate from -0.08% to -0.35% while reducing decoder runtime from 126% to 103% for this case. This shows that the combination method can improve compression performance compared to only using motion compensated padding. Further, template matching boundary padding outperforms motion compensated padding in the low delay case in both compression efficiency and decoder runtime increase.

Test 3 also considers the case of reference subpicture padding. The results show luma BD-rate gains of -0.68% / -0.25% for the combination method (c) compared to -0.61% / -0.16% for motion compensated padding (b) while increasing decoder runtime from 125% / 114% to 131% / 122% for the random access / low delay B configuration respectively for reference subpicture coding on 360 video. The template matching padding can improve luma BD-rate from -0.16% to -0.22% while reducing decoder runtime from 114% to 103% for this case. This supports the previous statements on a different test set.

Class F results of the CTC suggest that the statements also hold for sequences of small resolution.

The tests were performed with a fixed set of parameters. The scheme is very flexible and compression efficiency can potentially be improved if parameters are fitted to the sequences and optimized by the enocoder.

Since template matching boundary padding outperforms motion compensated padding for the low delay case and a combination does improve compression efficiency on average compared to using motion compensated padding alone, it is proposed to integrate template matching boundary padding (alongside motion compensated padding) for reference subpicture padding in ECM.

# References

[1] JVET-AA0096 - Fabrice Le Léannec, Pierre Andrivon, Miloš Radosavljevć „EE2-2.2: Motion compensated picture boundary padding“

[2] N. Horst, P. Das and M. Wien ”A Template Matching Approach for Reference Picture Padding in Video Coding” IEEE International Conference on Acoustics, Speech and Signal Processing ICASSP. 2023

[3] N. Horst, P. Das, T. Classen and M. Wien ”Balancing Complexity of Template Matching-based Reference Picture Padding for Video Coding” accepted at Picture Coding Symposium PCS. 2024

[4] N. Horst and M. Wien ”Complexity Reduction of Template Matching-based Reference Picture Padding in Video Coding” accepted at IEEE International Conference on Acoustics, Speech and Signal Processing ICASSP. 2024

[5] JVET-AB2010 - Frank Bossen, Xiang Li, Karl Sharman, Vadim Seregin, Karsten Suehring „VTM and HM common test conditions and software reference configurations for SDR 4:2:0 10-bit video“

[6] JVET-M1032 - Hendry, Robert Skupin, Wade Wan „Description of Core Experiment 12 (CE12): Tile Set Boundary Handling“

[7] JVET-L1012 - P. Hanhart, J. Boyce, K. Choi, J.-L. Lin „JVET common test conditions and evaluation procedures for 360° video“

# Patent rights declaration(s)

RWTH Aachen University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).