| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 17–24 April 2024 | Document: JVET-AH0169-v2 |
| --- | --- |

| Title: | Video CICP problem reports and proposed actions |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Errata report (basically editorial) |  |  |
| Author(s) or Contact(s): | Gary J. Sullivan | Email: | g-j-sullivan@outlook.com |
| Source: | Dolby Labs |  |  |

_____________________________

# Abstract

This contribution reports some editorial problems with the pre-published text of Rec. ITU-T H.273 “Coding-independent code points for video signal type identification” and suggests action to address them. Version 1 of the contribution corresponds to contribution SG16-C499 submitted to the current meeting of ITU-T SG16. The reported problems also generally apply to the in-progress draft text of ISO/IEC 23091-2. An additional problem is reported in version 2.

# Problem reports

H.273 V3 was Consented in July 2023 and has been pre-published. However, it remains pending final publication due to its informative reference to SMPTE ST 2128, since that standard remains unpublished. This contribution reports on editorial problems in the text and suggests action to address them. We consider all of these problems to be essentially editorial, with the possible exception of the SMPTE ST 2128 problem.

1)	The reference to SMPTE ST 2128 is found in clause 8.3, Table 4 – Interpretation of matrix coefficients (MatrixCoefficients) value, in the row for MatrixCoefficients equal to 15 (IPT-C2). Since this reference is merely informative, the reference could simply be deleted, although this would result in an absence of an explanation of why this colour representation scheme is specified in H.273.

2)	In clause 5.4, Equation 4, the definition of the Clip3( ) function, there is an obvious typographical error in the specification.

Clip3( x, y, z ) =  x;z>xy;z>yz;otherwise						(4)

This should be

Clip3( x, y, z ) =  x;z<xy;z>yz;otherwise						(4)

The intent is to clip the value of z between the range from x to y. The same formula is defined in several other standards (e.g. H.264, H.265, H.266, H.274), but without the error.

3)	In clause 8.2, Table 3 – Interpretation of transfer characteristics (TransferCharacteristics) value, in the row for TransferCharacteristics equal to 16 (for SMPTE ST 2084 (2014) for 10-, 12-, 14- and 16-bit systems and the Rec. ITU-R BT.2100-2 perceptual quantization (PQ) system), there is an error in the definition of the constant n. The text has “n = 653 ÷ 4096 = 0.1593017578125”, which any calculator will show is simply a false statement. This was apparently the result of an attempt at editorial simplification of a prior form of the calculation expressed as “0.25 * 2610 ÷ 4096. The “653 ÷ 4096” should be changed to “1305 ÷ 8192” or something else that is editorially equivalent to the given correct decimal number 0.1593017578125. This error is also found in the current edition of Rec. H.264 but is not found in the corresponding expression in Rec. H.265.

4)	In clause 8.3, Equations 24 and 25, there is a clear sign error.

BitDepthRGB = BitDepthY + 2							(24)

BitDepthRGB = BitDepthY + 1							(25)

The addition in each of these equations should be a subtraction. The equations are describing how lower bit-depth RGB components can be losslessly converted into higher bit-depth Y, Cb and Cr components, which obviously means that BitDepthRGB should be less than BitDepthY rather than being greater.

5)	In clause 8.3, Equation 85, the equation number is not aligned at the right side of the page. Instead, it appears next to the equation itself, making the equation hard to read.

6)	In clause 5.3, in the definitions of the arithmetic right and left shift operations (“x >> y” and “x << y”), it is stated that the operation is “defined only for positive integer values of y”, where y is the amount of shifting to be performed. These operations may also be applied when y is zero, so “positive” should be replaced by “non-negative”. Moreover, since these are defined as “arithmetic” shift operations (rather than bit-wise operations), it may be better for their definitions to be in clause 5.1 rather than clause 5.3.

In sum, we suggest discussion of the status of SMPTE ST 2128 and correction of the identified editorial errors prior to publication.

# Patent rights declaration

Dolby Laboratory may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

_______________________