| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes,FR, 17–24 April 2024 | Document: JVET-AH0170-v2 |
| --- | --- |

| Title: | Non-EE2: ALF residuals scaling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Philippe Bordes, Franck Galpin, Guillaume Boisson | Email: | philippe.bordes@interdigital.com, franck.galpin@interdigital.com, guillaume.boisson@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This contribution presents an ALF correction (residuals) scaling method. The scaling factors are coded per slice for the non-fixed APS-ALF. The proposed method was implemented on top of ECM-12.0, it reportedly provides {Y, U, V} BD-reduction with encoder and decoder runtimes as follows:

RA: -0.04%, -0.01%, 0.06%, 100.0% (EncT), 100.0% (DecT), LDB: -0.09%, -0.19%, 0.14%, 100.0% (EncT), 100.0% (DecT).

# Introduction

The APS-ALF signals non-fixed coefficients that may be re-used for the following pictures. However, these coefficients have been optimized by encoder for one frame and may not be perfectly tailored for other frames with different characteristics and coding parameters (QP…).

In EE1, when a NN filter is being applied to reconstructed pictures, a scaling factor is signaled for each color component in the picture header. The difference between the input samples and the NN filtered samples (residues) are scaled by the scaling factors before being added to input samples [1].

# Proposal and results

It is proposed to signal luma scaling factors to be applied on the non-fixed APS-ALF correction residuals. Different scaling factors may be associated with different group of class indexes.

The proposal has been implemented in ECM-12.0 and tested with CTC-RA/LDB conditions.

Table 1: Performance results of proposal in RA.

|  | Random Access Main10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DncVmPeak |
| Class A1 | -0,06% | -0,06% | 0,04% | 100,2% | 100,2% | 100,1% | 99,8% |
| Class A2 | -0,02% | -0,02% | 0,06% | 100,2% | 100,0% | 100,0% | 99,9% |
| Class B | -0,03% | 0,01% | 0,07% | 99,6% | 100,3% | 100,1% | 100,2% |
| Class C | -0,04% | -0,01% | 0,08% | 100,7% | 100,1% | 99,3% | 99,8% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0,04% | -0,02% | 0,06% | 100,1% | 100,2% | 99,9% | 100,0% |
| Class D | -0,08% | 0,15% | 0,14% | 100,8% | 99,8% | 100,2% | 100,1% |
| Class F |  |  |  |  |  |  |  |
| Class TGM |  |  |  |  |  |  |  |

Table 2: Performance results of proposal in LDB.

|  | Low Delay B Main10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DncVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0,06%* | 0,19%* | 0,36%* | #NUM! | #NUM! |  |  |
| Class C | -0,11% | 0,21% | 0,27% | 99,3% | 99,9% | 100,0% | 100,1% |
| Class E | -0,13% | -1,35% | -0,42% | 100,1% | 99,8% | 99,8% | 99,6% |
| Overall | -0,09% | -0,19% | 0,14% | #NUM! | #NUM! |  |  |
| Class D | -0,20% | -0,49% | -0,28% | 100,0% | 100,0% | 100,3% | 100,1% |
| Class F |  |  |  |  |  |  |  |
| Class TGM |  |  |  |  |  |  |  |

# Conclusion

This contribution proposes to signal scaling factors for non-fixed APS-ALF. The results show that the proposed method can increase the compression performance of ECM-12.0. It is proposed to include this method in the next round of EE2.

# References

- Y.Li, K.Zhang, L.Zhang (Bytedance), H.Wang, M.Coban, A.Kotra, M. Karczewicz(Qualcomm), F.Galpin (InterDigital), “EE1-1.6: Test on Deep In-Loop Filter with Adaptive Parameter Selection and Residual Scaling based on SADL implementation,” document JVET-Z0112, 26th Meeting, by teleconference, 20–29 April 2022.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).