| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 17–24 April 2024 | Document: JVET-AG0308-v2 |
| --- | --- |

| Title: | Crosscheck of JVET-AH0245 (EE2-2.10f: MR-SAD/SATD based IntraTMP BV search with signalling and BV reordering) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | G. Verba | Tel: Email: | gverba@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution reports the cross-check result of EE2 Test 2.10f in JVET-AH0245 proposed by InterDigital and Kwai combining:

- a SATD based cost function replacing the current IntraTMP BV search on refinement search;

- an additional cost metric for IntraTMP BV search for the sparse search with a flag signaled to the decoder to indicate the choice between the default metric (SAD) and the additional one (MR-SAD);

- a reordering method to reduce the signaling overhead of IntraTMP BV.

The crosschecking report presents the BD-rate performance in AI and RA configurations over ECM-12.0. The simulation results match the reported results JVET-AH0245 in term of BD-rate performance for AI configuration. For RA configuration, a divergence in BD-rate performance is observed. The observed runtimes are higher than ones reported by the proponents. 2 sequences cannot be decoded properly in RA configurations.

# Crosschecking results

This contribution provides the crosscheck result of Test 2.10f of EE2 reported in JVET-AH0245. The proposed methods were implemented on top of ECM-12.0. The BD-rate results match the results reported. The simulation results are as follows:

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.02% | 0.03% | 0.10% | 107.3% | 101.9% | 100.2% | 100.7% |
| Class A2 | -0.12% | -0.11% | -0.12% | 106.2% | 108.8% | 100.8% | 100.7% |
| Class B | -0.10% | -0.04% | -0.06% | 107.6% | 110.6% | 100.4% | 101.2% |
| Class C | -0.08% | -0.13% | -0.05% | 107.9% | 106.3% | 100.7% | 101.5% |
| Class E | -0.25% | -0.37% | -0.25% | 108.2% | 110.3% | 100.6% | 101.4% |
| Overall | -0.11% | -0.12% | -0.07% | 107.5% | 107.8% | 100.6% | 101.1% |
| Class D | -0.06% | 0.10% | -0.22% | 105.3% | 106.2% | 100.9% | 101.6% |
| Class F | -0.23% | 0.00% | -0.12% | 110.4% | 108.9% | 100.9% | 101.4% |
| Class TGM | -0.21% | -0.19% | -0.32% | 109.1% | 108.0% | 100.4% | 101.1% |

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B* | -0.06% | -0.02% | -0.07% | 100.8% | 100.7% | 100.0% | 100.6% |
| Class C | -0.02% | -0.12% | -0.02% | 99.8% | 99.9% | 100.2% | 101.4% |
| Class E |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D | 0.01% | -0.20% | 0.28% | 100.5% | 100.8% | 101.1% | 101.6% |
| Class F | -0.13% | -0.18% | -0.01% | 105.4% | 103.9% | 100.3% | 100.2% |
| Class TGM | 0.04% | 0.00% | 0.01% | 102.9% | 100.2% | 100.0% | 97.7% |

* A decoding mismatch is observed for MarketPlace at QP22, the point is replaced with anchor
* A decoder fault is observed for BQTerrace at QP22, the point is replaced with anchor

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).