| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 17–24 April 2024 | Document: JVET-AH0339-v2 |
| --- | --- |

| Title: | Crosscheck of JVET-AH0058 (Non-EE2: derived MIP modes with fusion) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | T. Dumas | Email: | thierry.dumas@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This contribution summarizes the crosscheck results of JVET-AH0058 (Non-EE2: derived MIP modes with fusion). It is reported that the partial BD-rates obtained by the cross-check match the BD-rates reported by JVET-AH0058.

# Introduction

In JVET-AH0058 [1], a method for deriving MIP modes with fusion is proposed for intra prediction. For the current luma CB to be predicted, during a derivation phase, a template-based method is utilized to derive the two best MIP modes and the associated transposed flags. Then, during a prediction phase, the two predictions of the current luma CB via the two derived MIP modes respectively are fused with weights. For small and non-square blocks, the proposed method applies the DIMD process to the upsampled prediction samples to derive the LFNST transform set.

# Cross-check

The code of JVET-AH0058 was run following the AHG12 CTC [2]. It is reported that the partial BD-rates resulting from this run match the BD-rates reported in JVET-AH0058.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |
| Class C | -0.08% | 0.10% | 0.14% |  |  |  |  |
| Class E |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D | -0.09% | 0.11% | 0.06% |  |  |  |  |
| Class F |  |  |  |  |  |  |  |
| Class TGM |  |  |  |  |  |  |  |

Figure 1: DB-rates results of JVET-AH0058 versus ECM-12.0 in AI.

# Reference

- JVET-AH0058, “Non-EE2 : derived MIP modes with fusion”, Z. Xie, Y. Yu, H. Yu. 34th Meeting, Rennes, 17–24 April 2024.

- JVET-AF2017, “Common test conditions and evaluation procedures for enhanced compression tool testing”, M. Karczewicz, Y. Ye, 32nd Meeting, Hannover, DE, 13–20 October 2023.