| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 17–24 April 2024 | Document: JVET-AH0350-v2 |
| --- | --- |

| Title: | AHG9: Handling of a processing chain |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Nokia Technologies, Tampere, Finland Ye-Kui Wang Bytedance Inc., San Diego, USA | Email: | miska.hannuksela@nokia.com yekui.wang@bytedance.com |
| Source: | Nokia, Bytedance Inc. |  |  |

_____________________________

# Abstract

The contribution proposes specification text for handling of processing chains defined by SEI processing order (SPO) SEI messages.

# Introduction

In JVET's review of aspects 5 and 6 of JVET-AH0121, the following was noted:

It is agreed that the appropriate place to describe the interfaces in the processing chain is the SPO SEI rather than VSEI as was suggested in JVET-AH0047. Proponents of 0047 are asked to carefully check and communicate offline with proponents of 0121 for correctness on items 5 and 6. A sentence should be added that a decoder should operate at most one processing chain at one time.

It was suggested that a more generic handling of the case when an SEI message needs information about the nature of its input (as in case of colour conversion) would be desirable. E.g., if the previous stage has not modified chroma format idc, it needs to be inferred from some earlier stage.

In response to the comments above, this input document proposes a new subclause within D.11 ("SEI processing order and processing order nesting SEI message").

Two options are proposed for JVET's consideration:

- Breadth-first: all the pictures are processed by a processing stage before moving on the next processing stage.

- Depth-first: a picture is processed by all the processing stages, before moving on the next picture in output order.

Section 2 of this contribution presents some examples of asserted impacts of breadth-first and depth-first handling.

# Examples

## Quality enhancement followed by picture rate upsampling

In this example, a processing chain has the following two processing stages:

- Quality enhancement NNPF, which takes three consecutive cropped decoded pictures, in output order, as input and filters the mid-most picture. The mid-most input picture to each quality enhancement NNPF activation has a lower quality than the surrounding input pictures, and the quality of the mid-most input picture is enhanced to be closer to the quality of the surrounding input pictures. The NNPFA SEI messages activating the quality enhancement NNPF are present in picture units 4, 8, 12, and 16.

- Picture rate upsampling NNPF, which takes two consecutive pictures, in output order, as input and generates one intermediate picture as output. The NNPFA SEI messages activating the picture rate upsampling NNPF are present in picture units 2, 4, 6, 8, 10, 12, 14, and 16.

The breadth-first and depth-first handlings of the processing chain are depicted in Figure 1 and Figure 2, respectively. In the depth-first handling, the NNPFA SEI messages that activate the picture rate upsampling in picture units 2, 6, 10, and 14 are processed before the respective quality-enhanced pictures 2, 6, 10, and 14 have been generated. Hence, the breadth-first and depth-first handlings result into different output sequences.

Figure 1 – Breadth-first handling of a processing chain of a quality enhancement NNPF followed by a picture rate upsampling NNPF.

Figure 2 – Depth-first handling of a processing chain of a quality enhancement NNPF followed by a picture rate upsampling NNPF.

## Quality enhancement followed by film grain synthesis

In this example, a processing chain has the following two processing stages:

- Quality enhancement NNPF, which takes three consecutive cropped decoded pictures, in output order, as input and filters the mid-most picture. The mid-most input picture to each quality enhancement NNPF activation has a lower quality than the surrounding input pictures, and the quality of the mid-most input picture is enhanced to be closer to the quality of the surrounding input pictures. The NNPFA SEI messages activating the quality enhancement NNPF are present in picture units 4, 8, 12, and 16.

- Film grain synthesis, which processes each picture individually. The film grain characteristics SEI messages are present in picture units 0, 2, 4, 6, 8, 10, 12, 14, and 16.

The breadth-first and depth-first handlings of the processing chain are depicted in Figure 3 and Figure 4, respectively. It can be observed that in the depth-first handling of pictures 2, 6, 10, and 14, the film grain synthesis is performed prior to the quality enhancement NNPF, conflicting with the processing order defined in the SPO SEI message. For example, when picture 2 is processed, the film grain synthesis is performed with the cropped decoded picture 2 as input. After that, when picture 4 is processed, the NNPFA SEI message that is present in picture unit 4 is processed, resulting into quality-enhanced picture 2.

Figure 3 – Breadth-first handling of a processing chain of a quality enhancement NNPF followed by film grain synthesis.

Figure 4 – Depth-first handling of a processing chain of a quality enhancement NNPF followed by film grain synthesis.

# Proposed specification text

#### D.11.1.2 SEI processing order SEI message semantics

…

SeiProcessingOrderSeiList is set to consist of the payloadType values 3, 4, 5, 19, 137, 142, 144, 147, 148, 149, 150, 153, 155, 165, 177, 210, and 211. The value of po_sei_payload_type[ i ] for each i in the range of 0 to po_num_sei_messages_minus2 + 1, inclusive, shall be equal to a value in SeiProcessingOrderSeiList.

SpoProcessingList is set to consist of the payloadType values 19, 142, 155, 210, and 211.

...

### D.11.3 Handling of a processing chain

Processing chains are alternative to each other, i.e., at most one processing chain can be chosen to be applied by a decoding system at one time.

A decoding system may choose and apply a processing chain as follows:

–	First, the bitstream is decoded, and the list PoPicList is set to be the list of the cropped decoded pictures in output order that resulted from decoding the bitstream, and a processing chain is chosen.

–	(Option 1: picture-by-picture for one filter, zig zag, breadth first) For each of the SEI message types of the chosen processing chain, the following applies in a non-decreasing order of the corresponding po_sei_processing_order[ i ] values:

–	The following applies for each picture picA in PoPicList in output order, when an SEI message associated with the i-th SEI message type persists for picA or a picture for which an NNPF that generated picA was activated by a preceding process in the processing chain:

–	When picA is not a cropped decoded picture, the following exceptions apply for the interpretation of the SEI message:

–	The interface variables for purposes of interpretation of the SEI message are derived from picA instead of the syntax elements indicating properties for the respective cropped decoded picture.

–	The semantics of the SEI message, or the semantics of the SEI message and, when the SEI message is an NNPFA SEI message, the associated NNPFC SEI message, apply to pictures in PoPicList instead of cropped decoded pictures.

–	When the i-th SEI message type is present in SpoProcessingList, the process implied by the SEI message is performed and PoPicList is updated by replacing pictures with the corresponding processed pictures, if any, resulting from the process and inserting the other pictures, if any, resulting from the process into PoPicList so that the output order is obeyed.

–	(Option 2: filter-by-filter for one picture, zag zip, depth first) The following is repeatedly applied, in output order, for each picture picA in PoPicList for which a set of SEI messages associated with SEI message types in SpoProcessingList of the chosen processing chain persist for picA, the following applies:

–	The following applies for each of the set of SEI messages in a non-decreasing order of the corresponding po_sei_processing_order[ i ] values:

–	When the current SEI message is not the first in the set of the SEI messages, the following exceptions apply for the interpretation of the SEI message:

–	The interface variables for purposes of interpretation of the SEI message are derived from the pictures in the updated PoPicList instead of the syntax elements indicating properties for the respective cropped decoded pictures.

–	The semantics of the SEI message, or of the SEI message and, when the SEI message is an NNPFA SEI message, the associated NNPFC SEI message, apply to pictures in PoPicList instead of cropped decoded pictures.

–	The process implied by the SEI message is invoked repeatedly, in output order, for picA and each of the pictures in PoPicList that are, or correspond to, interpolated or extrapolated pictures generated by the application of the process implied by any preceding SEI message, if any, to picA. After each invocation of the process, PoPicList is updated by replacing pictures with the corresponding processed pictures, if any, resulting from the process and inserting the other pictures, if any, resulting from the process into PoPicList so that the output order is obeyed.

# Patent rights declaration(s)

Nokia and Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, are prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).