| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes,FR, 17–24 April 2024 | Document: JVET-AH0352-v2 |
| --- | --- |

| Title: | AHG9: Updated semantics for syntax element po_sei_wrapping_flag[ i ] |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry Miska M. Hannuksela | Email: | dr.hendry@lge.com miska.hannuksela@nokia.com |
| Source: | LG Electronics and Nokia |  |  |

_____________________________

# Abstract

This contribution proposes updated semantics for syntax element po_sei_wrapping_flag[ i ]. The proposed update is the result of offline discussion to improve the proposed solution by contribution JVET-AH0159 Item 1.

In the first revision of this contribution, updates to the proposed semantics suggested by an expert during review of v1 of this contribution are incoroporated.

# Problem Statement

After the first round review of contribution JVET-AH0159 Item 1, it was suggested to do offline discussion to work on refining the original proposed solution. The asserted problem was that the current semantic of po_sei_wrapping_flag[ i ] may be interpreted as it is suggesting for processing order nesting (PON) SEI message to be present when its value is equal to 1. This was confirmed to be not the intent.

The original text modification suggested by JVET-AH0159 Item 1 has the issue such that when po_sei_wrapping_flag[ i ] is equal to 1, it disallows an SEI that is present outside PON SEI message but matches the payloadType of that i-th SEI message specified by the sei processing order (SPO) SEI message. From the offline discussion, it was concluded that the semantics should express that the above case should be allowed and the semantics should express that such SEI is not the SEI that is applicable / considered as the i-th SEI in that SPO SEI message.

# Proposal and text implementation

To solve the issue described above, it is proposed to update the semantics of po_sei_wrapping_flag[ I ] as follows:

po_sei_wrapping_flag[ i ] equal to 1 specifies that one or more processing order nesting SEI messages with both of the following constraints should be present:

po_sei_wrapping_flag[ i ] equal to 1 specifies that an SEI message that applies as the i-th SEI message type in the processing chain specified in this SPO SEI message, if present, is an SEI message that is included in a PON SEI message for which both of the following conditions are true:

–	pon_target_po_id[ j ] with any value of j is equal to po_id.

–	There is a k-th loop entry in the processing order nesting SEI message such that the payloadType of the k-th nested SEI message is equal to po_sei_payload_type[ i ] and pon_processing_order[ k ] is equal to po_sei_processing_order[ i ].

When po_sei_wrapping_flag[ i ] is equal to 0, an SEI message with payloadType equal to po_sei_payload_type[ i ] (and, when po_sei_prefix_flag[ i ] equal to 1, prefix data that matches the values of po_sei_prefix_data_bit[ i ][ j ]) should be present outside of the processing order nesting SEI message.

po_sei_wrapping_flag[ i ] equal to 0 specifies that an SEI message that applies as the i-th SEI message type in the processing chain specified in this SPO SEI message, if present, is an SEI message that is not included in a PON SEI message and for which both of the following conditions are true:

–	The payloadType of the SEI message is equal po_sei_payload_type[ i ].

–	po_sei_prefix_flag[ i ] is equal to 0, or when po_sei_prefix_flag[ i ] is equal to 1, the payload of the SEI message starts with the values of po_sei_prefix_data_bit[ i ][ j ].

NOTE 2 – po_sei_wrapping_flag[ i ] equal to 1 enables SEI messages to be carried within the processing order nesting SEI message to prevent such SEI messages from being incorrectly interpreted by decoders that do not process the SPO SEI message. Thus, po_sei_wrapping_flag[ i ] equal to 1 is intended to be used when po_sei_wrapping_flag[ i ] equal to 0 can lead to unintended results being produced by such decoders.

# Patent rights declaration(s)

LG Electronics and Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).