| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0060-v1 |
| --- | --- |

| Title: | AHG9: Photosensitive Content Information Signaling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sachin Deshpande | Tel: Email: | +1 360 817 8486 sdeshpande@sharplabs.com |
| Source: | Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

It is proposed to indicate in an SEI, photosensitive content information such as strobe lights/ flashing lights in an image and sequence of images, which could cause photosensitive epilepsy/ seizures. Two alternative options are proposed.

- Option 1: A new txt_descr_purpose is proposed to indicate and provide textual information regarding photosensitive content in text description information SEI.

- Option 2: A new SEI message to indicate photosensitive content information is proposed.

# Introduction

The VSEI V4 draft 2 [1] includes a text description information SEI.

## It has been reported that photosensitive epilepsy occurs in 1 in 4000 of the population. Video sequences can precipitate photosensitive seizures, typically when the image includes photosensitive content such as strobe lights/ flashing lights.

It is proposed to indicate in an SEI, photosensitive content information such as strobe lights/ flashing lights in an image and sequence of images, which could cause photosensitive epilepsy/ seizures. Two alternative options are proposed.

- Option 1: It is proposed to use the text description information SEI to indicate a picture (or sequence of pictures) which include photosensitive content such as strobe lights and flashes which may cause photosensitive seizures. It is proposed to use txt_descr_purpose equal to 6 and then to specify details about the type of photosensitive material in the txt_descr_string[ i ] syntax element in the SEI.

- Option 2: A new SEI message to indicate photosensitive content information is proposed.

# Proposal

It is proposed to indicate photosensitive content information via SEI. Two alternative options are proposed.

## Option 1:

## A new txt_descr_purpose is proposed to indicate and provide textual information regarding strobe lights/ flashing lights in an image and sequence of images which could cause photosensitive epilepsy.

## The proposed modifications are shown on top of JVET-AH2006 [1].

txt_descr_purpose indicates the purpose of the text description SEI as specified in Table xx.

Table xx – Definition of txt_descr_purpose

| Value | Interpretation |
| --- | --- |
| 0 | Application defined |
| 1 | Copyright information |
| 2 | AI marking information |
| 3 | General comment information |
| 4 | Content advisory rating information conforming to US. And Canadian Rating Region Tables (RRT) |
| 5 | Tag URI for identifying the bitstream |
| 6 | Photosensitive content information such as strobe light(s) or flashing light(s) |
| 67-255 | Reserved |

Although the txt_descr_purpose equal to 6 is used above, some other value could be used instead.

txt_descr_string[ i ] specifies i-th text description information string whose value is interpreted as specified by the txt_descr_purpose.

When txt_descr_purpose is equal to 0, the interpretation of what information is conveyed in the txt_descr_string is application-defined.

When txt_descr_purpose is equal to 1, the txt_descr_string[ i ] specifies copyright information that pertains to the picture(s) in the persistence scope defined by txt_cancel_flag and txt_persistence_flag.

When txt_descr_purpose is equal to 2, the txt_descr_string[ i ] specifies, when not a null string, AI marking information that pertains to the picture(s) within the persistence scope of this SEI message.

NOTE: When txt_descr_purpose is equal to 2 the string can contain information about the machine-learning-based processing, intended use of the decoded pictures, or other aspects relevant to the associated pictures.

When txt_descr_purpose is equal to 3, the txt_descr_string[ i ] specifies a general text label description that pertains to the picture(s) in the persistence scope defined by txt_cancel_flag and txt_persistence_flag.

When txt_descr_purpose is equal to 4, the txt_descr_string[ i ] specifies content advisory rating information conforming to US. And Canadian Rating Region Tables (RRT) [3] that pertains to the picture(s) in the persistence scope defined by txt_cancel_flag and txt_persistence_flag

When txt_descr_purpose is equal to 5, txt_descr_string[ i ] contains a tag URI with syntax and semantics as specified in IETF RFC 4151 identifying the CLVS.

When txt_descr_purpose is equal to 6, txt_descr_string[ i ] specifies information about photosensitive content such as strobe light(s) or flashing lights, and severity of the photosensitive content.

## Option 2:

Alternatively, it is proposed to define a new SEI message to indicate photosensitive content information such as strobe light(s) or flashing light(s) present in the image or sequence of images. The syntax and semantics for the new photosensitive content information SEI is shown below.

Photosensitive Content information SEI message

## Photosenstive content information SEI message syntax

| photosensitive_content_information( payloadSize ) { | Descriptor |
| --- | --- |
| pci_cancel_flag | u(1) |
| if(!pci_cancel_flag) { |  |
| pci_persistence_flag | u(1) |
| pci_severity_idc | u(2) |
| pci_light_types | u(4) |
| pci_num_descr_strings | u(8) |
| for( i = 0; i <= pci_num_descr_strings; i++ ) { |  |
| pci_descr_string_lang[ i ] | st(v) |
| pci_descr_string[ i ] | st(v) |
| } |  |
| pci_rectangle_info_present_flag | u(1) |
| if(pci_rectangle_info_present_flag) { |  |
| pci_rect_top_left_x | u(16) |
| pci_rect_top_left_y | u(16) |
| pci_rect_width | u(16) |
| pci_rect_height | u(16) |
| } |  |
| } |  |
| } |  |

## Photosenstive content information SEI message semantics

pci_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous photosensitive content information SEI message in output order. pci_cancel_flag equal to 0 indicates that photosensitive content information follows.

pci_persistence_flag specifies the persistence of the photosensitive content information SEI message for the current layer.

pci_persistence_flag equal to 0 specifies that the photosensitive content information SEI message applies to the current decoded picture only.

pci_persistence_flag equal to 1 specifies that the photosensitive content information SEI message applies to the current decoded picture and persists for all subsequent pictures of the current layer in output order until one or more of the following conditions are true:

– A new CLVS of the current layer begins.

– The bitstream ends.

– A picture in the current layer in an AU associated with a photosensitive content information SEI message is output that follows the current picture in output order.

pci_severity_idc indicates the level of severity of the photosensitive content in the associated decoded picture(s). The values defined are as follows: 01 indicates low severity, 10 indicates medium severity, 11 indicates high severity. 00 is reserved.

pci_light_types indicates the type(s) of lights as specified in Table X, that are present in the associated decoded picture(s) and that cause photosensitivity. Regarding Table X, (pci_light_types & bitmask) not equal to 0 indicates that the specified light type is present in the associated decoded pictures and (pci_light_types & bitmask) equal to 0 indicates that the specified light type is not present in the associated decoded pictures. When pci_light_types is equal to 0, the information regarding types of lights is not specified.

Table X – Definition of pci_light_types

| bitMask | Interpretation |
| --- | --- |
| 0x01 | Flashing lights |
| 0x02 | Strobe lights |
| 0x04 | Fireworks |
| 0x08 | Explosion lights |

pci_num_descr_strings indicates the number of entries for pci_descr_string_lang[ i ] and pci_descr_string[ i ] that follow.

pci_descr_string_lang[ i ] specifies the language of the pci_descr_string[ i ]. The language of the pci_descr_string[ i ]  shall be given by a language tag as defined by IETF RFC 5646.

pci_descr_string[ i ] specifies text description information about photosensitive content in the associated decoded picture(s).

pci_rectangle_info_present_flag equal to 0 specifies that the photosensitive content information provided in the SEI message applies to the entire associated decoded picture(s). pci_rectangle_info_present_flag equal to 1 specifies the rectangle syntax elements (pci_rect_top_left_x, pci_rect_top_left_y, pci_rect_width, pci_rect_height) follow. When pci_rectangle_info_present_flag is equal to 1, only the specified rectangle defined by (pci_rect_top_left_x, pci_rect_top_left_y, pci_rect_width, pci_rect_height) in the associated decoded picture(s) include photosensitive content information.

pci_rect_top_left_x and pci_rect_top_left_y respectively specify the luma sample position in horizontal and vertical pixel coordinates indicating the top left corner of the rectangle which includes photosensitive content in the associated decoded picture(s). pci_rect_top_left_x  shall be less than CroppedWidth and pci_rect_top_left_y  shall be less than CroppedHeight.

pci_rect_width specifies the width of the rectangle which includes photosensitive content in the associated decoded picture(s). pci_rect_width shall be less than or equal to (CroppedWidth- pci_rect_top_left_x).

pci_rect_height specifies the height of the rectangle which includes photosensitive content in the associated decoded picture(s). pci_rect_height shall be less than or equal to (CroppedHeight- pci_rect_top_left_y).

# Conclusion

It is proposed to adopt the proposed changes to the next draft of VSEI version 4.

# Patent rights declaration(s)

Sharp Labs of America, Inc., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- JVET-AH2006, J. Boyce, S. McCarthy, J. Chen, S. Deshpande, M. M. Hannuksela, Hendry, G. J. Sullivan, Y.-K. Wang, SEI messages for VSEI version 4 (Draft 2), May 2024

Acknowledgement
These research results were obtained from the commissioned research (No.05101) by National Institute of Information and Communications Technology (NICT), Japan.