| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0062-v1 |
| --- | --- |

| Title: | [AHG9]: Neural network post-filter for tone mapping operations |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | C.H. Demarty F. Aumont L. Blondé E. Reinhard O. Le Meur | Tel: Email: | [claire-helene.demarty, franck.aumont, laurent.blonde, erik.reinhard, olivier.lemeur]@interdigital.com |
| Source: | InterDigital |  |  |

# Abstract

The neural-network post-filter characteristics (NNPFC) SEI message includes the nnpfc_purpose syntax element, aiming to define and to trigger one or more neural-network post-filters. Several purposes have been defined such as picture rate upsampling, bit depth upsampling or colourization.

This contribution proposes to add a new purpose, called Tone Mapping.

To distinguish between tone mappers operating on the color, or on the brightness dimension, it is proposed to add a new sub-purpose to the Tone Mapping operation, related to brightness adaptation.

Assuming that the energy consumption of display devices is correlated to the amount of emitted light [1], the NNPF Tone Mapping purpose, with the Brightness Adaptation sub-purpose, can be used to adapt the energy consumption of display devices. Two different use-cases illustrating the benefits of such new purpose are presented and evaluated.

Beyond the proposed brightness manipulation operation, the proposed purpose also leaves the door open for additional tone-mapping-based post-filters.

# Introduction

At the 34th JVET meeting, it was proposed a new NNPF (Neural network post filter) purpose for Recovering Images from Energy-Aware Images (JVET-AH0126).

The Energy-Aware process (i.e., producing from an original image an image that will consume less when displayed) and its inverse process (i.e., recovering an image close to the original from an energy-reduced image) can be considered as a Tone Mapping operation.

As it was suggested by the experts that NNPF should cover basic image processing operations, it is therefore proposed to add a new NNPF Tone Mapping purpose in the NNPF SEI message [2].

To distinguish between tone mappers operating on the color, or on the brightness dimension, it is further proposed to add a sub-purpose to the Tone Mapping operation, related to brightness adaptation.

Some metadata related to brightness adaptation sub-purpose are also proposed to be added, among which the amount of brightness adaptation and the appropriate display model, as suggested by the experts during the 34th JVET meeting.

Brightness adaptation can serve different goals from enhancing aesthetics to adapting the energy consumption of the content while displayed.

# Proposal

A new NNPF purpose dedicated to Tone Mapping is added. A sub-purpose related to Brightness Adaptation is added to the new Tone Mapping purpose, to indicate that Tone Mapping operation will use brightness adaptation.

The proposed modification and the associated semantics are detailed in Table 1.

| Bitmask | Interpretation |
| --- | --- |
| 0x01 | General visual quality improvement |
| 0x02 | Chroma upsampling (from the 4:2:0 chroma format to the 4:2:2 or 4:4:4 chroma format, or from the 4:2:2 chroma format to the 4:4:4 chroma format) |
| 0x04 | Resolution resampling (increasing or decreasing the width or height) |
| 0x08 | Picture rate upsampling |
| 0x10 | Bit depth upsampling (increasing the luma bit depth or the chroma bit depth) |
| 0x20 | Colourization |
| 0x40 | Temporal extrapolation (i.e., generating one or more future pictures) |
| 0x80 | Spatial extrapolation (i.e., generating content outside of the spatial area of the input pictures) |
| 0x100 | Tone mapping (i.e., modification of the color values, in terms of contrast, color, saturation, luminance, etc.). |

Table 1: Modified list of purposes.

The value of nnpfc_purpose shall be in the range of 0 to 511, inclusive, in bitstreams conforming to this edition of this document. Values of 512 to 65 535, inclusive, for nnpfc_purpose are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this edition of this document.

The SEI message syntax is modified as followed (highlighted in yellow):

| nn_post_filter_characteristics( payloadSize ) { | Descriptor |
| --- | --- |
| nnpfc_purpose | u(16) |
| nnpfc_id | ue(v) |
| nnpfc_base_flag | u(1) |
| nnpfc_mode_idc | ue(v) |
| if( nnpfc_mode_idc = = 1 ) { |  |
| while( !byte_aligned( ) ) |  |
| nnpfc_alignment_zero_bit_a | u(1) |
| nnpfc_tag_uri | st(v) |
| nnpfc_uri | st(v) |
| } |  |
| nnpfc_property_present_flag | u(1) |
| if( nnpfc_property_present_flag ) { |  |
| /* input and output formatting */ |  |
| nnpfc_num_input_pics_minus1 | ue(v) |
| if( nnpfc_num_input_pics_minus1 > 0 ) { |  |
| for( i = 0; i <= nnpfc_num_input_pics_minus1; i++ ) |  |
| nnpfc_input_pic_filtering_flag[ i ] | u(1) |
| nnpfc_absent_input_pic_zero_flag | u(1) |
| } |  |
| if( ChromaUpsamplingFlag ) |  |
| nnpfc_out_sub_c_flag | u(1) |
| if( ColourizationFlag ) |  |
| nnpfc_out_colour_format_idc | u(2) |
| if( ResolutionResamplingFlag ) { |  |
| nnpfc_pic_width_num_minus1 | ue(v) |
| nnpfc_pic_width_denom_minus1 | ue(v) |
| nnpfc_pic_height_num_minus1 | ue(v) |
| nnpfc_pic_height_denom_minus1 | ue(v) |
| } |  |
| if( PictureRateUpsamplingFlag ) |  |
| for( i = 0; i < nnpfc_num_input_pics_minus1; i++ ) |  |
| nnpfc_interpolated_pics[ i ] | ue(v) |
| if( TemporalExtrapolationFlag ) |  |
| nnpfc_extrapolated_pics_minus1 | ue(v) |
| if( SpatialExtrapolationFlag ) { |  |
| nnpfc_spatial_extrapolation_left_offset | ue(v) |
| nnpfc_spatial_extrapolation_right_offset | ue(v) |
| nnpfc_spatial_extrapolation_top_offset | ue(v) |
| nnpfc_spatial_extrapolation_bottom_offset | ue(v) |
| } |  |
| if( ToneMappingFlag ) { |  |
| nnpfc_tone_mapping_subpurpose | u(8) |
| if( TM_BrightnessAdaptationFlag ){ |  |
| nnpfc_ratio_value | u(8) |
| nnpfc_ratio_sign_flag | u(1) |
| nnpfc_display_model | u(4) |
| } |  |
| } |  |
| nnpfc_component_last_flag | u(1) |
| nnpfc_inp_format_idc | ue(v) |
| nnpfc_auxiliary_inp_idc | ue(v) |
| nnpfc_inp_order_idc | ue(v) |
| if( nnpfc_inp_format_idc = = 1 ) { |  |
| if( nnpfc_inp_order_idc != 1 ) |  |
| nnpfc_inp_tensor_luma_bitdepth_minus8 | ue(v) |
| if( nnpfc_inp_order_idc > 0 ) |  |
| nnpfc_inp_tensor_chroma_bitdepth_minus8 | ue(v) |
| } |  |
| nnpfc_out_format_idc | ue(v) |
| nnpfc_out_order_idc | ue(v) |
| if( nnpfc_out_format_idc = = 1 ) { |  |
| if( nnpfc_out_order_idc != 1 ) |  |
| nnpfc_out_tensor_luma_bitdepth_minus8 | ue(v) |
| if( nnpfc_out_order_idc != 0 ) |  |
| nnpfc_out_tensor_chroma_bitdepth_minus8 | ue(v) |
| } |  |
| nnpfc_separate_colour_description_present_flag | u(1) |
| if( nnpfc_separate_colour_description_present_flag ) { |  |
| nnpfc_colour_primaries | u(8) |
| nnpfc_transfer_characteristics | u(8) |
| if( nnpfc_out_format_idc = = 1 ) { |  |
| nnpfc_matrix_coeffs | u(8) |
| nnpfc_full_range_flag | u(1) |
| } |  |
| } |  |
| if( nnpfc_out_order_idc > 0 ) |  |
| nnpfc_chroma_loc_info_present_flag | u(1) |
| if( nnpfc_chroma_loc_info_present_flag ) |  |
| nnpfc_chroma_sample_loc_type_frame | ue(v) |
| nnpfc_overlap | ue(v) |
| nnpfc_constant_patch_size_flag | u(1) |
| if( nnpfc_constant_patch_size_flag ) { |  |
| nnpfc_patch_width_minus1 | ue(v) |
| nnpfc_patch_height_minus1 | ue(v) |
| } else { |  |
| nnpfc_extended_patch_width_cd_delta_minus1 | ue(v) |
| nnpfc_extended_patch_height_cd_delta_minus1 | ue(v) |
| } |  |
| nnpfc_padding_type | ue(v) |
| if( nnpfc_padding_type = = 4 ) { |  |
| if( nnpfc_inp_order_idc != 1 ) |  |
| nnpfc_luma_padding_val | ue(v) |
| if( nnpfc_inp_order_idc != 0 ) { |  |
| nnpfc_cb_padding_val | ue(v) |
| nnpfc_cr_padding_val | ue(v) |
| } |  |
| } |  |
| nnpfc_complexity_info_present_flag | u(1) |
| if( nnpfc_complexity_info_present_flag ) { |  |
| nnpfc_parameter_type_idc | u(2) |
| if( nnpfc_parameter_type_idc != 2 ) |  |
| nnpfc_log2_parameter_bit_length_minus3 | u(2) |
| nnpfc_num_parameters_idc | u(6) |
| nnpfc_num_kmac_operations_idc | ue(v) |
| nnpfc_total_kilobyte_size | ue(v) |
| } |  |
| nnpfc_num_metadata_extension_bits | ue(v) |
| if( nnpfc_num_metadata_extension_bits > 0 ) { |  |
| if( nnpfc_purpose = = 0 ) { |  |
| nnpfc_application_purpose_tag_uri_present_flag | u(1) |
| if( nnpfc_application_purpose_tag_uri_present_flag ) |  |
| nnpfc_application_purpose_tag_uri | st(v) |
| } |  |
| if( SpatialExtrapolationFlag ) |  |
| nnpfc_scan_type_idc | u(2) |
| nnpfc_reserved_metadata_extension | u(v) |
| } |  |
| } |  |
| /* ISO/IEC 15938-17 bitstream */ |  |
| if( nnpfc_mode_idc = = 0 ) { |  |
| while( !byte_aligned( ) ) |  |
| nnpfc_alignment_zero_bit_b | u(1) |
| for( i = 0; more_data_in_payload( ); i++ ) |  |
| nnpfc_payload_byte[ i ] | b(8) |
| } |  |
| } |  |

The variables ChromaUpsamplingFlag, ResolutionResamplingFlag, PictureRateUpsamplingFlag, BitDepthUpsamplingFlag, ColourizationFlag, TemporalExtrapolationFlag, SpatialExtrapolationFlag, and ToneMappingFlag, specifying whether nnpfc_purpose indicates the purpose of the NNPF to include chroma upsampling, resolution resampling, picture rate upsampling, bit depth upsampling, colourization, temporal extrapolation, and tone mapping respectively, are derived as follows:

ChromaUpsamplingFlag = ( ( nnpfc_purpose & 0x02 ) > 0 ) ? 1 : 0
ResolutionResamplingFlag = ( ( nnpfc_purpose & 0x04 ) > 0 ) ? 1 : 0
PictureRateUpsamplingFlag = ( ( nnpfc_purpose & 0x08 ) > 0 ) ? 1 : 0	(75)
BitDepthUpsamplingFlag = ( ( nnpfc_purpose & 0x10 ) > 0 ) ? 1 : 0
ColourizationFlag = ( ( nnpfc_purpose & 0x20 ) > 0 ) ? 1 : 0
TemporalExtrapolationFlag = ( ( nnpfc_purpose & 0x40 ) > 0 ) ? 1 : 0
SpatialExtrapolationFlag = ( ( nnpfc_purpose & 0x80 ) > 0 ) ? 1 : 0

ToneMappingFlag = ( ( nnpfc_purpose & 0x100 ) > 0 ) ? 1 : 0

nnpfc_tone_mapping_subpurpose indicates the sub-purpose of the tone mapping operation as specified in Table 2, where (nnpfc_tone_mapping_subpurpose & bitMask) not equal to 0 indicates the NNPF of purpose Tone Mapping has the sub-purpose associated with the bitMask value in Table 2.

The value of the nnpfc_tone_mapping_subpurpose shall be 1, in bitstreams conforming to this version of this Specification. Values of 2 to 255, inclusive, for nnpfc_tone_mapping_subpurpose are reserved for future use.

| Bitmask | Interpretation |
| --- | --- |
| 0x01 | Brightness adaptation |

Table 2: Semantic of nnpfc_tone_mapping_subpurpose

The variable TM_BrightnessAdaptationFlag specifying whether nnpfc_tone_mapping_subpurpose indicates the sub-purpose of the Tone Mapping NNPF to include brightness adaptation, is derived as follows:

TM_BrightnessAdaptationFlag = ( ( nnpfc_tone_mapping_subpurpose & 0x01 ) > 0 ) ? 1 : 0

nnpfc_ratio_value, when ToneMappingFlag is equal to 1 and TM_BrightnessAdaptationFlag is equal to 1, specifies the expected percentage of brightness adaptation, through brightness decrease or increase (accordingly with the value of the nnpfc_ratio_sign_flag field), that the Tone Mapping post-filter should achieve. The percentage of brightness adaptation is to be considered relatively to the average brightness of the content before applying the NNPF Tone Mapping. The percentage ratio is computed using the following equation:

ratio= nnpfc_ratio_value100

The expected brightness after application of the NNPF Tone Mapping may be derived as follows:

brightnessnew=nnpfc_ratio_sign_flag*ratio*brightnessold

nnpfc_ratio_sign_flag specifies if the value of nnpfc_ratio_value should be considered as negative or positive. If nnpfc_ratio_sign_flag equals 0, the use of the NNPF results in an increase of content brightness. If nnpfc_ratio_sign_flag equals 1, the use of the NNPF results in a decrease in terms of content brightness.

nnpfc_display_model is a bit field mask which indicates the recommended display models on which the tone mapping neural network post filter for brightness adaptation may be used.

| Bit number | Display model |
| --- | --- |
| 0 | Transmissive pixel |
| 1 | Emissive pixel |
| 2..3 | Reserved for future types |

Table 3: Interpretation of the bits of nnpfc_display_model.

For example, nnpfc_display_type_field=112 means that the NNPF can be used for both “Transmissive pixel” and “Emissive pixel” display models.

# Examples of use cases related to the adaptation of the energy consumption at the display side

## Decreasing the brightness for decreasing the energy consumption of display devices

### Use case and test protocol

With the hypothesis that energy consumption of display devices is correlated to the amount of emitted light [1], a NNPF Tone Mapping is used to decrease the brightness of the decoded content, thus decreasing its energy consumption when displayed onscreen.

Figure 1: Use-case related to the reduction of brightness-at the decoder side.

The deep neural network proposed in [4] is used in the following experiment to decrease the brightness of the input content, with the below protocol:

  - nnpfc_ratio_value=10,20,40,60, meaning that the brightness is adjusted by 10%, 20%, 40% and 60%, respectively.

  - nnpfc_ratio_sign_flag=1, indicating that the brightness should be decreased

  - Three sequences 1920×1080, 30 fps, 420, 8 bits:

      - Mitsubishi_Traffic (299 frames)

      - RedKayak_8bit (570 frames)

      - RushFieldCuts_8bit (570 frames)

  - Quality metrics Y-PSNR, MS-SSIM, VMAF

  - Power consumption measurements were made on a LG42C2 screen

### Results

Quality on a frame basis:

Figure 2 illustrates the PSNR_Y, MS-SSIM and VMAF scores on a frame basis. Results indicate that the objective quality decreases with the amount of brightness reduction. According to SSIM quality metric, a decrease of 10 to 20% in brightness does not significantly impact the quality.

Figure 2: Objective quality for different brightness reduction ratio.

Quality vs power reduction:

Figure 3 gives the quality vs power reduction curves for the 3 test sequences and different quality metrics. As expected, the quality as well as the power consumption decreases with the amount of brightness reduction. Interestingly, a power reduction of 10 to 20% can be achieved while the SSIM value is greater than 0.99. A similar observation can be made for VMAF scores. These observations are consistent with those made in the following paper [5].

Figure 3: Quality vs power reduction for nnpfc_ratio_value=10,20,40,60

## Increasing the brightness for recovering the original content

### Use case and protocol

In this use case, it is supposed that the video content has been preprocessed prior to encoding to consume less energy when display onscreen, through some adaptation (decrease) of its brightness. This energy-aware video content, i.e., with reduced brightness, is encoded, transmitted, and decoded.

At the decoder, two options are possible as illustrated by Figure 4:

- the decoded video content with a reduced brightness is displayed: the energy consumption of the display device is lower;

- When the energy constraint at the user side is relaxed (e.g., some extra energy is available), the NNPF Tone Mapping can be used to increase the value of the brightness, to recover a video content close to the original.

Figure 4: Use-case related to the transmission of a brightness-reduced video.

The invertible deep neural network proposed in [3] is used in the following experiment. Such a deep neural network can work in a forward manner, or in a backward manner, thus reverting the forward processing in this case. The forward pass reduces the brightness of the video content while maximizing the quality of experience, while the backward pass allows to recover the original content, while increasing the brightness.

The backward pass is used as NNPF Tone Mapping in the following experiment with the below protocol:

  - nnpfc_ratio_value=10,20,40,60, meaning that the brightness is adjusted by 10%, 20%, 40% and 60%, respectively.

  - nnpfc_ratio_sign_flag=0, indicating that the brightness should be increased

  - Three sequences 1920×1080, 30 fps, 420, 8 bits

      - Mitsubishi_Traffic (299 frames)

      - RedKayak_8bit (570 frames)

      - RushFieldCuts_8bit (570 frames)

  - Quality metrics Y-PSNR, MS-SSIM, VMAF

  - Power consumption measurements were made on a LG42C2 screen

### Results

Quality of the backward pass on a frame basis:

Figure 5 illustrates the PSNR_Y, MS-SSIM and VMAF scores on a frame basis. Results indicate that the post-filter is able to recover the original quality of the content with a Y-PSNR greater than 50dB, a SSIM close to 1 and a VMAF score close to 100.

Figure 5: Recovered quality by using the neural post filter.

# Conclusion

In this contribution, a new NNPFC purpose for Tone Mapping is proposed. A first sub-purpose for Brightness Adaptation, with related metadata, is also proposed.

Two use cases are presented. In a first use case, the brightness of the content is decreased in order to consume less energy when the decoded content is displayed onscreen. In a second use-case, the content is preprocessed prior to the encoding to consume less energy during the rendering stage. Provided that an extra energy is available, a content close to the original in terms of brightness is recovered. The second use case is useful when the constraint of energy consumption is relaxed, or the quality level shall be increased for High-End devices.

This contribution offers not only new service opportunities but is also timely, as the EU Energy Efficiency Directive [6] identifies the Information & Communication Technology (ICT) sector as an energy-hungry sector [7].

It is therefore suggested to include this new purpose in the VSEI specification V4.

# References

[1] 	Nam, Yeon-Oh, Dong-Yoon Choi, and Byung Cheol Song. "Power-constrained contrast enhancement algorithm using multiscale retinex for OLED display." IEEE Transactions on Image Processing 23.8 (2014): 3308-3320.

[2] 	S. McCarthy, T. Chujoh, M. M. Hannuksela, G. J. Sullivan, Y.-K. Wang, "Additional SEI messages for VSEI (Draft 4), " JVET-AD2006, April, 2023.

[3]	Le Meur, Olivier, and Claire-Hélène Demarty. "Invertible Energy-Aware Images. " IEEE Signal Processing Letters (2023).

[4]	Le Meur, Olivier, Claire-Hélène Demarty, and Laurent Blondé. "Deep-learning-based energy aware images." 2023 IEEE International Conference on Image Processing (ICIP). IEEE, 2023.

[5]	Sampaio, Emmanuel, Claire-Hélène Demarty, and Olivier Le Meur. "How to make images less power-hungry: An objective benchmark study." Proceedings of the Second International ACM Green Multimedia Systems Workshop. 2024.

[6] 	Energy Efficiency Directive (europa.eu)

[7]		Digitalization of the energy systems (europa.eu)

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).