| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0073-v1 |
| --- | --- |

| Title: | AHG9: On the handling of a processing chain specified by an SPO SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela, Francesco Cricri, Jill Boyce | Email: | miska.hannuksela@nokia.com francescro.cricri@nokia.com jill.boyce@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

JVET-AH0350 proposed a new subclause within D.11 ("SEI processing order and processing order nesting SEI message") of VVC version 4. Two options were proposed for JVET's consideration:

- Breadth-first: all the pictures are processed by a processing stage before moving on to the next processing stage.

- Depth-first: a picture is processed by all the processing stages, before moving on to the next picture in output order.

JVET's decision on JVET-AH0350 was to document both approaches as TuC text specification to determine whether depth-first has some advantage. JVET meeting notes also state that the maximum delay and picture buffering requirements should be further studied.

This contribution is a follow-up of JVET-AH0350 and proposes the following options:

- Only breadth-first handling of a processing chain:

  - Move the breadth-first handling of a processing chain from the TuC to VSEI v4.

- Conditional use of the breadth-first or the depth-first handling of a processing chain, including the following aspects:

  - Move both the breadth-first and depth-first handlings of a processing chain from the TuC to VSEI v4.

  - Add po_breadth_first_flag in the SEI processing order (SPO) SEI message. When po_breadth_first_flag is equal to 1, the breadth-first handling is used. When po_breadth_first_flag is equal to 0, both the depth-first handling and the depth-first handling may be used.

The authors of this contribution prefer option 2, since the shortcut indication of po_breadth_first_flag equal to 0 is believed to be common. It is asserted that when po_breadth_first_flag is equal to 0, the processing in a decoding system is simplified, since NNPFC SEI messages related to the processing chain do not need to be analyzed.

# Introduction

In JVET's review of aspects 5 and 6 of JVET-AH0121, the following was noted:

It is agreed that the appropriate place to describe the interfaces in the processing chain is the SPO SEI rather than VSEI as was suggested in JVET-AH0047. Proponents of 0047 are asked to carefully check and communicate offline with proponents of 0121 for correctness on items 5 and 6. A sentence should be added that a decoder should operate at most one processing chain at one time.

It was suggested that a more generic handling of the case when an SEI message needs information about the nature of its input (as in case of colour conversion) would be desirable. E.g., if the previous stage has not modified chroma format idc, it needs to be inferred from some earlier stage.

In response to the comments above, JVET-AH0350 proposed a new subclause within D.11 ("SEI processing order and processing order nesting SEI message"). Two options were proposed for JVET's consideration:

- Breadth-first: all the pictures are processed by a processing stage before moving on to the next processing stage.

- Depth-first: a picture is processed by all the processing stages, before moving on to the next picture in output order.

JVET's decision on JVET-AH0350 was to document both approaches as TuC text specification to determine whether depth-first has some advantage. JVET meeting notes also state that the maximum delay and picture buffering requirements should be further studied.

This input document discusses the breadth-first and depth-first handling of the processing chain. Based on the discussion and findings, the contribution proposes options for changes to be taken in VVC v4.

# Discussion

Examples originally presented in JVET-AH0350-v2 are reproduced in Section 5. The examples demonstrate cases where the depth-first handling of a processing chain fail. Therefore, this contribution concludes that the depth-first processing order is not suitable as a generic approach that handles all cases. Consequently, both proposed options include the breadth-first handling of a processing chain.

The specification text for the breadth-first processing order incurs processing delay, since all the pictures are processed by a processing stage before moving on to the next processing stage. It is remarked that the breadth-first handling of the processing chain is used to specify the input pictures to each processing stage unambiguously, and implementations may be optimized for a shorter delay provided that the input pictures are concluded to be the same as those obtained by the breadth-first handling.

It is asserted that an upper bound of the required delay in count of pictures in output order may be concluded from the NNPFC SEI messages applying to the processing chain.

# Overview of the proposal

This contribution proposes the following options:

- Only breadth-first handling of a processing chain:

  - Move the breadth-first handling of a processing chain from the TuC to VSEI v4.

- Conditional use of the breadth-first or the depth-first handling of a processing chain, including the following aspects:

  - Move both the breadth-first and depth-first handlings of a processing chain from the TuC to VSEI v4.

  - Add po_breadth_first_flag in the SPO SEI message. When po_breadth_first_flag is equal to 1, the breadth-first handling is used. When po_breadth_first_flag is equal to 0, both the depth-first handling and the depth-first handling may be used.

It is asserted that the options have the following characteristics:

- Option 1 provides the shortest specification text additions and is asserted to work properly in all cases.

- Option 2 enables decoding systems to determine based on the SPO SEI message whether the depth-first handling of a processing chain can be used, in which case the entire processing chain can be performed picture by picture.

The authors of this contribution prefer option 2, since the shortcut indication of po_breadth_first_flag equal to 0 is believed to be common. It is asserted that when po_breadth_first_flag is equal to 0, the processing in a decoding system is simplified, since NNPFC SEI messages related to the processing chain do not need to be analyzed.

# Proposed specification text

Editorial comment: SpoProcessingList is a list of payloadType values of the SEI messages implying processes. It is assumed to be specified in JVET-AH2005 (VVC v4 draft), which is not available at the time of writing this input document. If this input document is adopted, the name of the variable should be checked at the time of integrating this input document to VVC v4 text.

## Option 1

Add the following clause D.11.3 (originating from TuC, with editorial cleanup):

### D.11.3 Breadth-first handling of a processing chain

Processing chains are alternatives to each other, i.e., at most one processing chain can be chosen to be applied by a decoding system at one time.

A decoding system may choose and apply a processing chain as follows:

–	First, the bitstream is decoded, and the list PoPicList is set to be the list of the cropped decoded pictures in output order that resulted from decoding the bitstream, and a processing chain is chosen.

–	For each of the SEI message types of the chosen processing chain, the following applies in a non-decreasing order of the corresponding po_sei_processing_order[ i ] values:

–	The following applies for each picture picA in PoPicList in output order, when an SEI message associated with the i-th SEI message type persists for picA or a picture for which an NNPF that generated picA was activated by a preceding process in the processing chain:

–	When picA is not a cropped decoded picture, the following exceptions apply for the interpretation of the SEI message:

–	The interface variables for purposes of interpretation of the SEI message are derived from picA instead of the syntax elements indicating properties for the respective cropped decoded picture.

–	The semantics of the SEI message, or the semantics of the SEI message and, when the SEI message is an NNPFA SEI message, the associated NNPFC SEI message, apply to pictures in PoPicList instead of cropped decoded pictures.

–	When the i-th SEI message type is present in SpoProcessingList, the process implied by the SEI message is performed and PoPicList is updated by replacing pictures with the corresponding processed pictures, if any, resulting from the process and inserting the other pictures, if any, resulting from the process into PoPicList so that the output order is obeyed.

## Option 2

Add D.11.3 as in option 1.

Implement the following changes in yellow highlights:

## D.11 SEI processing order and processing order nesting SEI message

### D.11.1 SEI processing order SEI message

#### D.11.1.1 SEI processing order SEI message syntax

| sei_processing_order( payloadSize ) { | Descriptor |
| --- | --- |
| po_id | u(8) |
| po_num_sei_messages_minus2 | u(7 8) |
| po_breadth_first_flag | u(1) |
| for( i = 0, i < po_num_sei_messages_minus2 + 2; i++ ) { |  |
| po_sei_wrapping_flag[ i ] | u(1) |
| po_sei_importance_flag[ i ] | u(1) |
| po_sei_payload_type[ i ] | u(13) |
| po_sei_prefix_flag[ i ] | u(1) |
| po_sei_processing_order[ i ] | u(8) |
| } |  |
| for( i = 0; i < po_num_sei_messages_minus2 + 2; i++ ) |  |
| if( po_sei_prefix_flag[ i ] ) { |  |
| po_num_bits_in_prefix_indication_minus1[ i ] | u(8) |
| for( j = 0; j <= po_num_bits_in_prefix_indication_minus1[ i ]; j++ ) |  |
| po_sei_prefix_data_bit[ i ][ j ] | u(1) |
| while( !byte_aligned( ) ) |  |
| po_byte_alignment_bit_equal_to_one /* equal to 1 */ | f(1) |
| } |  |
| } |  |

#### D.11.1.2 SEI processing order SEI message semantics

…

po_breadth_first_flag equal to 1 specifies that the breadth-first handling of a processing chain specified in clause D.11.3 is applied to determine the pictures that are used for interpreting the semantics of the SEI messages applied as a part of the processing chain specified by this SPO SEI message. po_breadth_first_flag equal to 0 specifies that both the breadth-first handling of a processing chain specified in clause D.11.3 and the depth-first handling of a processing chain specified in clause D.11.4 may be applied to determine the pictures that are used for interpreting the semantics of the SEI messages applied as a part of the processing chain specified by this SPO SEI message.

Add the following clause D.11.4 (originating from TuC, with editorial cleanup):

### D.11.4 Depth-first handling of a processing chain

Processing chains are alternatives to each other, i.e., at most one processing chain can be chosen to be applied by a decoding system at one time.

A decoding system may choose and apply a processing chain as follows:

–	First, the bitstream is decoded, and the list PoPicList is set to be the list of the cropped decoded pictures in output order that resulted from decoding the bitstream, and a processing chain is chosen.

–	The following is repeatedly applied, in output order, for each picture picA in PoPicList for which a set of SEI messages associated with SEI message types in SpoProcessingList of the chosen processing chain persist for picA:

–	The following applies for each SEI message of the set of SEI messages in a non-decreasing order of the corresponding po_sei_processing_order[ i ] values:

–	When the current SEI message is not the first in the set of the SEI messages, the following exceptions apply for the interpretation of the SEI message:

–	The interface variables for purposes of interpretation of the SEI message are derived from the pictures in the updated PoPicList instead of the syntax elements indicating properties for the respective cropped decoded pictures.

–	The semantics of the SEI message, or of the SEI message and, when the SEI message is an NNPFA SEI message, the associated NNPFC SEI message, apply to pictures in PoPicList instead of cropped decoded pictures.

–	The process implied by the SEI message is invoked repeatedly, in output order, for picA and each of the pictures in PoPicList that are, or correspond to, interpolated or extrapolated pictures generated by the application of the process implied by any preceding SEI message, if any, to picA. After each invocation of the process, PoPicList is updated by replacing pictures with the corresponding processed pictures, if any, resulting from the process and inserting the other pictures, if any, resulting from the process into PoPicList so that the output order is obeyed.

# Examples

## Quality enhancement followed by picture rate upsampling

In this example, a processing chain has the following two processing stages:

- Quality enhancement NNPF, which takes three consecutive cropped decoded pictures, in output order, as input and filters the mid-most picture. The mid-most input picture to each quality enhancement NNPF activation has a lower quality than the surrounding input pictures, and the quality of the mid-most input picture is enhanced to be closer to the quality of the surrounding input pictures. The NNPFA SEI messages activating the quality enhancement NNPF are present in picture units 4, 8, 12, and 16.

- Picture rate upsampling NNPF, which takes two consecutive pictures, in output order, as input and generates one intermediate picture as output. The NNPFA SEI messages activating the picture rate upsampling NNPF are present in picture units 2, 4, 6, 8, 10, 12, 14, and 16.

The breadth-first and depth-first handlings of the processing chain are depicted in Figure 1 and Figure 2, respectively. In the depth-first handling, the NNPFA SEI messages that activate the picture rate upsampling in picture units 2, 6, 10, and 14 are processed before the respective quality-enhanced pictures 2, 6, 10, and 14 have been generated. Hence, the breadth-first and depth-first handlings result into different output sequences.

Figure 1 – Breadth-first handling of a processing chain of a quality enhancement NNPF followed by a picture rate upsampling NNPF.

Figure 2 – Depth-first handling of a processing chain of a quality enhancement NNPF followed by a picture rate upsampling NNPF.

## Quality enhancement followed by film grain synthesis

In this example, a processing chain has the following two processing stages:

- Quality enhancement NNPF, which takes three consecutive cropped decoded pictures, in output order, as input and filters the mid-most picture. The mid-most input picture to each quality enhancement NNPF activation has a lower quality than the surrounding input pictures, and the quality of the mid-most input picture is enhanced to be closer to the quality of the surrounding input pictures. The NNPFA SEI messages activating the quality enhancement NNPF are present in picture units 4, 8, 12, and 16.

- Film grain synthesis, which processes each picture individually. The film grain characteristics SEI messages are present in picture units 0, 2, 4, 6, 8, 10, 12, 14, and 16.

The breadth-first and depth-first handlings of the processing chain are depicted in Figure 3 and Figure 4, respectively. It can be observed that in the depth-first handling of pictures 2, 6, 10, and 14, the film grain synthesis is performed prior to the quality enhancement NNPF, conflicting with the processing order defined in the SPO SEI message. For example, when picture 2 is processed, the film grain synthesis is performed with the cropped decoded picture 2 as input. After that, when picture 4 is processed, the NNPFA SEI message that is present in picture unit 4 is processed, resulting into quality-enhanced picture 2.

Figure 3 – Breadth-first handling of a processing chain of a quality enhancement NNPF followed by film grain synthesis.

Figure 4 – Depth-first handling of a processing chain of a quality enhancement NNPF followed by film grain synthesis.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).