| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0076-v1 |
| --- | --- |

| Title: | AHG12: The Modification of BV Candidate List Construction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Taesik Lee Seonjae Kim Byeongju Park Dongho Lee Jeongil Seo Dongsan Jun (Dong-A University) 550 Nakdongdae-ro, Saha-gu, Busan, 49315, KOREA | Tel: Email: | +82-51-200-5593 tslee@donga-ispl.kr sjkim@donga-ispl.kr bjpark@donga-ispl.kr dhlee@donga-ispl.kr jeongilseo@dau.ac.kr dsjun@dau.ac.kr |
| Source: | Dong-A University (DAU) |  |  |

_____________________________

- Abstract

ECM-13.0 uses neighboring candidate blocks in BV candidate list construction, which consist of adjacent spatial candidate blocks and non-adjacent spatial candidate blocks. This contribution is proposed to extend adjacent spatial candidate blocks in the process of BV candidate list construction. Experimental results on top of ECM-13.0 are reported as below:

AI: { -0.01% Y, -0.01% U, 0.01% V, 100.2% EncT, 100.5% DecT} for Overall

{ -0.11% Y, 0.05% U, 0.00% V, 100.6% EncT, 100.4% DecT} for class F

{ -0.08% Y, -0.11% U, -0.10% V, 100.5% EncT, 100.5% DecT} for class TGM

Introduction

In ECM-13.0 [1], the BV candidate list is constructed for DIMD, OBIC, SGPM, and IntraTMP [2]. The BV candidate list is composed of adjacent and non-adjacent spatial candidate blocks. As shown in Figure 1, there are five adjacent spatial candidate blocks and eighteen non-adjacent spatial candidate blocks.

(a) Adjacent Spatial candidate blocks             (b) Non-adjacent spatial candidate blocks

Figure 1. Spatial candidate blocks in BV candidate list construction.

Proposed Method

In this contribution, we propose modified adjacent spatial candidate blocks in the BV candidate list construction. As shown in Figure 2, there are thirteen adjacent spatial candidate blocks and eighteen non-adjacent spatial candidate blocks.

(a) Adjacent Spatial candidate blocks             (b) Non-adjacent spatial candidate blocks

Figure 2. Spatial candidate blocks in the proposed method.

Experimental Results

The table below summarizes the coding efficiency, runtime, and peak memory consumption of the proposed methods under the JVET common test conditions [3].

Table 1. Performance of the proposed method on top of ECM-13.0 (AI)

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | 0.00% | -0.01% | -0.04% | 100.3% | 100.3% | 99.7% | 99.9% |
| Class A2 | -0.01% | -0.06% | -0.01% | 100.0% | 100.4% | 98.9% | 100.0% |
| Class B | 0.00% | 0.00% | 0.01% | 100.3% | 100.6% | 103.0% | 100.0% |
| Class C | 0.00% | 0.02% | 0.09% | 100.1% | 100.4% | 99.5% | 100.0% |
| Class E | -0.03% | -0.02% | 0.00% | 100.2% | 100.6% | 103.2% | 99.9% |
| Overall | -0.01% | -0.01% | 0.01% | 100.2% | 100.5% | 101.0% | 100.0% |
| Class D | 0.01% | 0.06% | -0.19% | 100.4% | 100.3% | 100.1% | 100.0% |
| Class F | -0.11% | 0.05% | 0.00% | 100.6% | 100.4% | 99.5% | 99.9% |
| Class TGM | -0.08% | -0.11% | -0.10% | 100.5% | 100.5% | 101.2% | 100.0% |

Conclusions

The proposed method uses extended adjacent spatial candidate blocks in the BV candidate list construction. Experimental results showed that the proposed method provides 0.01% average luma BD-rate gain in Overall and 0.11% and 0.08% in Class F and TGM, respectively. It is suggested that the proposed method can be studied in the next round of EE2.

References

- ECM-13.0 reference software, https://vcgit.hhi.fraunhofer.de/ecm/ECM.git

- M. Coban et al., “Algorithm description of Enhanced Compression Model 13 (ECM 13)”, JVET-AH-2025, Apr. 2024.

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, Jan. 2022.

Patent rights declaration(s)

DONG-A University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).