| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0097-v1 |
| --- | --- |

| Title: | AHG12: A follow-up CTC modification for encoding runtime reduction under low-delay configuration |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhipin Deng1, Kai Zhang2, Lei Zhao1, Wenbin Yin1, Yang Wang1, Na Zhang1, Li Zhang2 (Bytedance Inc.) R. Utida, S. Puri, I. Marzuki, C. Bonnineau (InterDigital) | Email: | {zhipin.deng, zhangkai.video, lizhang.idm}@bytedance.com Renan.Utida@interdigital.com |
| Source: | Bytedance Inc. InterDigital Inc. |  |  |

_____________________________

# Abstract

This is a follow-up proposal of JVET-AH0180 to reduce the worst-case encoding runtime for LDB. It is asserted that the encoding runtime of the ECM software reaches the worst case on sequences of class TGM. Five tests on SCC sequences under LDB configurations are conducted to investigate the trade-off between the compression efficiency and the encoding runtime, using configurations proposed in JVET-AH0180. The results in a form of {Y, U, V, EncT, DecT} over ECM-13.0 are reported, and the worst-case encoding runtime for each test is also listed:

Test #1 (LDB):

class F     { 0.33%, 0.35%, 0.39%, 93.0%, 99.5% }

class TGM { 0.80%, 0.92%, 0.92%, 92.3%, 99.9% }

65% and 72% encoding time for the worst cases (FlyingGraphic QP 22 and AoV QP 22 respectively)

Test #2 (LDB):

class F     { 0.37%, 0.42%, 0.40%, 91.1%, 99.6% }

class TGM { 0.84%, 0.99%, 0.99%, 90.7%, 99.2% }

60% and 65% encoding time for the worst cases (FlyingGraphic QP 22 and AoV QP22 respectively)

Test #3 (LDB):

class F     { 0.95%, 1.03%, 1.05%, 88.4%, 99.2% }

class TGM { 1.74%, 1.99%, 2.07%, 88.6%, 99.1% }

49% and 53% encoding time for the worst cases (FlyingGraphic QP 22 and AoV QP 22 respectively)

Test #4 (LDB):

class F     { 0.72%, 1.24%, 0.69%, 87.2%, 99.8% }

class TGM { 1.91%, 2.29%, 2.21%, 86.9%, 99.9% }

60% and 65% encoding time for the worst cases (FlyingGraphic QP 22 and AoV QP 22 respectively)

Test #5 (LDB):

class F     { 1.30%, 1.86%, 1.33%, 84.6%, 99.8% }

class TGM { 2.83%, 3.33%, 3.32%, 85.0%, 100.0% }

49% and 53% encoding time for the worst cases (FlyingGraphic QP 22 and AoV QP 22 respectively)

# Introduction

In JVET-AH0180 [1] and JVET-AE0163 [2], a modified maximum MTT hierarchy depth was adopted to reduce the worst-case encoding runtime for sequences in class B under low delay configurations. However, this evaluation does not consider the encoding runtime for sequences of class TGM. It is asserted that the FlyingGraphic of class TGM QP 22 under LDB configurations takes the longest encoding runtime. For example, ECM-13.0 anchor encoder needs 15 days to encode FlyingGraphic of class TGM with QP = 22, and 14 days to encode ArenaOfValor of class F with QP = 22 on top of Intel(R) Xeon(R) Platinum 8336C CPU @ 2.30GHz.

# Proposed method

In this contribution, the per-QP encoding parameter configurations studied in JVET-AH0180 and JVET-AE0163 are extended to SCC sequences. In total five possible configurations are tested.

Test #1:

- MaxMTTHierarchyDepthByTidOverrideByQP: 22 332222

Test #2 (the settings used for classB low delay configurations in ECM-13.0 and VTM-11.0ecm13.0):

- MaxMTTHierarchyDepthByTidOverrideByQP: 22 322222

Test #3:

- MaxMTTHierarchyDepthByTidOverrideByQP: 22 322111

Test #4:

- MaxMTTHierarchyDepthByTidOverrideByQP: 22 322222

- MaxMTTHierarchyDepthByTidOverrideByQP: 27 333222

Test #5:

- MaxMTTHierarchyDepthByTidOverrideByQP: 22 322111

- MaxMTTHierarchyDepthByTidOverrideByQP: 27 333222

# Experimental results

The proposed methods are tested on class F and class TGM, on top of ECM-13.0 and on top of VTM-11.0ecm13.0, respectively, following the common test conditions [3]. The simulation results of class F and class TGM sequences as compared to ECM-13.0 and VTM-11.0ecm13.0 are summarized in Table 1 and 2, respectively.

Table 1: Coding performance of Test #1-#5 over ECM-13.0

|  |  | Low delay B Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Over ECM-13.0 |  |  |  |  |  |  |
|  |  | Y | U | V | EncT | DecT | VmPeak | worstCaseEncT for each class (AoV QP 22 , FlyingGraphic QP22) |
| Test #1 | Class F | 0.33% | 0.35% | 0.39% | 93.0% | 99.5% | 100.2% | 72% |
|  | Class TGM | 0.80% | 0.92% | 0.92% | 92.3% | 99.9% | 100.6% | 65% |
| Test #2 | Class F | 0.37% | 0.42% | 0.40% | 91.1% | 99.6% | 100.3% | 65% |
|  | Class TGM | 0.84% | 0.99% | 0.99% | 90.7% | 99.2% | 100.6% | 60% |
| Test #3 | Class F | 0.95% | 1.03% | 1.05% | 88.4% | 99.2% | 100.4% | 53% |
|  | Class TGM | 1.74% | 1.99% | 2.07% | 88.6% | 99.1% | 100.6% | 49% |
| Test #4 | Class F | 0.72% | 1.24% | 0.69% | 87.2% | 99.8% | 100.0% | 65% |
|  | Class TGM | 1.91% | 2.29% | 2.21% | 86.9% | 99.9% | 100.2% | 60% |
| Test #5 | Class F | 1.30% | 1.86% | 1.33% | 84.6% | 99.8% | 100.1% | 53% |
|  | Class TGM | 2.83% | 3.33% | 3.32% | 85.0% | 100.0% | 100.1% | 49% |

Table 2: Coding performance of Test #1-#5 over VTM-11.0ecm13.0

|  |  | Low delay B Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Over VTM-11.0ecm13.0 |  |  |  |  |  |
|  |  | Y | U | V | EncT | DecT | worstCaseEncT for each class (AoV QP 22 , FlyingGraphic QP22) |
| Test #1 | Class F | 0.28% | 0.34% | 0.43% | 89.9% | 99.3% | 59% |
|  | Class TGM | 0.54% | 0.67% | 0.67% | 88.1% | 98.7% | 57% |
| Test #2 | Class F | 0.29% | 0.35% | 0.41% | 89.7% | 99.1% | 58% |
|  | Class TGM | 0.54% | 0.67% | 0.68% | 87.8% | 98.7% | 55% |
| Test #3 | Class F | 0.71% | 0.82% | 0.98% | 85.7% | 99.2% | 45% |
|  | Class TGM | 1.04% | 1.29% | 1.32% | 83.0% | 99.8% | 41% |
| Test #4 | Class F | 1.16% | 1.35% | 1.62% | 81.7% | 99.8% | 58% |
|  | Class TGM | 2.02% | 2.46% | 2.50% | 76.9% | 98.5% | 55% |
| Test #5 | Class F | 1.59% | 1.83% | 2.22% | 79.2% | 97.6% | 45% |
|  | Class TGM | 2.53% | 3.10% | 3.16% | 72.7% | 98.8% | 41% |

# Conclusions

The proposed method can significantly reduce the worst-case encoding time of ECM with limited compression efficiency change under low delay configurations. It is proposed to adopt Test #2 for CTC modification for the next JVET meetings.

# References

[1] R. Utida, S. Puri, I. Marzuki, C. Bonnineau, “AHG12: Modified CTC for encoding time reduction under low-delay configuration”, JVET-AH0180, Apr. 2024.

[2] S. Puri, C. Bonnineau, F. Le Leannec, T. Poirier, “AHG 12: modified CTC proposal for low-delay configurations”, JVET-AE0163, Jul. 2023.

[3] M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-AF2017, Oct. 2023.

# Patent rights declaration(s)

Bytedance Inc. does not have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

InterDigital Inc. does not have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).