| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0102-v2 |
| --- | --- |

| Title: | Non-EE2: Regression-based SGPM blending |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xinwei Li Ru-Ling Liao Jie Chen Yan Ye | Tel: Email: | {sid.lxw, ruling.lrl, jiechen.cj, yan.ye}@alibaba-inc.com |
| Source: | Alibaba group |  |  |

_____________________________

- Abstract

This contribution proposes to extend the SGPM candidate list with regression-based SGPM candidates. Each regression-based SGPM candidate contains two intra prediction modes and a derived blending matrix. The two intra prediction modes are selected from the primary MPM list and up to 6 block vectors. The matrix-based intra prediction replacement method is also extended to SGPM. It is reported that on top of ECM-13.0, the overall coding performance impact for {Y, U, V, EncT, DecT} is {-0.12%, -0.01%, -0.02%, 103.9%, 102.3%} for AI and {*-0.07%, -0.08%, -0.09%, 101.0%, 100.4%} for RA.

Introduction

SGPM [1] is an intra mode similar to the inter GPM mode, where two intra predictors are used and then blended together according to its partition mode. An SGPM candidate list is built with each entry containing one partition mode and two intra prediction modes as shown in Figure 1. For each partition mode, an intra prediction mode list is derived for each part which contains 3 conventional intra prediction modes and up to 6 block vectors. The SGPM candidate list is reordered based on SAD cost of the template (1 line) and the first 16 candidates with smallest SAD cost are retained. Then, the selected candidate index out of the 16 candidates is signaled.

Figure 1 The SGPM candidate list

A regression-based GPM blending method was proposed for inter GPM [2]. In the regression-based GPM blending method, two integer blending matrices (W0 and W1) are derived from the template (1 line) which can be modelled as an affine linear function of the sample positions (x,y) in the current CU as:

W0x,y=32-W1(x,y)                              (1)

W1x,y=a*x+b*y+c                             (2)

The parameters (a, b, c) are derived from the template using the same solver as the one used for CCCM. A list of candidate pairs is built from the regular GPM candidates and reordered with the template cost. If a pair of candidates is selected, the two candidates are used to predict the current block and blended with the derived blending matrices as:

predx,y=W0x,y*pred0x,y+W1x,y*pred1x,y+16≫5             (3)

Proposed methods

It is proposed to extend the idea of regression-based GPM blending method from inter GPM to SGPM. The SGPM candidate list is further extended with regression-based SGPM candidates as shown in Figure 2. Each regression-based candidate contains two intra prediction modes which are selected from the primary MPM list and up to 6 block vectors of neighbouring blocks. Two integer blending matrices are derived from the template (1 line) for each regression-based SGPM candidate as equations (1) and (2).

Figure 2 The proposed SGPM candidate list

The existing SGPM candidates in ECM-13.0 and the proposed regression-based SGPM candidates are reordered together and the first 16 candidates with smallest SAD cost are retained in the same way as ECM-13.0, so that there is no additional signaling.

If a regression-based SGPM candidate is selected, the two intra prediction modes are used to predict the current block and blended with the derived blending matrix as equation (3). Moreover, the DIMD method is used to derive an intra mode to select the transform kernel. The proposed regression-based SGPM method is only applied to blocks with both width and height greater than 4. At the encoder, up to two RDOs are added to select from the proposed SGPM candidate list.

The matrix-based intra prediction replacement method is also extended to SGPM. Each of the two conventional intra prediction modes in SGPM predition will be replaced by a matrix-based intra prediction mode if the replacement conditions are satisfied. The same replacement conditions as in ECM-13 are used.

Simulation results

The proposed methods are implemented on top of ECM-13.0 and are tested under CTC for enhanced compression tool testing [3]. Table 1 shows the simulation results of the proposed method.

Table 1. Simulation results of proposed method

|  | All Intra Main 10 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |  |  |  |  |
|  | Y | U |  | V |  | EncT |  | DecT |  | EncVmPeak | DecVmPeak |
| Class A1 | -0.12% | 0.00% |  | 0.04% |  | 103.4% |  | 101.3% |  | 100.1% | 100.6% |
| Class A2 | -0.11% | -0.10% |  | 0.02% |  | 102.7% |  | 101.8% |  | 100.2% | 100.7% |
| Class B | -0.11% | -0.02% |  | -0.13% |  | 104.3% |  | 102.7% |  | 103.3% | 101.0% |
| Class C | -0.10% | -0.05% |  | 0.05% |  | 103.9% |  | 102.9% |  | 100.7% | 101.3% |
| Class E | -0.18% | 0.16% |  | 0.00% |  | 104.8% |  | 102.5% |  | 100.6% | 101.2% |
| Overall | -0.12% | -0.01% |  | -0.02% |  | 103.9% |  | 102.3% |  | 101.2% | 101.0% |
| Class D | -0.08% | -0.05% |  | -0.28% |  | 103.9% |  | 103.9% |  | 100.8% | 101.4% |
| Class F | -0.06% | 0.04% |  | 0.07% |  | 102.6% |  | 102.1% |  | 100.6% | 101.3% |
| Class TGM | -0.02% | -0.02% |  | -0.02% |  | 102.7% |  | 102.6% |  | 100.4% | 101.1% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |  |
|  | Over ECM-13.0 |  |  |  |  |  |  |  |  |  |  |
|  | Y |  | U |  | V |  | EncT |  | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.09% |  | -0.20% |  | -0.20% |  | 100.8% |  | 100.8% | 100.2% | 100.2% |
| Class A2 | *-0.05% |  | -0.21% |  | -0.04% |  | 100.5% |  | 100.1% | 99.7% | 100.4% |
| Class B | -0.07% |  | 0.06% |  | -0.12% |  | 101.3% |  | 100.5% | 100.5% | 100.9% |
| Class C | -0.07% |  | -0.06% |  | 0.01% |  | 101.0% |  | 100.4% | 100.9% | 101.2% |
| Class E |  |  |  |  |  |  |  |  |  |  |  |
| Overall | *-0.07% |  | -0.08% |  | -0.09% |  | 101.0% |  | 100.4% | 100.4% | 100.7% |
| Class D | 0.03% |  | -0.04% |  | 0.01% |  | 100.3% |  | 100.4% | 100.7% | 101.8% |
| Class F | -0.02% |  | 0.26% |  | 0.24% |  | 101.2% |  | 101.0% | 100.2% | 101.7% |
| Class TGM | -0.12% |  | -0.09% |  | 0.04% |  | 100.8% |  | 100.1% | 101.3% | 101.1% |

Conclusion

This contribution proposes a regression-based SGPM method. It is reported that -0.12% BD-rate reduction can be achieved in all intra condition. It is recommended to further study this proposal in the next round of EE2.

References

- F. Wang, Y. Yu, H. Yu, D. Wang, A. Natesan, J. N. Shingala, J. R. Arumugam, V. Valvaiker, T. Lu, F. Pu, P. Yin, S. McCarthy, K. Naser, Y. Chen, A. Robert and K. Reuzé, “EE2-1.6: Combination of spatial GPM tests”, JVET-AB0155, Oct. 2022.

- P. Bordes, K. Reuzé, F. Galpin, F. Urban, K. Naser, F. Le Leannec, E. Francois, “EE2-2.11: Regression-based GPM blending (tests a,b,c)”, JVET-AG0112, Jan. 2024.

- M. Karczewicz and Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-AF2017, Oct. 2023.

Patent rights declaration(s)

Alibaba group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).