| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0107-v2 |
| --- | --- |

| Title: | EE1-1.5: VLOP using LOP3 architecture |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Du Liu, Jacob Ström, Mitra Damghanian, Per Wennersten (Ericsson) Dmytro Rusanovskyy, Yun Li, Marta Karczewicz (Qualcomm) | Email: | {du.liu, jacob.strom, mitra.damghanian, per.wennersten}@ericsson.com dmytror@qti.qualcomm.com yli30@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Ericsson, Qualcomm |  |  |

_____________________________

# Abstract

NN filter for Very Low Operational Point (VLOP), targeting complexity of 5kMAC/pixel level, was proposed in JVET-AH0051 and adopted to NNVC. This contribution reports the results of EE1-1.5, which adapt the LOP3 architecture (17kMAC/pixel) to VLOP. Complexity reduction was achieved by adjusting the numbers of channels and backbone blocks. The proposed VLOP model has a reported complexity of 5.16 kMAC/pixel and 60.7k parameters. Stage 3 training of the proposed model over LOP dataset was performed independently by its proponents. Independently trained models performed within a relatively narrow margin (<0.1% of luma BD-rate change) to each other, which indicates on successful joint training. The performance of the int16 model (including NNIntra) compared to the NNVC-9.1 VLOP (NNIntra+VLOP) is reported below:

RA  	-0.22% (Y), -1.63% (U), -0.75% (V), 
AI 		-0.07% (Y), -1.43% (U), -0.88% (V), 
LDB 	-0.08% (Y), -3.77% (U). -1.82% (V).

It is proposed to adopt the method into NNVC.

# Introduction

NN filter for Very Low Operational Point (VLOP), targeting complexity of 5kMAC/pixel level, was proposed in JVET-AH0051 and adopted to NNVC. Adopted VLOP filter is based on LOP2 architecture and feature the reduced complexity transition block. The current VLOP model has a complexity of 5.18 kMAC/pixel and 16k parameters [1]. The latest LOP model was tested and adopted into NNVC as LOP3 [2]. The architecture of LOP3 is shown in Fig. 1. LOP3 backbone operates at the reduced spatial resolution, comparing to input data results, and utilizes 2x2 DCT transform over pixels and supplementary data to exchange spatial resolution to the number channels at the headblock.

Fig. 1. The architecture of LOP3.

# Proposed model for VLOP

In this contribution, we use the same architecture as LOP3 and modify the number of channels and backbones blocks to meet the VLOP complexity.

The proposed parameters compared to the current VLOP are given below. The notation “4à8” means that the VLOP model proposed in JVET-AH0051 had the value 4, and this has changed to 8 in the architecture proposed in this contribution.

Head block:

- d1: 4à8

- d2: 2à4

- d3: 1à2

- d4: 1

- d5: 1

- d6 (=C): 16à64

Backbone block:

- Luma

  - NY: 8à6

  - CY: 16à32

  - C1Y: 32à96

  - C21Y: 16à32

- Chroma

  - NUV: 1

  - CUV: 16

  - C1UV: 16à32

  - C21UV: 16

Note that transformed inputs are used, and before splitting into luma and chroma branches, the channel size is CY + CUV.

The complexity of the proposed VLOP model is 5.16 kMAC/pixel, and the number of parameters is 60686. As a comparison, the current VLOP has a complexity of 5.18 kMAC/pixel and 16005 parameters.

# Training and inference information

Training of the model is performed over LOP2 dataset only for stage 3. Training follows the LOP3 training hyperparameters: luma:chroma weighting 12:2, batch size 64, and milestones 41, 51, 55. The only difference is the learning rate, and in this training 4e-4 is used.

| Table 1. Network Information for the model in EE1-1.5 in training. |  |  |
| --- | --- | --- |
| Network Information in Training Stage |  |  |
| Mandatory | GPU Type | GPU: NVIDIA A100-PCIE-40GB |
|  | Framework: | PyTorch v2.1 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | Stage3: 60 |
|  | Batch size: | Stage3: 64 |
|  | Loss function: | L1 and MSE |
|  | Training time: | Stage3: 4 days |
|  | Training data information: | DIV2K, BVI-DVC, TVD |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | The LOP stage 3 dataset |
| Optional |  |  |
|  | Number of iterations |  |
|  | Patch size | 144x144 |
|  | Learning rate: | Stage3: 4e-4, milestones [41, 51, 55] |
|  | Optimizer: | ADAM |
|  | Preprocessing: |  |
|  | Mini-batch selection process: |  |
|  | Other information: |  |

| Table 2. Network Information for the model in inference. |  |  |
| --- | --- | --- |
| Network Information in Inference Stage |  |  |
| Mandatory | HW environment: |  |
|  | GPU Type | N/A |
|  | Framework: | SADL |
|  | Number of GPUs per Task | 0 |
|  | Total Parameter Number | 60686 |
|  | Parameter Precision (Bits) | fp32, int16 |
|  | Memory Parameter (MB) | fp32: 0.24MB, int16: 0.12MB |
|  | Multiplay Accumulate (MAC) | 5.16 kMAC/pixel |
| Optional |  |  |
|  | Total Conv. Layers |  |
|  | Total FC Layers |  |
|  | Total Memory (MB) |  |
|  | Batch size: | 1 |
|  | Patch size | 128x128, 256x256 |
|  | Changes to network configuration or weights required to generate rate points |  |
|  | Peak Memory Usage (Total) |  |
|  | Peak Memory Usage (per Model) |  |
|  | Border handling |  |
|  | Other information: |  |

# Results

The proposed model is tested on top of NNVC-9.1. Two independent trained models have been trained. The best performing model was selected for inclusion. The results are in comparison to the NNVC VLOP CTC [3]. The tested model is int16, with assessment of the quantization’s impact are being provided for information. The encoding time and decoding time is not reliable since they are not run on the same machine.

Compared to VLOP as proposed in JVET-AH0051 evaluated in NNVC-9.1.

|  | Random access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-9.1 VLOP (NNIntra+VLOP) |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -0.90% | -1.83% | -0.38% | 89% | 93% |
| Class A2 | -0.33% | -1.58% | -0.97% | 100% | 94% |
| Class B | -0.12% | -2.08% | -0.84% | 121% | 113% |
| Class C | 0.26% | -0.94% | -0.73% | 125% | 114% |
| Class E |  |  |  |  |  |
| Overall | -0.22% | -1.63% | -0.75% | 110% | 105% |
| Class D | 0.73% | 0.80% | 0.54% | 101% | 95% |
| Class F | 0.14% | 0.69% | 0.58% | 126% | 118% |
|  | All Intra |  |  |  |  |
|  | BD-rate Over NNVC-9.1 VLOP (NNIntra+VLOP) |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -0.63% | -1.46% | -0.40% | 114% | 104% |
| Class A2 | -0.15% | -1.13% | -0.76% | 117% | 116% |
| Class B | -0.04% | -1.68% | -0.66% | 112% | 113% |
| Class C | 0.30% | -0.55% | -0.10% | 129% | 124% |
| Class E | 0.01% | -2.48% | -2.90% | 128% | 115% |
| Overall | -0.07% | -1.43% | -0.88% | 120% | 114% |
| Class D | 0.60% | 0.42% | 0.78% | 133% | 122% |
| Class F | 0.30% | 0.64% | 0.72% | 120% | 119% |

|  | Low delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-9.1 VLOP (NNIntra+VLOP) |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.28% | -4.07% | -1.17% | 100% | 94% |
| Class C | 0.16% | -3.62% | -2.36% | 110% | 103% |
| Class E | -0.07% | -3.46% | -2.20% | 99% | 108% |
| Overall | -0.08% | -3.77% | -1.82% | 103% | 101% |
| Class D | 0.50% | -3.07% | -2.08% | 102% | 94% |
| Class F | 0.20% | -0.72% | -0.10% | 105% | 104% |

Compared to NNVC-9.1 VTM without NN-loopfilters and without NN-intra:

|  | Random access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -6.95% | -6.46% | -5.90% | 98% | 2699% |
| Class A2 | -5.55% | -5.96% | -4.34% | 100% | 2576% |
| Class B | -5.17% | -7.86% | -6.53% | 99% | 2628% |
| Class C | -4.83% | -6.91% | -6.21% | 103% | 2496% |
| Class E |  |  |  |  |  |
| Overall | -5.51% | -6.95% | -5.88% | 100% | 2596% |
| Class D | -4.71% | -5.12% | -5.68% | 104% | 2354% |
| Class F | -2.72% | -3.58% | -2.94% | 108% | 1180% |
|  | All Intra |  |  |  |  |
|  | BD-rate Over VTM |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -8.07% | -9.32% | -8.62% | 154% | 2244% |
| Class A2 | -6.39% | -8.33% | -6.90% | 152% | 2104% |
| Class B | -6.52% | -9.19% | -8.83% | 149% | 2027% |
| Class C | -6.09% | -8.33% | -7.99% | 149% | 1708% |
| Class E | -9.26% | -10.72% | -10.47% | 153% | 2259% |
| Overall | -7.12% | -9.13% | -8.56% | 151% | 2033% |
| Class D | -5.86% | -7.20% | -7.57% | 151% | 1759% |
| Class F | -4.11% | -5.62% | -5.31% | 114% | 1622% |

|  | Low delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -3.93% | -8.68% | -4.37% | 99% | 2441% |
| Class C | -3.58% | -7.64% | -4.98% | 100% | 2306% |
| Class E | -3.79% | -2.89% | -2.61% | 104% | 2474% |
| Overall | -3.78% | -6.89% | -4.13% | 101% | 2403% |
| Class D | -3.99% | -5.20% | -2.88% | 99% | 2234% |
| Class F | -1.70% | -2.04% | -0.96% | 107% | 1174% |

Impact of the quantization on the resulting model is shown in Table below.

| Int16 vs float32 | AI |  |  | RA |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |
| Class C | 0.05% | 0.00% | -0.02% | 0.05% | -0.16% | -0.03% |
| Class E | 0.01% | 0.02% | -0.08% |  |  |  |
| Overall |  |  |  |  |  |  |
| Class D | 0.10% | -0.03% | 0.00% | 0.09% | 0.06% | 0.10% |

Compared to NNVC-9.1 CTC which includes LOP3 and NN-intra:

|  | Random access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-9.1 LOP3 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | 2.05% | 6.34% | 7.65% | 77% | 41% |
| Class A2 | 2.18% | 7.54% | 4.13% | 81% | 40% |
| Class B | 1.87% | 7.51% | 6.44% | 86% | 42% |
| Class C | 1.83% | 7.74% | 6.61% | 113% | 45% |
| Class E |  |  |  |  |  |
| Overall | 1.96% | 7.34% | 6.27% | 90% | 42% |
| Class D | 2.15% | 7.70% | 6.32% | 128% | 52% |
| Class F | 1.17% | 4.50% | 3.42% | 112% | 49% |
|  | All Intra |  |  |  |  |
|  | BD-rate Over NNVC-9.1 LOP3 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | 1.29% | 5.90% | 7.52% | 85% | 43% |
| Class A2 | 1.24% | 6.34% | 4.32% | 84% | 48% |
| Class B | 1.20% | 5.79% | 6.51% | 82% | 47% |
| Class C | 1.48% | 6.11% | 6.10% | 90% | 54% |
| Class E | 1.92% | 6.40% | 7.08% | 87% | 49% |
| Overall | 1.40% | 6.07% | 6.32% | 85% | 48% |
| Class D | 1.52% | 5.19% | 5.55% | 98% | 58% |
| Class F | 1.18% | 4.49% | 3.79% | 83% | 47% |

|  | Low delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-9.1 LOP3 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 1.66% | 1.57% | 3.89% | 86% | 41% |
| Class C | 1.84% | 4.84% | 4.57% | 88% | 41% |
| Class E | 1.98% | 1.55% | 0.65% | 87% | 44% |
| Overall | 1.80% | 2.65% | 3.31% | 87% | 41% |
| Class D | 2.20% | 5.51% | 5.40% | 94% | 41% |
| Class F | 1.40% | 4.55% | 1.33% | 86% | 40% |

Simulation results achieved with Candidate 2 model are shown below. Independently trained Candidate 2 reportedly performs within a relatively narrow margin (<0.1% of luma BD-rate change) from a Candidate 1 model.

Comparative performance of the Candidate 2 model with nnIntra ON vs NNVC9.1-VLLOP is shown below.

|  | Random access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-9.1 VLOP (NNIntra+VLOP) |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -0.97% | -0.24% | 0.35% | 97% | 88% |
| Class A2 | -0.20% | -0.57% | -0.04% | 95% | 87% |
| Class B | -0.02% | -0.43% | 0.19% | 97% | 87% |
| Class C | 0.44% | 0.19% | 0.31% | 98% | 87% |
| Class E |  |  |  |  |  |
| Overall | -0.12% | -0.26% | 0.21% | 97% | 87% |
| Class D | 1.01% | 0.68% | 1.39% | 97% | 82% |
| Class F | 0.06% | 0.35% | 0.34% | 96% | 97% |
|  | All Intra |  |  |  |  |
|  | BD-rate Over NNVC-9.1 VLOP (NNIntra+VLOP) |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -0.66% | -0.31% | 0.14% | 97% | 87% |
| Class A2 | -0.15% | -0.49% | -0.07% | 98% | 87% |
| Class B | -0.05% | -0.63% | -0.17% | 98% | 87% |
| Class C | 0.35% | -0.10% | 0.54% | 97% | 85% |
| Class E | 0.07% | -1.75% | -2.12% | 94% | 87% |
| Overall | -0.06% | -0.62% | -0.27% | 97% | 87% |
| Class D | 0.64% | 0.32% | 1.34% | 94% | 85% |
| Class F | 0.24% | 0.24% | 0.47% | 98% | 85% |

Comparative performance of the Candidate 2 model with nnIntra ON vs NNVC9.1-VTM is shown below.

|  | Random access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -7.02% | -4.97% | -5.20% | 114% | 2804% |
| Class A2 | -5.43% | -5.02% | -3.44% | 111% | 2628% |
| Class B | -5.07% | -6.34% | -5.57% | 115% | 2616% |
| Class C | -4.65% | -5.88% | -5.24% | 116% | 2446% |
| Class E |  |  |  |  |  |
| Overall | -5.42% | -5.68% | -4.98% | 114% | 2608% |
| Class D | -4.44% | -5.24% | -4.91% | 113% | 2332% |
| Class F | -2.79% | -3.89% | -3.18% | 115% | 1141% |
|  | All Intra |  |  |  |  |
|  | BD-rate Over VTM |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -8.10% | -8.27% | -8.13% | 154% | 2580% |
| Class A2 | -6.40% | -7.74% | -6.24% | 161% | 2139% |
| Class B | -6.52% | -8.23% | -8.38% | 157% | 2228% |
| Class C | -6.05% | -7.94% | -7.41% | 155% | 1744% |
| Class E | -9.21% | -10.05% | -9.75% | 148% | 2469% |
| Overall | -7.11% | -8.39% | -8.00% | 155% | 2185% |
| Class D | -5.81% | -7.29% | -7.09% | 144% | 1651% |
| Class F | -4.17% | -5.99% | -5.53% | 132% | 1561% |

# Conclusions

This contribution reports the results of the study of EE1-1.5, which tested VLOP using the LOP3 architecture. The complexity of the proposed model is 5.16 kMAC/pixel, and the number of parameters is 60.7k. The performance compared to NNVC-9.1 anchor shows a BD-rate impact of (Y/U/V) {-0.22%, -1.63%, -0.75%} for RA and {-0.07%, -1.43%, -0.88%} for AI with the int16 model. It is proposed to adopt the model into NNVC.

# References

[1] EE1-5: Study of the NN architecture at Very Low Operational Point, JVET-AH0051, D. Rusanovskyy, Y. Li, M. Karczewicz (Qualcomm), J. Li, Y. Li, C. Lin, K. Zhang, L. Zhang (Bytedance), Z. Xie, Y. Yu, H. Yu, D. Wang (OPPO), Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 1417–24 April 2024.

[2] EE1-1.2: Joint LOP model with inputs transformed, JVET-AH0080, D. Liu, J. Ström, M. Damghanian, P. Wennersten, D. Rusanovskyy, Y. Li, M. Karczewicz, T. Shao, P. Yin, S. McCarthy, J. N. Shingala, A. Shyam, A. Suneja, S. P. Badya, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 17–24 April 2024.

[3] Common Test Conditions and evaluation procedures for neural network-based video coding technology, E. Alshina, R.-L. Liao, S. Liu, A. Segall, JVET-AF2016, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, 32nd Meeting, Hannover, DE, 13–20 October 2023.

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).