| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0109-v2 |
| --- | --- |

| Title: | Non-EE2: A dual merge mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ru-Ling Liao Jie Chen Yan Ye Xinwei Li | Tel: Email: | {ruling.lrl, jiechen.cj, yan.ye, sid.lxw}@alibaba-inc.com |
| Source: | Alibaba group |  |  |

_____________________________

# Abstract

This contribution proposes a dual merge mode. In this mode, a dual merge candidate list is constructed. Each dual merge candidate in the list has two motion information which is derived from previous coded blocks, and each motion information can be either uni- or bi-predicted motion. The prediction for this mode is generated by averaging the two predictions which are separately predicted using the two motion information. It is reported that on top of ECM-13.0, the overall coding performance impact for {Y, U, V, EncT, DecT} is {-0.04%, -0.01%, -0.06%, 102.8%, 105.0%} for RA and {-0.07%, 0.01%, -0.31%, 101.8%, 101.2%} for LDB.

# Introduction

In ECM, multiple merge modes are supported including subblock merge mode, geometric partition mode, template matching, adaptive decoder-side motion vector refinement, merge with MVD and regular merge mode. In these merge modes, only one motion information, which comprises inter prediction direction, motion vectors, reference picture indices, BCW index, LIC parameters and MHP information, is inherited from previous coded blocks. To improve the prediction efficiency, a dual merge mode which inherits two motion information from previous coded blocks is proposed.

# Proposed methods

For a CU coded in merge mode, a dual merge flag is signaled after the MMVD flag to indicate whether the proposed dual merge mode is used to predict the CU. When the CU is coded in dual merge mode, a dual merge list is constructed as follows:

- derive dual merge candidates from spatial, temporal and non-adjacent spatial neighboring blocks if the neighboring blocks are coded in the dual merge mode;

- if the dual merge list is not full, construct a regular merge list using a process similar to that of the regular merge mode, and candidates in the regular merge list are paired and added to the dual merge list;

- perform ARMC to dual merge candidates.

Each dual merge candidate in the dual merge list has two motion information, and each motion information can be either uni- or bi-predicted motion information. An index is signaled to indicate which one of dual merge candidates is selected. The two selected motion information are used to generate two prediction blocks using the same motion compensation process as normal inter modes. That is, the two selected motion information are separately refined by the DMVR if the corresponding conditions are satisfied. Then, motion compensation including BCW, LIC, BDOF and OBMC processes is performed. Then, the two prediction blocks are averaged to generate the final prediction for the CU. An illustration of prediction process for the dual merge mode is shown in Figure 1. It is noted that the MHP is not applied to the dual merge mode. Therefore, up to four motions are used in the dual merge mode.

To store the two motion information for the dual merge coded blocks, the size of motion storage buffer is doubled. When deriving merge candidates from a dual merge coded block for other merge modes, the two motion information are treated as two separate merge candidates. In addition, both motion for two GPM partitions are stored for the blending area of GPM mode.

Figure 1. An illustration of prediction process for the dual merge mode.

# Simulation results

The proposed dual merge mode is implemented on top of ECM-13.0 and tested under CTC for enhanced compression tool testing. The simulation result is shown in Table 1. Incomplete rate points are replaced with anchor for Class A1, Class A2 in RA and Class B in LDB.

Table 1. Simulation results of proposed dual merge mode

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1* | -0.05% | -0.14% | -0.21% | 102.8% | 107.9% | 101.8% | 102.2% |
| Class A2* | -0.01% | -0.06% | 0.02% | 103.2% | 102.6% | 101.8% | 102.0% |
| Class B | -0.06% | 0.07% | -0.08% | 103.0% | 105.5% | 102.0% | 101.7% |
| Class C | -0.03% | 0.01% | 0.01% | 102.2% | 103.9% | 100.9% | 100.7% |
| Class E |  |  |  |  |  |  |  |
| Overall* | -0.04% | -0.01% | -0.06% | 102.8% | 105.0% | 101.6% | 101.6% |
| Class D | 0.03% | 0.39% | 0.20% | 101.7% | 103.6% | 100.7% | 100.4% |
| Class F |  |  |  |  |  |  |  |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B* | -0.02% | 0.08% | -0.41% | 101.5% | 100.2% | 100.8% | 101.3% |
| Class C | -0.15% | 0.00% | -0.39% | 101.1% | 102.5% | 99.7% | 100.7% |
| Class E | -0.06% | -0.10% | -0.06% | 103.3% | 101.0% | 100.0% | 101.2% |
| Overall* | -0.07% | 0.01% | -0.31% | 101.8% | 101.2% | 100.2% | 101.1% |
| Class D | -0.04% | 0.35% | -0.23% | 101.7% | 100.9% | 100.7% | 100.4% |
| Class F |  |  |  |  |  |  |  |

# Conclusion

This contribution proposes a dual merge mode which generates prediction using two motion information derived from previous coded block. It is reported that 0.04% and 0.07% coding gain are achieved in RA and LDB conditions, respectively. It is recommended to further study this proposal in the next round of EE2.

# Patent rights declaration(s)

Alibaba group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).