| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0114-v2 |
| --- | --- |

| Title: | Non-EE2: SATD-based reordering |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yang Wang1, Kai Zhang2, Wenbin Yin1, Zhipin Deng1, Na Zhang1, Lei Zhao1, Li Zhang2 1Fangheng Building, No.27, 3rd Ring West Rd, Haidian District, 100098, Beijing, China 28910 University Center Lane, San Diego, CA 92122, USA | Tel: Email: | wangyang.cs@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

- Abstract

This contribution presents to use sum of absolute transformed difference (SATD) cost for reordering in two aspects. In Aspect #1, SATD-based reordering is used for intra coding tools and intra block copy. In Aspect #2, SATD-based reordering is used for inter coding tools. On top of ECM-13.0, the simulation results are reported as below:

Aspect #1:

AI: {-0.02%, 0.00%, -0.03%; 100.2%, 99.8%};

RA: {-0.01%, 0.00%, -0.05%; 98.0%, 99.8%}.

Aspect #2:

RA: {-0.03%, 0.03%, -0.08%; 100.7%, 100.4%};

*LDB: {-0.07%, 0.04%, -0.24%; 99.3%, 100.8%}.

*One QP point for Class B is copied from the anchor.

Introduction

In ECM-13.0, reordering method is used for coding tools including ARMC, Template-based multiple reference line intra prediction (TMRL), Extrapolation filter-based intra prediction (EIP), Template matching based BCW index derivation for merge mode, etc. Sum of absolute difference (SAD) is calculated as the template cost in these reordering methods.

Proposed method

In this contribution, SATD is calculated as the template cost in the reordering process. In Aspect #1, SATD-based reordering is used for intra and IBC coding tools, such as reordering in TMRL and EIP, and ARMC for IBC. In Aspect #2, SATD-based reordering is used for inter coding tools, such as ARMC.

Simulation results

The proposed method is implemented on top of ECM-13.0 [1]. Simulations are performed following the common test conditions of EE2 [2]. The simulation results of Aspect #1 and Aspect #2 compared to ECM-13.0 are shown in Table 1 and Table 2.

Table 1: Coding performance of Aspect #1 compared to ECM-13.0

|  | AI |  |  |  |  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.03% | 0.02% | 101.5% | 100.1% | -0.03% | 0.05% | -0.18% | 100.3% | 99.4% |
| Class A2 | -0.03% | -0.02% | 0.02% | 100.5% | 100.0% | 0.03% | 0.02% | -0.01% | 100.1% | 100.0% |
| Class B | -0.02% | -0.01% | -0.14% | 99.9% | 99.7% | -0.02% | -0.05% | -0.23% | 96.0% | 99.9% |
| Class C | 0.00% | 0.01% | 0.09% | 99.4% | 99.7% | -0.02% | 0.02% | 0.24% | 97.3% | 99.8% |
| Class E | -0.04% | 0.09% | -0.09% | 99.9% | 99.8% |  |  |  |  |  |
| Overall | -0.02% | 0.00% | -0.03% | 100.2% | 99.8% | -0.01% | 0.00% | -0.05% | 98.0% | 99.8% |
| Class D | 0.00% | 0.15% | -0.08% | 100.0% | 99.8% | -0.01% | -0.21% | 0.08% | 99.1% | 99.3% |
| Class F | -0.08% | 0.00% | 0.02% | 100.7% | 100.8% | 0.04% | 0.15% | -0.13% | 96.9% | 100.0% |

Table 2: Coding performance of Aspect #2 compared to ECM-13.0

|  | RA |  |  |  |  | LDB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | -0.01% | -0.23% | 101.3% | 100.5% |  |  |  |  |  |
| Class A2 | -0.07% | -0.02% | -0.06% | 100.6% | 100.5% |  |  |  |  |  |
| Class B | -0.03% | 0.04% | -0.17% | 100.2% | 100.2% |  |  |  |  |  |
| Class C | 0.01% | 0.10% | 0.11% | 100.7% | 100.4% | -0.05% | -0.20% | -0.46% | 100.5% | 100.7% |
| Class E |  |  |  |  |  | -0.04% | 0.90% | -0.12% | 96.0% | 99.7% |
| Overall | -0.03% | 0.03% | -0.08% | 100.7% | 100.4% |  |  |  |  |  |
| Class D | 0.07% | 0.23% | 0.11% | 92.2% | 99.8% | -0.11% | -0.43% | -0.18% | 96.3% | 99.9% |
| Class F | -0.01% | 0.06% | 0.01% | 93.3% | 100.7% |  |  |  |  |  |

Conclusions

In this contribution, SATD cost is proposed to be used in reordering. It is recommended to study the proposed method in EE2.

Reference

- ECM-13.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-13.0.

- M. Karczewicz and Y. Ye, “Common test conditions and evaluation procedures for enhanced compression tool testing,” Document of Joint Video Experts Team, JVET-AF2017, Hannover, DE, 13–20 October 2023.

Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).