| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0118-v1 |
| --- | --- |

| Title: | AHG 12: Alternative affine motion model for affine DMVR |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | K. Andersson | Tel: Email: | +46730371390 kenneth.r.andersson@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

Affine motion compensation is a useful coding tool to represent non-translational motion. Affine motion compensation is used together with several other tools in ECM such as GPM, MMVD and DMVR. Common to the usage of affine models in ECM is that they can be represented by 2 or 4 non-translational parameters in addition to the two translational parameters. The representation of the 4-parameter affine model is based on motion vectors located at two control points at the corners in the top of the current block. The 6-parameter model is also represented by the two control point motion vectors at the top of the current block but also a control point motion vector located at the bottom-left corner of the current block. However, ECM lacks a representation based on two control point motion vectors located at the corners at the left side of the current block. This may be suboptimal if the current block’s motion model follows those two closer than the two control point motion vectors located at the top of the current block. In this contribution the initial motion vectors of affine DMVR are derived from control point motion vectors on the left side of the current block when the width of the current block is higher than the height of the current block and the affine type is equal to the 4-parameter model. This approach is also applied on refined control point motion vectors. This can create a motion model that is more different from the 6-parameter model than using control point motion vectors from the top side of the current block.

In comparison to ECM-13 we can get a small but consistent objective benefit for all classes and on average -0.02%. The decoding time is 99.9%. Encoding time only tested for class C and D and is 100.1%

It is suggested to further study the method in an EE.

# Introduction

Affine motion compensation is a useful tool in ECM and it has been combined with several other tools such as GPM, MMVD and DMVR. In ECM the motion model is defined by motion vectors located at control points at the corners on the left and top side of the current block. The most general one is based on three control point potion vectors located at the top-left, the top-right and the bottom-left of the current block. There exists also a less general model that can handle zoom and rotation but not more complex affine motion as it is based on two control point motion vectors located at the top-left and the top-right of the current block.  ECM currently lacks a representation of the 4-parameter model only given by control point motion vectors on the left side.

# Proposal

When the affine motion type indicates the 4-parameter model, the width is compared with the height of the block and when the width is greater than the height an alternative 4-parameter model based on control point vectors on the left side is derived as:

&mvx=mv2y-mv0yHx+mv2x-mv0xHy+mv0x&mvy=mv0x-mv2xHx+mv2y-mv0yHy+mv0y

mv0

mv2

Otherwise, the motion model is derived as in ECM.

The reason for deploying the alternative 4-parameter model when the width is greater than the height is that the motion field will become more different to the motion field that can be represented by the 6-parameter model since the distance to the used control point motion vectors will be longer than when control point motion vectors at the top corners of the current block is used.

The method is used for affine DMVR to derive initial motion vectors before DMVR search and also on refined control point motion vectors to derive an affine prediction.

# Results

The results are compared against ECM-13 and gcc 12.2.0 was used. The encoding time is not accurate except for class C and D which indicate 100.1%. Decoding time is 99.9%.

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0,03% | -0,10% | -0,07% | 82,3% | 99,3% | 99,9% | 100,2% |
| Class A2 | -0,01% | -0,05% | 0,06% | 94,3% | 100,2% | 99,7% | 99,7% |
| Class B | -0,02% | -0,08% | -0,14% | 103,8% | 100,0% | 99,7% | 100,2% |
| Class C | -0,01% | -0,02% | -0,01% | 100,1% | 100,1% | 100,3% | 100,0% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0,02% | -0,06% | -0,05% | 96,3% | 99,9% | 99,8% | 100,0% |
| Class D | -0,03% | 0,05% | 0,03% | 100,2% | 100,0% | 100,0% | 100,0% |

# Conclusions

It is proposed to study the use of an alternative 4-parameter affine motion model for affine DMVR in next round of EE2.

# References

[1] M. Karczewice, Y. Yan, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-AF2017, Hannover, Germany, October 2023.

[2] ECM-13.0, available at https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tags/ECM-13.0

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).