| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0122-v1 |
| --- | --- |

| Title: | AHG9: Signaling source and added pictures in EOI SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jangwon Lee Hendry Tan Chul Keun Kim Junghak Nam Jaehyun Lim Seung hwan Kim | Email: | jangw.lee@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

This contribution proposes to include signaling information of source and added pictures in EOI SEI message when temporal upsampling during / prior to encoding is applied. Source pictures and added pictures are pictures that are used as input for and pictures that are generated from the temporal upsampling process, respectively. It is asserted that such information may be useful when one or more pictures needs to be removed from the bitstream because it is naturally preferred to remove the added pictures rather than the source pictures.

# Problem statement

The current design of encoder optimization information (EOI) SEI message includes signaling information that indicates that temporal upsampling is applied prior to / during encoding process. However, EOI SEI message does not include signaling information that may help decoder / receiver of the bitstream to differentiate source pictures (i.e., pictures that are used as input for the temporal upsampling process) from added pictures (i.e., pictures that are generated from the temporal upsampling process) in the bitstream. It is asserted that such information may be useful when one or more pictures needs to be removed from the bitstream because it is naturally more preferred to remove the added pictures rather than the source pictures.

# Use case

Currently, many TV System-on-Chips (SoCs) adopt 120Hz displays to support high frame rate content, at the same time, many low-end TV SoCs still use 60Hz displays. When a video stream with 120fps is played on such low-end TVs, one of every two decoded output pictures is discarded. If a video stream which is upsampled to 120fps from 60fps, is played on a low-end TV, a user might see either only the source pictures or only the added pictures. The proposed signaling information can be used to help the decoder to display only the source pictures.

# Proposal

Syntax:

| encoder_optimization_info( payloadSize ) { | Descriptor |
| --- | --- |
| eoi_cancel_flag | u(1) |
| if( !eoi_cancel_flag ) { |  |
| eoi_persistence_flag | u(1) |
| eoi_for_human_viewing_idc | u(2) |
| eoi_for_machine_analysis_idc | u(2) |
| eoi_type | u(16) |
| if( EoiObjectBasedFlag ) |  |
| eoi_object_based_idc | ue(v) |
| if( EoiTemporalResamplingFlag ) { |  |
| eoi_temporal_resampling_type_flag | u(1) |
| eoi_num_int_pics | ue(v) |
| if( eoi_temporal_resampling_type_flag && eoi_num_int_pics > 0 ) { |  |
| eoi_dist_from_src_pic_present_flag | u(1) |
| if( eoi_dist_from_src_pic_present_flag ) |  |
| eoi_dist_from_src_pic | ue(v) |
| } |  |
| } |  |
| if( EoiSpatialResamplingFlag ) |  |
| eoi_spatial_resampling_type_flag | u(1) |
| if( EoiPrivacyProtectionFlag ) { |  |
| eoi_privacy_protection_type_idc | u(4) |
| eoi_privacy_protected_info_type | u(8) |
| } |  |
| } |  |
| } |  |

Semantics:

eoi_dist_from_src_pic_present_flag equal to 1 indicates the syntax element eoi_dist_from_src_pic is present in the SEI message. eoi_dist_from_src_pic_present_flag equal to 0 indicates the syntax element eoi_dist_from_src_pic is not present in the SEI message.

eoi_dist_from_src_pic greater than 0 indicates the distance in picture count, from the last source picture which precedes the current picture in output order, to the current picture. eoi_dist_from_src_pic equal to 0 indicates that the current picture is a source picture. The value of eoi_dist_from_src_pic shall be in the range of 0 to eoi_num_int_pics, inclusive. When eoi_temporal_resampling_type_flag is equal to 1, eoi_num_int_pics is greater than 0 and eoi_dist_from_src_pic_present_flag is equal to 0, the distance from the source picture to the current picture is unknown.

NOTE –  When temporal upsampling optimization is applied and the number of added picture(s) is available, the distance from the source picture to the current picture, when available, can be used to determine source pictures and added pictures from the temporal upsampling optimization.

# Example of determining source picture and added pictures

The variable SrcPicFlag[i] specifying whether the i-th picture in the persistence scope of this SEI message is a source picture, where the variable NumPics indicates the number of pictures in the persistence scope of this SEI message, is derived as follows:

DistFromSrcPic = eoi_dist_from_src_pic;
for (i = 0; i < NumPics; i++) {
	SrcPicFlag[ i ] = ((DistFromSrcPic % (eoi_num_int_pics + 1)) = = 0) ? 1 : 0
	DistFromSrcPic ++;
}

# Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).