| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0132-v2 |
| --- | --- |

| Title: | AHG12: SGPM improvements |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Keqin Ding Hongdong Qin Jacek Konieczny Alexey Filippov Vasily Rufitskiy Tianyu Dong 7/F, Block G1, TCL International E-City 1001 Zhongshanyuan Road Nanshan, Shenzhen, Guangdong 518052 People’s Republic of China | Email | keqin.ding@tcl.com hongdong.qin@tcl.com jacek.k@tcl.com alexey.filippov@tcl.com vasily.rufitskiy@tcl.com tianyu.dong@tcl.com |
| Source: | TCL |  |  |

_____________________________

# Abstract

This contribution proposes to introduce regression-based blending methods to SGPM to derive the blending weights and an improvement on coding the SGPM candidate index.

The experimental results over ECM-13.0 are summarized as follows:

AI:		x.xx% (Y), x.xx % (U), x.xx % (V), 100 % (EncT), 100 % (DecT)

RA:	x.xx% (Y), x.xx % (U), x.xx % (V), 100 % (EncT), 100 % (DecT)

# Introduction

## Spatial Geometric partitioning mode

SGPM is an intra mode which resembles the inter GPM mode. This intra mode partitions a coding block into two parts and generates two corresponding intra prediction modes. 26 geometric partitions and 9 of intra prediction modes are used to form the combinations. Instead of explicitly signalling these modes, a candidate list approach is adopted with each entry containing one partition split and two intra prediction modes.

At the current stage, the SGPM candidate list is sorted based on the templated matching (TM) cost and the length of sorted candidates list is set equal to 16. In the blending process applied to templates, a partition-based prediction is generated for the template with the partitioning line extended to the template. However, the blending weights in the template are binarized, as shown in the Figure 1.

Figure 1

The selected candidate index (sgpm_cand_idx) is signalled by truncated binary code.

## Regression-based GPM blending

The regression-based blending [1] is applied to GPM as an additional implicit mode, where the two integer blending matrices (W0 and W1) which are used to blend the predicted samples of each separated region are derived from the template. The blending matrices are modelled as an affine linear function of the sample positions (x, y) in the whole block as well as the areas outside the current block. The blending matrices need to be normalized first and then clipped to [1, 31].

ω1x,y=  a⋅x+b⋅y+c

W1=Clip31, 31,  ω1x,y

W0=32-W1

The parameters (a, b, c) are derived from the reference template using the same solver (MSE minimization) as the one used for CCCM, GLM or GL-CCCM. A list of pair of candidates is built from the regular GPM candidates to derive their respective affine linear models and re-ordered according to the template cost.

In particular, this regression-based GPM mode is signaled by a CU-level flag (gpm_implicit_flag). If gpm_implicit_flag is true, a merge-idx is coded to signal the pair of GPM candidates to be used. With the linear model derived from the reference template, blending matrices for the whole block can be calculated and then used to blend the predicted samples.

# Proposed methods

## Expanding SGPM candidate list with regression-based blending candidates

This contribution proposes to expand SGPM candidate list by introducing regression-based blending candidates:

- Build a regression-based blending candidate (RBC) list of the combinations of predictors being used to form the regular partition-based candidate (PBC) list in the current SGPM. Each RBC consists of a set of coefficients of the linear model for computing blending weights and is derived from the reference template in a way similar to regression-based GPM blending.

- Merge the RBC list and the existing PBC list to construct the SGPM candidate list.

In the combined SGPM list, elements from the RBC appear first, followed by elements from the PBC.

If the block has a maximum block dimension of 8 luma pixels, the length of RBC list is set to 2; otherwise, the length of RBC list is set to 4.

Table 1. List Lengths for Different Block Sizes

| Block size | RBC list length | PBC list length | SGPM list length |
| --- | --- | --- | --- |
| Max(width,height)≤8 | 2 | 16 | 18 |
| Max(width,height)>8 | 4 | 16 | 20 |

It should be noted that the values in the computed blending matrices are clipped to [0, 32]. The rest parts of SATD and RDO tests are kept unchanged.

## Entropy coding of SGPM index

In order to distinguish the probabilities of the SGPM candidate index, the bins after binarization of the candidate index are compressed with three or four contexts.

Table 2. Contexts for the candidate index coding

| Length of candidate list | Index | Code word | Context |
| --- | --- | --- | --- |
| 20 | R0, P0 | 000 + binary | C0C1C2 |
|  | R1, R2 | 001 + binary | C0C1C2 |
|  | R3, P1~P3 | 01 + truncated binary | C0C1 |
|  | P4~P15 | 1 + truncated binary | C0 |
| 18 | R0, P0 | 000 + binary | C0C1C2 |
|  | R1, P1 | 001 + binary | C0C1C2 |
|  | P2~P5 | 01 + truncated binary | C0C1 |
|  | P6~P15 | 1 + truncated binary | C0 |

R0 - R3: Regression-based candidates

P0 - P15: Partition-based candidates (regular)

# Experimental results

The proposed method was tested on top of ECM-13.0 [2] under the common test conditions [3]. The table below shows the performance results.

SGPM candidate list with regression-based blending and entropy coding.

|  | AI |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 (-3) | -0.03% | 0.09% | 0.01% |  |  |  |  |
| Class A2 (-5) | -0.01% | -0.02% | 0.01% |  |  |  |  |
| Class B (-1) | -0.01% | -0.08% | -0.11% |  |  |  |  |
| Class C | -0.01% | 0.04% | 0.06% |  |  |  |  |
| Class E | -0.04% | -0.05% | -0.14% |  |  |  |  |
| Overall | -0.02 | -0.01 | -0.04 |  |  |  |  |
| Class D | -0.01% | 0.00% | -0.10% |  |  |  |  |
| Class F (-2) | -0.07% | 0.09% | 0.00% |  |  |  |  |
| Class TGM |  |  |  |  |  |  |  |

(-x): x rows are copied from anchor.

# Conclusion

In this contribution, regression-based blending is incorporated into the SGPM candidate list as a candidate mode. The proposed methods offer 0.0X% gain in AI with higher gains in SC classes. It is recommended that the proposed methods be studied in the next round of EE2.

# References

- P. Bordes, K. Reuzé, F. Galpin, F. Urban, K. Naser, F. Le Léannec, E. Francois, “EE2-2.11: Regression-based GPM blending (tests a, b, c)”, document JVET-AG0112, Jan. 2024

- ECM-13.0: https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-13.0

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, document JVET-AF2017, Oct. 2023.

# Patent rights declaration(s)

TCL may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).